Quantec Geoscience Ltd.

116 Spadina Ave., Suite 400
Toronto, ON, M5V 2K6

Phone (416) 306 1941

Fax (416) 306 1949

Geophysical Survey
Logistics Report

E. Data

K. Mokubung

E. Martinez

CA00681T

August, 2009

TITAN-24 MT and DC/IP Surveys over Ketza River Project, ~70km south of Ross River, Yukon Territories. For Ketza River Holdings Ltd., Vancouver, BC, Canada

TABLE OF CONTENTS

1.	Introduction	2

2.	General Survey Details	3

2.1	Location	3

2.2	Access	4

2.3	Survey Area	4

3.	Survey Work Undertaken	5

3.1	Generalities	5

3.2	Personnel	5

3.3	Survey Specifications	5

3.4	Survey Coverage	7

3.5	Instrumentation	8

3.6	Parameters	8

3.7	Data Accuracy and Repeatability	10

3.8	Data Presentation	11

4.	References	13

List of appendices

APPENDIX A: Statement of Qualifications

APPENDIX B: Production Summary

APPENDIX C: IP Pseudosections

APPENDIX D: MT Sounding Curves

APPENDIX E: MT Pseudosections

APPENDIX F: Parallel Sensor Test

APPENDIX G: Instrument Specifications

APPENDIX H: Digital Archive

List of figures

Figure 1: Ketza River Project General Location Map	3

Figure 2: Ketza Grid Line Location Map	4

Figure 3: Common DC/IP Survey Layouts	6

Figure 4: Titan-24 Tensor MT and DC/IP Schematic Survey Layout.	6

Figure 5: Spectral Chargeability Model and Calculated Halverson-Wait Decays	9

Figure 6: Example of Apparent Resistivity and Phase (XY and YX) Sounding Curves	10

Figure 7: Example of Time Series from Parallel Sensor Test (BF-6 coils)	11

Figure 8: Example of Time Series from Parallel Sensor Test (P50 coils)	11

List of TABLES

Table 1: Ketza River Project Survey Lines (UTM Referenced NAD 27, Zone 08V)	4

Table 2: Ketza Grid – DC/IP Survey coverage	7

Table 3: Ketza Grid - MT Survey Coverage	7

Table 4:  Minimum Errors for DC/IP Measurements	10

# Introduction

- Quantec Project No:	CA00681T

- Project Name:	Ketza River Project

- Client: 	Ketza River Holdings Ltd.

- Client Address:	540 – 688 West Hastings Street
	Vancouver, BC
	Canada
	V6B 1P1

- Client Representative:	Graham C. Dickson
	Tel: (604) 688-9427
	Fax: (604) 688-9426
	E-mail: graham@ygcr.ca

- Grid Name:	Ketza Grid

- Survey Type: 	Tensor Magnetotelluric (MT) 
   	DC Resistivity and Induced Polarization (DCIP).

- Survey Period:	Thirteen days ( from August 9, 2009 to August 21, 2009)

- Objectives:

To map and detect potential manto mineralization to depth within the Ketza Project for drill targeting, delineation and ground condemnation. To evaluate Titan 24 as an exploration tool for providing deep vectoring tools for manto occurrences within the Ketza region.

The DC/IP component of the survey should provide an excellent means of delineating target mineralization within the top 500 to 750 meters pending geologic and cultural environment. MT resistivity provides additional resistivity information from surface to depths beyond one kilometer. The MT resistivity is useful for mapping geological contacts with resistivity contrasts and deep conductors that may potentially represent alteration or mineralization.

The Titan 24 Distributed Acquisition System (DAS) employs a combination of multiplicity of sensors, 24-bit digital sampling, and advanced signal processing (Sheard N., 1998). It provides three independent datasets capable of measuring subsurface resistivities (structure, alteration & lithology) and chargeability (mineralization) to depth.

- Report Type:

Survey logistics, describing the survey parameters and methodology, as well as presenting the survey results in digital/plot forms.

# General Survey Details

## Location

- General Location:	Approx. 70km south of Ross River,

Yukon Territories (Figure 1)

- Province:	Yukon Territories

- Nearest Settlements:	Ross River, Yukon

- Datum & Projection:	NAD 27, Zone 08V

- Lat / Long (L0N, 0E):	approx.: 6132'05.99 / 13217'13.39

- UTM position (L0N, 0E):	approx.: 6825180N, 644234E

Figure 1: Ketza River Project General Location Map

## Access

- Base of Operations:	Ketza River Holdings Ltd. Camp, Yukon Territories

- Mode of Access to Grid: 	Trucks & ATVs

- Mode of Access to Lines:	Trucks, ATV & by foot

## Survey Area

- Established by:	Ketza River Holdings Ltd.

- Coordinate Reference System: 	Survey Grid referenced to UTM Coordinates,		(Figure 2 and Table 1)

- Station Interval:	~100m

- Grid Azimuth:	~49 (True North)

- Declination:	~23E

- Method of Chaining:	Metric, pickets GPS surveyed

| Line | Array Coord. Start | Array Coord. End | UTM Coord. Start |  | UTM Coord. End |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Easting | Northing | Easting | Northing |
| L0N | 0E | 2400E | 644234 | 6825180 | 646051 | 6826769 |
| L400N | 0E | 2400E | 644014 | 6825515 | 645824 | 6827025 |
| L600N | 0E | 2400E | 643836 | 6825652 | 645632 | 6827242 |

Table 1: Ketza River Project Survey Lines (UTM Referenced NAD 27, Zone 08V)

Figure 2: Ketza Grid Line Location Map

# Survey Work Undertaken

## Generalities

- Survey Days (read time):	6 days

- Mob/Demob:	2 days

- Line Setup/Pickup/ Picketing:	5 days

- Parallel Sensor Test:	1 day (same for line setup)

- Weather/Down Days:	0 days

- Number of Lines Surveyed:	3 lines

- Survey Coverage:	DCIP survey: 7.2 km (11.1 km with current extensions)
 	(Table 2); MT survey: 7.2 km (Table 3)

## Personnel

- Project Manager:	Kevin Blackshaw
	Kevin Killin

- Responsible Geophysicist:	Evelio Martinez

- Data Processing (in field):	Emily Data
	Keeme Mokubung

- Crew Chief:	Jeff Violette

- IP Operator:	Dave Clinton
 	Jeff Violette

- MT Operators:	Mark Morrison
	Jeremie Chaput

- Remote Operators:	Shawna Miller

- Field Technicians:	Donnie McLaren
	Eric Hotvedt
	Jesse Rondeau
	Rodney Reneaud
	Steve Bates
	Leon Simpson
	Chris Marchildon
	Rylee Lamonthe
	Badden Leuszler

## Survey Specifications

### DCIP Surveys

- Survey Array: 	Dipole-Pole-Dipole Array			(combined PDR & PDL, Figure 3)

- Receiver Configuration:	24 Ex = Continuous In-line voltages (Figure 4) 
 	13 Ey = Alternating (2-station) cross-line voltages

- Array Length:	~2400 meters (3600m with current extensions)

- Number of Arrays/line:	1

- Dipole spacing: 	~100 meters

- Sampling Interval:	Ex = 100 meters
 	Ey = 200 meters

- Rx-Tx Separation: 	N-spacing (Rxn-Txn) = 0.5 to 23.5 (no extensions);

0.5 to 36.5 (with current extensions)
 	Current electrodes at midpoints between potential
 	electrodes (Figure 4).

- Infinite Pole Location: 	UTM: 648158E, 6834234N (NAD27, Z08V);

Grid Coordinates: 8901E, 4259N

- Spectral Domain:	Tx = Frequency-domain square-wave current
 	Rx = Full waveform time-series acquisition		Data processing/output in frequency-domain

Figure 3: Common DC/IP Survey Layouts

Figure 4: Titan-24 Tensor MT and DC/IP Schematic Survey Layout

### MT Surveys

- Technique:	Tensor MT soundings, remote-referenced

- Base Configuration:	24 Ex = Continuous In-line E-fields 
 	13 Ey = Alternating (2-stations) cross-line E-fields
 	1 pair LF coils
 	1 pair HF coils

- Remote Configuration	1 Ex = in line E-fields
 	1 Ey = cross-line E fields
 	1 pair LF coils
 	1 pair HF coils

- Array Length:	~2400 meters

- Number of Arrays/line:	1

- Dipole Spacing:	~100 meters

- Sampling Interval:	Ex = 100 meters
 	Ey = 200 meters

- Ex/Ey Sampling Ratio:	2:1

- E/H Sampling Ratio:	Ex = 24:1
 	Ey = 13:1

- Remote-reference Measurements:1 Hx/Hy set (1 Ey/Ex set for verification/monitoring)

- Remote Reference Position:	649118E; 6840658N (NAD 27, Zone 08V)

- Frequency bandwidth:	0.01 to 10000 Hz.

- Data Acquisition:	Full-waveform time-series acquisition
	Data processing/output in frequency-domain.

## Survey Coverage

### DCIP Survey

| LINE | SETUP | Min P1 | Max P2 | Min Tx | Max Tx | Coverage (km) | Coverage (km) with Tx extension |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L0N | 1 | 0E | 2400N | 650W | 3050E | 2.4 | 3.7 |
| L400N | 1 | 0E | 2400N | 650W | 3050E | 2.4 | 3.7 |
| L600N | 1 | 0E | 2400N | 650W | 3050E | 2.4 | 3.7 |
|  |  |  |  |  | TOTAL | 7.2 | 11.1 |

Table 2: Ketza Grid – DC/IP survey coverage

### MT Survey

| LINE | SETUP | Min EXTENT (m) | Max EXTENT (m) | Coverage (km) |
| --- | --- | --- | --- | --- |
| L0N | 1 | 0E | 2400E | 2.4 |
| L400N | 1 | 0E | 2400E | 2.4 |
| L600N | 1 | 0E | 2400E | 2.4 |
|  |  |  | TOTAL | 7.2 |

Table 3: Ketza Grid - MT Survey Coverage

## Instrumentation

- Receiver System:	Quantec Distributed Array Acquisition System comprising:
 	- 61channels max. per system (55ch operationally with
 	  internal A/D conversion (24bit @120db / dual speed
 	  @120-48kHz), and buffer memory (6Mb).
 	  22 x 2-channel Acquisition Modules (AMs)
 	  17 x 1-channel Acquisition Modules (Ams)
 	  AM data transmission using LAN cabling 		- 2 Central Recording Units (CRU), at base & remote (MT
 	  surveys) reference sites (140Gb data storage)
 	- 2 GPS synchronization clocks (10nsec precision /12.3MHz
 	  clock-speed), at base & remote (MT surveys) CPU’s
 	- 2 PC-based Central Processing Units (base & remote)

- Transmitter (DCIP Surveys):	GDD (5.0 kW) with frequency/waveform control, using
 	TRM, and Current Monitor (CM)

- Power Supply (DCIP Surveys):	Westinghouse Alternator (30 KVA @ 400 Hz / 220V / 3
 	phases) with Kolher Command 25 engine (25 HP / 2cyl)
 	and GDD TRM voltage regulator

- Receiver Electrodes:	Ground contacts using stainless steel rods

- Transmit electrodes 	4 to 10 x 1.2cm diameter 1 meter long stainless steel rods

- Receiver Coils (MT Surveys):	4 Phoenix magnetometers “P50 model” (0.0001Hz to 	1kHz); 2 at base & 2 at remote
	4 EMI magnetometers “BF-6 model” (1Hz to 25kHz);	2 at base & 2 at remote

## Parameters

### DCIP Survey

- Transmitter Waveform: 	30/256 Hz square waves at 100% duty cycle
 	(~4sec Pos./Neg.)

- Transmitter Output Current:	min 0.4 amperes to max 5.2 Amperes

- Receiver Sampling Speed: 	240 samples/second (24 bit A/D @ 120 db dynamic
 	range)

- Tx-Rx Synchronization:	using current monitor (10 sec time-accuracy)

- Time-Series Stacking: 	20 & 40 cycles (full-waveform)

- Read Time:	approx 3.0 minutes per event (20 cycles)

approx 6.0 minutes per event (40 cycles)

- Post-Processing:	using QGL QuickLayTM v.2.30.14 
	1) Time-series stacking
	2) Robust statistics
	3) Current waveform deconvolution
	4) Digital filtering (60Hz + harmonics)			5) Spectral model decay-curve fitting (Figure 5)

- Time-Domain Decay Window:	TO to TF= 0.80 to 1.93 seconds

- Final Data Output:	1) Normalized voltage (mVolts/Ampere)
 	2) Voltage error (mV/A)
 	3) Phase (milliradians)
 	4) Phase error (milliradians)
 	5) Apparent Resistivity (Ωhm-meters).

- Spectral Chargeability Model:	Halverson-Wait (Figure 5)

Figure 5: Example of Spectral Chargeability Model and Calculated Halverson-Wait Decays

### MT Survey

- Frequency Bandwidth: 	Operating:	0.01 to 48000 Hz
 	Effective:	0.1 to 20000 Hz

- Time-series Sampling:	High Range:	48000 samples/sec
 	Mid-Range:	9600 samples/sec 
	Low Range:	120 samples/sec.

- Remote-Base Synchronization: 	GPS clocks (10sec time-accuracy)

- Time-Series Stacking:	High Range:	1,534,999 samples
 	Mid-Range:	2020 (1,048,576) samples
 	Low Range:	219 (524,288) samples

- Sample/Record Time:	High Range:	min. 5 events @ ~30 seconds per event
 	Mid Range:	min. 2 events @ 2.5 minutes per event
 	Low Range:	3 events @ 80 minutes for a full event
 	(total recording and retrieving time approx. 7.5 hrs)

- Post-Processing:	using QGL QuickLayTM v.2.30.14
 	1) Coherent noise rejection using remote-reference
 	2) Proprietary digital filtering (scrubbing)
 	3) Coherency sorting
 	4) Impedance estimate stacking

- Final Data Output:	1) Auto and cross-power spectral estimates
 	2) Unrotated (XY & YX) Tensor impedances + errors
 	(apparent resistivities and E/H phase – Figure 6).

- Final Data Processing:	Edited and un-edited phase & resistivity sounding
 	curves (0.1-10000 Hz @ 8 pts/decade) using
 	GeotoolsTM.

Figure 6: Example of Apparent Resistivity and Phase (XY and YX) Sounding Curves

## Data Accuracy and Repeatability

### DCIP Survey

| ERROR TYPE | PHASE ERRORS | VOLTAGE ERRORS |
| --- | --- | --- |
| 1. Measured Data error (from csv files) using Halverson-Wait model calculation. | 86% of points have less than 10% error | 93% of points have less than 1% error |

Table 4: Minimum Errors for DCIP Measurements

### MT Survey

- Parallel Sensor Test:	Test for BF-6 (High to Mid Frequency Range)
 		Figure 7 and Appendix F
 		Test for BF-4 (Low Frequency Range) 			Figure 8 and Appendix F

- Data Error:		Apparent Resistivity = <1/20TH decade average.
 		Phase = <3 degrees average.

- Inversion Error: 	1/20TH decade, minimum acceptable.

Figure 7: Example of Time Series from Parallel Sensor Test (BF-6 coils)

Figure 8: Example of Time Series from Parallel Sensor Test (P50 coils)

## Data Presentation

### DCIP Survey

- Pseudosection Plots:	In-line DC/IP Resistivity and Chargeability Pseudo-sections,
 	posted, contoured (equal area zoning) and plotted in ground
 	units using Quantec’s QuickLay viewer (Appendix C).

- Digital: 	Raw Data: Raw Event Log File Folders (eg. Eventxxxx.dat).
 	Also contains AU.txt and Event.log files, which contain
  	information on the location and time of the event in 
	QuickLay propriety digital format (output to Matlab format 	upon request).

Processed Data: DCIP ASCII DATA, in *.CSV (comma
 	delimited) file format, from QuickLay, containing final
 	processed voltage and phase data (Ex)

Line 1:	Column headings

Column 1:	Event name/number (e.g., Event100020)
 	Column 2:	Transmitter site ID (e.g., Tx150) 		Column 3:	Receiver site ID (e.g., Rx150)		Column 4-11:	C1-C2/P1-P2 positions in X and Y meters) 	Column 12:	Current (Amperes) 			Column 13:	Current error (Amperes) 		Column 14:	Normalized voltage (Volts/Ampere) 		Column 15:	Voltage error (Volts/Ampere) 		Column 16:	Phase (milliradians) 	
 	Column 17: 	Phase error (milliradians) 		Column 18:	Apparent resistivity (Ohm-meters).

### MT Survey

- Sounding Curves:	MT Apparent Resistivity and Phase (XY and YX)
 	(Appendix D) in log frequency format, using GeotoolsTM
 	viewer.

- Pseudosection Plots:  	MT Apparent Resistivity and Phase Pseudosections (XY,
 	and YX), posted, contoured (equal area zoning) and plotted,
 	in grid units using GeotoolsTM viewer (Appendix E).

- Digital:	Raw data: Base and Remote Raw Event Log File Folders	(i.e. Base - Eventxxxxx.dat; Remote Eventxxxxxx.dat).
 	Also contains AU.txt and Event.log files, which contain
 	information on the location and time of the event in
  	QuickLay propriety digital format.

Processed data:  MT DATA, in .EDI (electronic data 
 	interchange) file, created in GeotoolsTM containing 
 	tensor-sounding data (XY & YX), for individual stations
 	(sites) and profiles (site-sets), in a format conforming to 	SEG standard for the storage MT data.

# References

- Halverson, M.O., Zinn, W.G., McAlister, E.O., Ellis, R., and Yates, W.C. (1981).  Assessment of results of broadband spectral IP field test.  In: Advances in Induced Polarization and Complex Resistivity, pp.295-346, University of Arizona.

- Johnson, I.M (1984).  Spectral induced polarization parameters as determined through time-domain measurements.  Geophysics, v. 49, pp. 1993-2003.

- Pelton, W.H., Ward, S.H., Hallof, P.G., Sill, W.R. and Nelson, P.H. (1978).  Mineral discrimination and removal of inductive coupling with multi-frequency IP.  Geophysics, v.43, pp.588-609.

- Sheard, N. (1998).  MIMDAS:  A new direction in geophysics.  Proceedings of the ASEG 13th International Conference, Hobart, Tasmania

RESPECTFULLY SUBMITTED
QUANTEC GEOSCIENCE INC.

[signed and sealed]

E. Martinez del Pino, M.Sc., P. Geo (ON) 
	Responsible Geophysicist
	Interpretation Group - QGL

E. Data
	Data Processor – QGL

K. Mokubung
	Data Processor – QGL

August, 2009
Toronto, ON

(Footnote continued)

UTM coordinates (NAD 27, Zone08V) supplied by Quantec Geoscience Ltd.

Ketza River Project General Location Map supplied by Ketza River Holdings Ltd.

PDR-pole-dipole right, PDL-pole-dipole left

Note: Cross-Line Ey voltages obtained for future reference purposes – not presented in cross-sectional plots.

T0-integration start time, TF- integration end time.

The Halverson-Wait model chargeability (Halverson et al., 1981) is similar to and improves upon the frequency-domain Cole-Cole model (Pelton et el., 1978) described in the time-domain by Johnson (1984).

HW model parameters calculated in frequency domain, with hatched green lines corresponding to theoretical HW decay with spectral r-factors of 0.1, 1.0 (default) & 10, k-factor of 0.2 (default).

Cross-line (YX) values not shown for presentation purposes.

Apparent resistivities calculated in 2d space using 4-electrode general array configuration (as per XY electrode positioning in columns 4-11 of csv file) – not based on pole-dipole calculations (K. Nurse, QGL, pers. comm., 07-2004).

XY denotes in-line electrical (E) field and orthogonal magnetic (H) field (Ex/Hy).  YX denotes in-line H field and orthogonal E-field (Ey/Hx).