Capstone

August 12, 2009

Line 0E

Azimuth

Array extents 0N to 2500N

Current extents 550S to 3250N

Dipole Size 100m inline, 100m cross line

Pole Dipole IP

Infinite Co ordinance 8901E 4259N

48K 50000 samples inline

48K 50000 samples crossline

120 25000 samples inline

120 25000 samples crossline

Screen Shot

| Event# | TX Site | Voltage | Current[Amps] | Comments |
| --- | --- | --- | --- | --- |
| 1001 | ST1 |  | 48000 | 50000 cycles |
| 1002 | ST1 |  | 120 | 25000 cycles |
| 1004 | 650W | 1200V | 3.3A | 200E bad |
| 1005 | 650W | 1200V | 3.3A | Repeat 200E bad Pig tail changed, eline and rods checked Fam at 100E changed out lan swapped out |
| 1017 | 650W | 1200V | 3.4A | fam at 200E changed |
| 1019 | 3050E | 1200V | 1.1A | Current leaks at 1600E ??? |
| 1025 | 650W | 840V | 2.2A |  |
| 1027 | 3050E | 2400V | 1.0A | wire separated from lans from 1450E to 1800E checked for leaks |
| 1029 | 650W | 840V | 2.2A | Repeat |
| 1031 | 50E | 600V | 4.1A |  |
| 1033 | 550W | 1680V | 2.1A |  |
| 1035 | 3050E | 2400V | 1.1A |  |
| 1037 | 450W | 1200V | 2.1A |  |
| 1039 | 2950E | 2400V | 0.9A |  |
| 1041 | 350W | 1680V | 1.6A | Current Monitor strange |
| 1042 | 350W | 1680V | 1.6A | Repeat |
| 1044 | 250W | 2400V | 1.0A | Rejected |
| 1046 | 2450E | 2400V | 1.0A | Lans and power wire separated around 1400E Bad |
| 1048 | 250W | 1200V | 3.0A | Repeat Lan crash in harvest |
| 1051 | 250W | 1200V | 3.0A | Repeat |
| 1053 | 2950W | 2400V | 1.0A |  |
| 1038 |  |  |  |  |
| 1040 |  |  |  |  |
| 1042 |  |  |  |  |
| 1044 |  |  |  |  |
| 1044 |  |  |  |  |
| 1048 |  |  |  |  |
| 1050 |  |  |  |  |
| 1052 |  |  |  |  |
| 1054 |  |  |  |  |
| 1056 |  |  |  |  |
| 1058 |  |  |  |  |
| 1060 |  |  |  |  |
| 1062 |  |  |  |  |
| 1063 |  |  |  |  |
| 1065 |  |  |  |  |
| 1067 |  |  |  |  |
| 1069 |  |  |  |  |
| 1071 |  |  |  |  |
| 1073 |  |  |  |  |
| 1075 |  |  |  |  |
| 1077 |  |  |  |  |
| 1079 |  |  |  |  |
| 1081 |  |  |  |  |
| 1083 |  |  |  |  |
| 1085 |  |  |  |  |
| 1088 |  |  |  |  |
| 1090 |  |  |  |  |
| 1092 |  |  |  |  |
| 1094 |  |  |  |  |
| 1096 |  |  |  |  |
| 1098 |  |  |  |  |
| 1100 |  |  |  |  |
| 1101 |  |  |  |  |

End IP