Ketza

August 18, 2009

Line 400N

Azimuth 49

Array extents 0E to 2400E

Current extents 650W to 3050E

Dipole Size 100m inline, 100m cross line

Pole Dipole IP

Infinite Co ordinance 8901E 4259N

Negative EY’s: 600E, 1000E, 1800E,2400E

48K 50000 samples inline

48K 50000 samples crossline

120 25000 samples inline

120 25000 samples crossline

Screen Shot of Marshal

Shot of Au’s

| Event# | TX Site | Voltage | Current[Amps] | Comments |
| --- | --- | --- | --- | --- |
|  |  |  |  | 5.0kw GGD TX 292, TRM 11, TR 292. CRU 9009, CM 1013, Lap Top Adriana |
| 3001 |  |  |  | 50000 sample 48K |
| 3002 |  |  |  | 25000 sample 120, stations 0E, 200E are bad swapping pigtails and battery cables |
| 3009 |  |  |  | 25,000 sample 120, station 0E is still bad, cleaning all connections, checking the e-line for nicks. |
| 3011 |  |  |  | 2500 sample 120 |
| 3019 | 2350e | 2400v | 1.8a | Top of mountain, rocky |
| 3020 | 2350e | 2400v | 1.8a | repeat |
| 3022 | 550e | 2400v | 1.0a | Top of other mountain, rocky |
| 3023 | 550e | 2400v | 1.0a | repeat |
| 3025 | 2450e | 2400v | 0.9a | Rocky |
| 3027 | 450e | 2400v | 1.1a | Top of mountain, rocky |
| 3029 | 2550e | 2400v | 0.8a | Heading down the side of mountain, rocky, Bad reading |
| 3031 | 2550e | 2400v | 0.8a | Repeat, 40 cycles, Bad reading, need more current!!!! |
| 3033 | 350e | 2400v | 1.2a | Rocky, side of mountain |
| 3035 | 250e | 2400v | 2.4a | Side of mountain, mossy area, shrubs |
| Day 2 |  |  |  |  |
| 3038 | 250e | 1680v | 3.5a | Repeat, transmitter kicked off |
| 3040 | 250e | 1200v | 2.9a | repeat |
| 3042 | 150e | 1200v | 2.5a | Side hill, coming off of mountain |
| 3044 | 2550e | 2400v | 1.3a | Side hill rocky, repeat, 10 rods, and salt water, bad reading, checking infinite for leaks close to doghouse |
| 3046 | 2550e | 2400v | 1.3a | repeat |
| 3047 | 2550e | 2400v | 1.3a | repeat |
| 3049 | 50e | 1200v | 3.3a | Off the mountain in the flat area, wet ground |
| 3051 | 2650e | 2400v | 1.6A | Side hill rocky |
| 3053 | 50w | 840v | 4.1a | Low wet ground |
| 3055 | 2750e | 2400v | 1.2a | Side hill rocky |
| 3057 | 150w | 1680v | 2.7a | Low ground |
| 3059 | 2850e | 2400v | 1.3a | Side hill, rocky |
| 3061 | 250w | 1200v | 3.4a | Low wet ground |
| 3063 | 350w | 1200v | 3.1a | Low wet ground |
| 3065 | 2950e | 2400v | 0.9a | Side hill rocky, 10 rods, lots of water, 40 cycles, bad reading |
| 3067 | 450w | 1200v | 2.7a | Low wet ground |
| 3069 | 2950e | 2400v | 1.7a | Rocky ground 10 rods |
| 3071 | 550w | 1200v | 3.7a | Low wet ground |
| 3073 | 3050e | 1680v | 2.5a | Edge of mountain, rocky |
| 3075 | 650w | 1200v | 3.4a | Low ground |
| 3077 | 2250e | 2400v | 0.7a | Top of mountain |
| 3078 | 2250e | 2400v | 0.7a | repeat |
| 3080 | 650e | 1680v | 2.7a | Side of mountain, rocky |
| 3082 | 2150e | 2400v | 1.3a | Top of mountain |
| 3083 | 2150e | 2400v | 1.3a | repeat |
| 3085 | 750e | 1680v | 2.4a | Side hill rocky |
| 3087 | 2050e | 2400v | 1.4a | Side hill rocky |
| 3089 | 850e | 1680v | 2.3a | Side hill, rocky |
| 3091 | 1950e | 2400v | 1.5a | Side hill rocky, bad reading current leak |
| 3093 | 1950e | 2400v | 1.5a | repeat |
| 3095 | 950e | 1680v | 3.4a | Bottom of mountain in valley |
| 3097 | 1850e | 2400v | 1.3a | Side hill, rocky, battery died |
| 3103 | 1850e | 2400v | 1.5a | Side hill |
| 3105 | 1050e | 1680v | 2.6a | valley |
| 3107 | 1750e | 840v | 3.5a | Side hill |
| 3109 | 1150e | 2400v | 1.5a | Rocky, side hill coming up other mountain |
| 3111 | 1650e | 2400v | 1.1a | Side hill, rocky |
| 3113 | 1250e | 2400v | 1.5a | Rocky side hill, edge of road |
| 3115 | 1550e | 2400v | 1.1a | Side hill, rocky |
| 3117 | 1350e | 2400v | 0.8a | Rocky, boulders, side hill, very shitty ground |
| 3119 | 1450e | 2400v | 1.4a | Side hill, rocky |
| 3120 | 1450e | 2400v | 1.4a | repeat |

End IP