Ketza

August 14, 2009

Line 600N

Azimuth 49

Array extents 0E to 2400E

Current extents 650W to 3050E

Dipole Size 100m inline, 100m cross line

Pole Dipole IP

Infinite Co ordinance 8901E 4259N

48K 50000 samples inline

48K 50000 samples crossline

120 25000 samples inline

120 25000 samples crossline

Screen Shot

| Event# | TX Site | Voltage | Current[Amps] | Comments |
| --- | --- | --- | --- | --- |
| 2002 | ST1 |  | 48000 | 50000 cycles Pig tail and battery cables changed at 100E and 400E |
| 2003 | ST2 |  | 120 | 25000 cycles Fam changed at 100E |
| 2005 | ST3 |  | 120 | 25000 cycles |
| 2006 | ST4 |  | 48000 | 50000 cycles |
| 2008 | 650E | 1680V | 2.0A | Funky at 200E, crap |
| 2010 | 650W | 600V | 3.9A | Neg Res at 1150E ??? |
| 2011 | 650W | 600V | 3.9A | Neg Res at 1150E ??? |
| 2013 | 3050E | 2400V | 1.3A |  |
| 2014 | 3050E | 2400V | 1.3A | Repeat Crappy |
| 2016 | 650E | 2400V | 2.0A | Repeat, crap |
| 2018 | 550W | 840V | 3.3A | Neg at 1150 |
| 2020 | 2950E | 2400V | 1.5A |  |
| 2022 | 450W | 1200V | 2.3A | Neg at 1150 |
| 2023 | 450W | 1200V | 2.3A | Repeat Neg at 1150 |
| 2025 | 2850E | 2400V | 2.0A |  |
| 2026 | 450W | 1200V | 2.3A | Repeat Fam from 900E fam 134 swapped with fam from 1100E Fam 36. No change |
| 2028 | 450W | 1200V | 2.3A | Repeat Rod moved at station 1100E No change |
| Day2 |  |  |  |  |
| 2035 | 450w | 1200v | 2.4a | Low ground, edge of mountain, repeat |
| 2037 | 2850e | 2400v | 2.1a | Side of mountain, moss, rocks, repeat |
| 2039 | 350w | 840v | 2.8a | Low ground, side of mountain |
| 2041 | 250w | 1200v | 3.0a | Low ground, side of mountain |
| 2043 | 150w | 1680v | 2.5a | Side hill, start of mountain |
| 2045 | 50w | 2400v | 1.7a | Side of mountain, rocks, moss |
| 2047 | 50e | 1200v | 2.8a | Side of mountain |
| 2049 | 2750e | 2400v | 0.5a | Side of mountain, rocky. 8 power rods and 5 gallons of salt water. 40 cycles |
| 2050 | 2750e | 2400v | 0.5a | Repeat, 40cycles |
| 2052 | 150e | 1200v | 2.9a | Side of mountain, 40 cycles by accident |
| 2054 | 2650e | 2400v | 1.7a | Side of mountain, rocky |
| 2056 | 250e | 1680v | 2.5a | Side of mountain, moss rocks |
| 2058 | 2550e | 2400v | 0.9a | Side of steep mountain face , rocky |
| 2059 | 2550e | 2400v | 0.9a | repeat |
| 2061 | 350e | 2400v | 2.2a | Top of mountain, rock and moss |
| 2063 | 450e | 1200v | 3.2a | Over the peak of the mountain, 25m on the down side of steep slope, black earth |
| 2065 | 2450e | 2400v | 1.4a | Side slope steep mountain face, boulders, lots of salt water |
| 2067 | 550e | 1200v | 3.7a | Steep side slope, black earth |
| 2069 | 2350e | 1200v | 2.6a | Steep side hill, boulders |
| 2071 | 650e | 2400v | 1.3a | Side hill, rocky, coming to the bottom of mountain in a valley, Repeat |
| 2073 | 750e | 1200v | 3.5a | Bottom of valley, creek bed |
| 2075 | 2250e | 1680v | 1.9a | Side of cliff, rocky, lots of salt water |
| 2077 | 850e | 1200v | 3.8a | Bottom of valley, moss |
| 2079 | 2150e | 2400v | 0.6a | Rocky cliff, edge of mountain peak, bad reading, moving rods for better amps. Starting to rain! |
| 2081 | 950e | 1680v | 2.0a | Valley, rocky, shrubs |
| 2083 | 2150e | 2400v | 1.1a | repeat |
| 2085 | 2150e | 2400v | 1.1a | Repeat 40 cycles, looks like coupling 1850e. Moving wires and checking for leaks at 1900e to 1600e, checking for leaks at 1000e to 1400e |
| 2087 | 1050e | 2400v | 1.4a | Steep side hill going up out of valley, rocky ground, 40 cycles |
| 2090 | 2050e | 1680v | 2.7a | Side of mountain, close to peak |
| 2092 | 1150e | 2400v | 1.0a | Steep side hill, rocky |
| 2094 | 1950e | 2400v | 1.2a | Edge of mountain peak, rocky, checked for leaks from 1550e to dog house, checked infinite for leaks close to dog house, cannot find leaks |
| 2095 | 1950e | 2400v | 1.2a | Repeat 40cycles |
| 2098 | 1250e | 2400v | 0.9a | Side hill, rocky |
| 2100 | 1850e | 1680v | 2.4a | Plateau of mountain |
| 2102 | 1350e | 2400v | 1.8a | Side hill coming onto plateau |
| 2104 | 1750e | 2400v | 1.3a | Plateau, rock moss |
| 2106 | 1450e | 2400v | 1.0a | Plateau, rock moss, bad |
| 2110 | 1650e | 2400v | 1.0a | Plateau, rock moss, bad, swapping battery cables at 1100 and 1000 |
| 2113 | 1650e | 2400v | 1.0a | Repeat, still shitty |
| 2115 | 1650e | 2400v | 1.0a | Repeat, 40 cycles |
| 2117 | 1450e | 2400v | 1.8a | repeat |
| 2119 | 1550e | 2400v | 1.7a | plateau |

End IP