CA00681T – KETZA PROJECT

KETZA GRID

KETZA RIVER HOLDINGS Ltd.

Processing Notes – Line 0N

# IP Processing

IP Acquisition:	August 12-13, 2009

Document Date:	Overnight of August 13, 2009

## IP Specific Association/Logistical Notes

### Line: 0N

| Name: | L0N |  |
| --- | --- | --- |
| Line extent: | 600W to 3000E | (2.4km with 600m current extensions) |
| Dipole Range: |  |  |
| In-line: | 50E to 2350E |  |
| Cross-line: | 0E to 2400E |  |
| Transmitters: | 650W to 3050E |  |
| Final Output: | L0N.csv |  |

|  | Grid Co-ordinates (metres) |  | Location: NAD27& Z08V |  |
| --- | --- | --- | --- | --- |
|  | Easting | Northing | Easting | Northing |
| Pole-Dipole Infinite | 8901 | 4259 | 648158 | 6834234 |

### Operator log points

- First line surveyed.

- Line was picketed and GPS surveyed by Quantec using hand-held GPS. Station spacings were marked using GPS.

- Power wire moved 100m away from line between 1400E and 1800E, around 450E & 950E

- Tx location moved ~10m south in attempt to get better currents for some events.

- FAMS & other equipment switched out / examined at 100E & 200E

### Marshal AU Info Window for IP Acquisition

Figure 2-1: Marshal AU Info window for IP acquisition, August 12, 2009

## IP Events

Notes By:	Keeme Mokubung

Day1 Events: 9009.001000 – 9009.001053

Events to Watch

| Rx Site ID | NetEvent# | Notes |
| --- | --- | --- |
| 200 | 1004, 1005, 1017, 1035, 1039, 1051 | Ey200 not tracking. (Figure 2-2)Channel disabled |
| Tx | 1005 | Tx has changing amplitude (Figure 2-3) |
| All | 1004, 1005, 1025, 1029, 1033, 1037 | Large spikes that could be due to coupling and large change in signal amplitude (Figure 2-4) |
| 450E | 1004, 1005, 1017, 1025, 1029, 1031, 1033, 1037, 1041, 1042, 1051 | Coupling (Figure 2-5) |
| 1350, 1650, 1750 | 1019, 1027, 1039 | Coupling (similar to Figure 2-5). Operator marked event 1019 as“Current leaks at 1600E ???”…repeat event 1027 has similar signal at 1650. |
| 150, 200 | 1027, 1035, 1039, 1053 | Drifting and small signal amplitude |

Day 2 Events: 9009.001059 – 9009.001136

Events to Watch

| Rx Site ID | NetEvent# | Notes |
| --- | --- | --- |
| Ey0, Ex150 | All | Looks like wind noise (?) or bad contact at these sites. (Figure 2-7). LAN, pigtail, FAM were swapped. E-lines & rods were checked. |
| 50E to 1050E | 1073, 1080, 1081, 1086, 1088, 1090, 1094, 1098, 1102, 1106, 1113, 1114, 1118, 1120,1122, etc. | Very noisy results when transmitting on eastern end of array (Figure 2-8). Applied currents are very low. Ey0 and Ex150 are all noise or drifting. |

### Overscale Issues

| Event # | Tx Site ID | Rx Site ID | # Overscales | Notes |
| --- | --- | --- | --- | --- |
| 1092 | 550E | 450E | 61466 | Disabled by QL |

Figure 2-2: Event 1004; Tx650W shows Ey200 not tracking.

Figure 2-3: Event 1005 shows Tx with changing amplitude

Figure 2-4: Event 1005; Tx 650W shows large spikes

Figure 2-5: Event 1017; Tx650W shows coupling at 450E

Figure 2-6: Event 1027; Tx3050 shows drifting and small signal amplitude

Figure 2-7: Event 1070; wind (?) noise/bad contact at Ey0 & Ex150

Figure 2-7: Event 1073; Very noisy TS when transmitting from eastern end (Tx3050)

## IP Processing Runs

### Common Processing Parameters

| Parameter | Value |  |
| --- | --- | --- |
| Method | JEEK v2.1 |  |
| Stacking |  |  |
| Initial ½ Periods to Skip | 2 |  |
| Stack Depth | 5 |  |
| Outlier Rejection | Median |  |
| Type | Quad |  |
| Harmonic | 1 |  |
| Error Margin | 1.5 |  |
| Vp/Phase Determination Specs. |  |  |
| Phase Start Time | 0.8s | Offset from Start |
| Phase End Time | 0.2s | Offset from End |
| Decay Window Type | Tukey |  |
| TD Response Filter Type | Hanning |  |

### L0N_Day1

Name:		L0N_Day1.IP

Setup:		Keeme Mokubung

Date:		August 12, 2009

Setup Notes:

- Processed all events acquired on August 12, 2009 that are marked good by the operator and after checking time series.

Review

Reviewer:	Keeme Mokubung

| Sensors |  |  | Description | Observations |
| --- | --- | --- | --- | --- |
| Tx. | Rx. |  |  |  |
| Tx leading |  |  |  |  |
| 2950 & 3050 |  | 1350 to 1750 | Phase & rho errors | Relatively large phase values |
| 2950 & 3050 |  | 50 to 1050 | Phase & rho errors | Very low Vp; noisy decay curves |
| Tx lagging |  |  |  |  |
| -650 to -250 |  | 150 | Phase & rho errors | Low signal amplitude (Figure 2-6); noisy decay curves; low Vp (less than 1mV) |
| -550 to 50 |  | 2150 to 2350 | Phase & rho errors | Low signal amplitude; noisy decay curves; low Vp |

### L0N_All

Name:		L0N_All.IP

Setup:		Emily Data

Date:		August 13, 2009

Setup Notes:

- Processed all events marked good by the operator and after checking time series.

- Events from both days were processed.

Review

Reviewer:	E.D.

| Sensors |  | Description | Observations |
| --- | --- | --- | --- |
| Tx. | Rx. |  |  |
| Tx leading |  |  |  |
| All | Ex150 | Low/Negative phase values, very low res (~1-10 ohm-m). Large errors | Very noisy TS and bad decay curves. Station was discussed in “events to watch” section. Vp drops to <<1mV for this station. |
| 1750E to 3050E | Ex50 to Ex1050 | Larger rho and phase errors | Noisy decay curves and TS (discussed in previous section – Figure 2.7). Vp’s generally <0.5mV |
| Tx lagging |  |  |  |
| 650W to 50E | Ex150 | Same as above | Same as above |
| 550W to 1450E | Ex2150 to Ex2350 | Area of larger rho and phase errors. Low res (~20 ohm-m) | Very noisy decay curves, Vp’s <1mV. TS noisy, some are drifting |

### L0N_Final

Name:		L0N.IP

Setup:		Emily Data

Date:		August 13, 2009

Setup Notes:

- Used best events for some repeat readings

Line 0N Pseudo-sections

### Apparent Resistivity (ohm-m) – Current Pole to East/Right of E-Dipoles

Line 0N Pseudo-sections

### Phase (mRads) – Current Pole to East/Right of E-Dipoles

Line 0N Pseudo-sections

### Apparent Resistivity (ohm-m) – Current Pole to West / Left of E-Dipoles

Line 0N Pseudo-sections

### Phase (mRads) – Current Pole to West / Left of E-Dipoles