CA00681T – KETZA PROJECT

KETZA GRID

KETZA RIVER HOLDINGS Ltd.

Processing Notes – Line 400N

# IP Processing

IP Acquisition:	August 18-19, 2009

Document Date:	August 19, 2009

## IP Specific Association/Logistical Notes

### Line: 400N

| Name: | L400N |  |
| --- | --- | --- |
| Line extent: | 600W to 3000E | (2.4km with 600m current extensions) |
| Dipole Range: |  |  |
| In-line: | 50E to 2350E |  |
| Cross-line: | 0E to 2400E |  |
| Transmitters: | 650W to 3050E |  |
| Final Output: | L400N_Run1.csv |  |

|  | Grid Co-ordinates (metres) |  | Location: NAD27& Z08V |  |
| --- | --- | --- | --- | --- |
|  | Easting | Northing | Easting | Northing |
| Pole-Dipole Infinite | 8901 | 4259 | 648158 | 6834234 |

### Operator log points

- Line was picketed and GPS surveyed by Quantec using hand-held GPS. Station spacings were marked using GPS.

- Contacts were much better on the second day of acquisition.

- 10 power rods and gallons of salt water used in attempt to get better currents when transmitting from the east.

### Marshal AU Info Window for IP Acquisition

Figure 2-1: Marshal AU Info window for IP acquisition, Aug. 18 (left) to Aug. 19 (right), 2009

## IP Events

Notes By:	Emily Data

Day1 Events: 9009.003000 – 9009.003035

Events to Watch

| Rx Site ID | NetEvent# | Notes |
| --- | --- | --- |
| 50E to 550E | 3019, 3020, 3025 | No signal and some drifting for these stations when transmitting from eastern end. Ey0 and Ey200 has LOTS of drifting |
| Ey0, 200, and 1800 | 3022, 3023 | Serious drifting/noise. Ey1800 due to bad battery cable (it will not be processed). Ey0 gets better when transmitting close-by |
| Ex1950 to Ex2350 | 3027, 3033 | Unstable stations. See figure 2-2 |
| Ex2350, Ey2400 | 3033 | Looks like current leak (?) at these sites. Operator did not repeat. Will window event for this station. See figure 2-3 |
| Ex1950 to Ex2050 | 3035 | Noisy stations. Repeated on day 2. Will disable this event. |

Day 2 Events: 9009.003036 – 9009.003120

Events to Watch

| Rx Site ID | NetEvent# | Notes |
| --- | --- | --- |
| Ey2000 | 3042 | Drifting |
| 50E to 550E | 3046, 3047, 3051, 3069, 3073, 3103 | No signal and lots of drifting for these stations when transmitting from eastern end. Ey200 has LOTS of drifting, Ey0 looks like wind noise |
| Ey0, 200 | All | Serious drifting/noise. Ey0 gets better when transmitting close-by. See example at Figure 2-4 |

### Overscale Issues

| Event # | Tx Site ID | Rx Site ID | # Overscales | Notes |
| --- | --- | --- | --- | --- |
| None |  |  |  |  |

Figure 2-2: Event 3027: unstable/noisy dipoles

Figure 2-3: Event 3033: looks like current leak at last station for Tx350 event. Will window event for affected sites since no repeat was done.

Figure 2-4: Event 3053 – Lots of drifting at Ey200 despite having very close transmit site

## IP Processing Runs

### Common Processing Parameters

| Parameter | L400N_Run1 |  |
| --- | --- | --- |
| Method | JEEK v2.1 |  |
| Stacking |  |  |
| Initial ½ Periods to Skip | 2 |  |
| Stack Depth | 5 |  |
| Outlier Rejection | Median |  |
| Type | Quad |  |
| Harmonic | 1 |  |
| Error Margin | 1.5 |  |
| Vp/Phase Determination Specs. |  |  |
| Phase Start Time | 0.8s | Offset from Start |
| Phase End Time | 0.2s | Offset from End |
| Decay Window Type | Tukey |  |
| TD Response Filter Type | Hanning |  |

### L400N_All

Name:		L400N_All.IP

Date:		August 19, 2009

Setup Notes:

- Processed all events marked good by the operator and after checking time series.

- Use windowed event 3033win for Ex2350

- Events from both days were processed.

Review

Reviewer:	E.D.

| Sensors |  | Description | Observations |
| --- | --- | --- | --- |
| Tx. | Rx. |  |  |
| Tx leading |  |  |  |
| 1150E to 3050E | 50E to 850E | Very high or negative phase values in area of very low app. resistivity. Very large errors | Discussed in Events to watch section: no signal at these sites when transmitting from eastern end. Some TS are drifting. Vp’s are very low (<0.3mV), decay curves are noisy & not decaying. |
| 2450E to 3050E | 950E to 1750E | Normal phase and apparent resistivity values, but area of larger errors | When transmitting outside of array, TS are flat (only EM spikes can be seen). However, decay curves are good. Vp’s are <2.5mV |
| 2550E to 2950E | 2050E to 2350E | Higher errors in area of low apparent resistivity. Phase values vary. | Decay curves show EM coupling for these stations. EM spikes dominate TS. Vp’s <1mV for 2050E and 2150E. |
| Tx lagging |  |  |  |
| 50E to 150E | 450E to 2350E | Moderate errors for these stations. Phase and apparent resistivity values look OK | Nothing apparent in TS. Decay curves look good for these events. Vp’s are moderate (>5mV) |
| 350E to 450E | 2050E to 2350E | Very large errors. Phase values alternate from very large to negative. Apparent resistivity looks OK. | Discussed in Events to watch section. Unstable dipoles, decay curves are bad. Vp’s are low (generally <1mV) |

### L400N_Final

Name:		L400N.IP

Date:		August 19, 2009

Setup Notes:

- Used best events for some repeat readings

- This will be final run

Line 400N Pseudo-sections

### Apparent Resistivity (ohm-m) – Current Pole to East/Right of E-Dipoles

Line 400N Pseudo-sections

### Phase (mRads) – Current Pole to East/Right of E-Dipoles

Line 400N Pseudo-sections

### Apparent Resistivity (ohm-m) – Current Pole to West / Left of E-Dipoles

Line 400N Pseudo-sections

### Phase (mRads) – Current Pole to West / Left of E-Dipoles