CA00681T – KETZA PROJECT

KETZA GRID

KETZA RIVER HOLDINGS Ltd.

Processing Notes – Line 400N

# MT Processing

MT Acquisition:	Overnight of August 18-19, 2009

Document Date:	August 19, 2009

## Association/Logistical Notes

### Line 400N

Rx Dipoles to Watch

| Rx Site ID | Notes |
| --- | --- |
| Ey1800 | Battery cable issues for first two 120Hz events |
| Ex50 to Ex250 | Drifting TS, but peaks line up. Sites were trouble-shot during the day, equipment changed out. |

Remote

|  | Location: (NAD27 & Z08V) |  |
| --- | --- | --- |
|  | Easting | Northing |
| Remote Site | 649118 | 6840658 |

Coil Locations

| Site ID Prefix | Titan Line Co-ordinates (metres) |  | #### Direction |  | Coil Serial #’s |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Easting | Northing | Name | Azimuth | High Freq. | Low Freq. |
| Ln | 1200E | 350N | In-line Cross-line | 49 319 | BF6-0302 BF6-0303 | P50-2235 P50-2236 |
| Rm (remote) | 13840E | 8477N | In-line Cross-line | 49 139 | BF6-9915 BF6-0305 | P50-1970 P50-2126 |

## MT Events

Notes By:	Emily Data

| Event Name | Sample Rate (sps) | NetEvents | Type | Timing Offsets (counts) | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Applied |  |
| L400N_120.1 | 120 | 9009.003509 9015.004605 | Line Remote | Ok | Noise burst near end of event at all stations and coils at the line. Will window this event |
| L400N_120.2 | 120 | 9009.003511 9015.004609 | Line Remote | Ok | Ey1800 bad battery cable |
| L400N_120.3 | 120 | 9009.003514 9015.004611 | Line Remote | Ok | Drift on Ey200 and Ey2200 |
| L400N_9600.1 | 9600 | 9009.003515 9015.004614 | Line Remote | Ok |  |
| L400N_9600.2 | 9600 | 9009.003517 9015.004617 | Line Remote | Ok | Ey1800 not tracking. Was disabled. |
| L400N_48k.1 | 48,000 | 9009.003518 9015.004619 | Line Remote | Ok |  |
| L400N_48k.2 | 48,000 | 9009.003521 9015.004624 | Line Remote | Ok |  |
| L400N_48k.3 | 48,000 | 9009.003522 9015.004626 | Line Remote | Ok |  |
| L400N_48k.4 | 48,000 | 9009.003523 9015.004628 | Line Remote | Ok |  |
| L400N_48k.5 | 48,000 | 9009.003525 9015.004630 | Line Remote | Ok |  |

## Marshal AU Info Windows for MT Acquisition

Figure 3-1: Marshal AU Info Windows for MT Acquisition – Overnight of August 18, 2009

## MT Processing Runs

### L400N_Order

Name:		L400N_Order.MT

Setup:		Emily Data

Date:		August 19, 2009

Setup Notes:

- Processed only 48k events by individual sdms to determine the best order of processing 48ks.

- Only site 1150E was processed

| Band (sps) | Events in Order | Notes |
| --- | --- | --- |
| 48,000 | 48k.1 48k.2 48k.3 48k.4 48k.5 | All events scrubbed |

Review

Date:		August 19, 2009

- Best order of 48k’s found and will be applied for the next processing runs.

### L400N_Run1

Name:		L400N_Run1.MT

Date:		August 19, 2009

Setup Notes:

- Processed all events with individual sdms for all sites.

- 48ks were processed in the order determined in “L400N_Order”

| Band (sps) | Events in Order | Notes |
| --- | --- | --- |
| 120 | 120.1win 120.2 120.3 | All events were scrubbed except low frequency events |
| 9600 | 9600.1 9600.2 |  |
| 48,000 | 48k.5 48k.3 48k.2 48k.4 48k.1 |  |

Review

Date:		August 19, 2009

- Very low apparent resistivity for stations 50E to 250E; very noisy LF results below 1Hz.

- 1750E to 2350E are noisy below 0.5Hz in both modes.  TS are drifting for all 120Hz events

- Stations 1750E to 2350E also show a YX-mode phase wrap at the lower frequencies.

### Spikerej, Window

Name:		spikerej.MT & window.MT

Date:		August 19, 2009

Setup Notes:

- To find out the best way to deal with spikes/overscales on remote coils, two test processing runs were carried out on station 1150E:

  - Spikerej: processed using all 120Hz events with scrubbing and spike rejection turned on.

  - Window: 120Hz events were windowed to remove OS/spikes caused by passing trucks.

| Run | Band (sps) | Events in Order | Notes |
| --- | --- | --- | --- |
| SpikeRej | 120 | 120.2 | Scrubbing and spike rejection were turned on |
| Window | 120 | 120.2win | No scrubbing |

Review

Date:		August 19, 2009

- Both results were noisy.  Run1 result (not scrubbed or windowed) was the best.

### L400N_Run2

Name:		L400N_Run2.MT

Date:		August 16, 2009

Setup Notes:

- Studying individual events from Run1 showed that some stations could be improved by picking the best events.  Stations processed were: Ex50, Ex250, Ex1750 and Ex1850

| Station | Band (sps) | Events in Order | Notes |
| --- | --- | --- | --- |
| 50E | 120 | 120.1win 120.2 120.3 | All events were scrubbed except low frequency events |
|  | 9600 | 9600.1 |  |
|  | 48k | 48k.5 48k.3 48k.2 48k.4 48k.1 |  |
| 250E | 120 | 120.2 | All events were scrubbed except low frequency events |
|  | 9600 | 9600.1 |  |
|  | 48k | 48k.5 48k.3 48k.2 48k.4 48k.1 |  |
| 1750E & 1850E | 120 | 120.3 | All events were scrubbed except low frequency events |
|  | 9600 | 9600.1 |  |
|  | 48k | 48k.5 48k.4 |  |

Review

Date:	August 19, 2009

- Results are slightly better.

- These sites will be combined with the good stations from Run1 and placed in the L400N_Final folder.

## Final MT Raw SDMs

SDM Processing Components

Line: 	L400N

Location: 	CA00681T - Ketza Project Raw Data\L400N\MT\Processing\L400N_Final

Source:	L400N_Run1 & L400N_Run2

Line 400N: Apparent Resistivity vs. Frequency

# Rho XY ------- Green

# Rho YX ------- Orange

# Rho XY ------- Green

# Rho YX ------- Orange

# Rho XY ------- Green

# Rho YX ------- Orange

Line 400N:  Phase vs. Frequency

# Phase XY ------- Green

Phase YX ------- Orange

# Phase XY ------- Green

Phase YX ------- Orange

# Phase XY ------- Green

Phase YX ------- Orange

## Final Product – Line 400N

EDI Processed Components

Files: 	L400N.edi

Location: 	CA00681T - Ketza Project Raw Data\L400N\MT\Processing\L400N_Final\sdmedit

Figure 3-3: Strip 1 (top) – Rho XY, Strip 2 – Phase XY, Strip 3 – Rho YX, Strip 4 (bottom) – Phase YX (L400N.edi)