CA00681T – KETZA PROJECT

KETZA GRID

KETZA RIVER HOLDINGS Ltd.

Processing Notes – Line 600N

# IP Processing

IP Acquisition:	August 14-15, 2009

Document Date:	Overnight of August 14, 2009

## IP Specific Association/Logistical Notes

### Line: 600N

| Name: | L600N |  |
| --- | --- | --- |
| Line extent: | 600W to 3000E | (2.4km with 600m current extensions) |
| Dipole Range: |  |  |
| In-line: | 50E to 2350E |  |
| Cross-line: | 0E to 2400E |  |
| Transmitters: | 650W to 3050E |  |
| Final Output: | L600N.csv |  |

|  | Grid Co-ordinates (metres) |  | Location: NAD27& Z08V |  |
| --- | --- | --- | --- | --- |
|  | Easting | Northing | Easting | Northing |
| Pole-Dipole Infinite | 8901 | 4259 | 648158 | 6834234 |

### Operator log points

- Line was picketed and GPS surveyed by Quantec using hand-held GPS. Station spacings were marked using GPS.

- Power wire generally >50m away from line

- FAMs switched between 900E and 1100E, & 1100E rod moved, but negative apparent resistivity was still present.

- 8 power rods and 5 gallons of salt water used in attempt to get better currents when transmitting from the east.

- Power wire & infinite were checked for leaks – none found.

### Marshal AU Info Window for IP Acquisition

Figure 2-1: Marshal AU Info window for IP acquisition, Aug. 14 (left) to Aug. 15 (right), 2009

## IP Events

Notes By:	Emily Data

Day1 Events: 9009.002000 – 9009.002028

Events to Watch

| Rx Site ID | NetEvent# | Notes |
| --- | --- | --- |
| Ex150, Ey200, Ex250 | 2008, 2016 | Drifting in TS (figure 2-2). Ey0 is noisy |
| Ex50 to Ey400 | 2010, 2011, 2018, 2022, 2023 | No signal when transmitting from the western end of array (figure 2-3) |
| Ex1150 | 2010, 2011, 2018, 2022, 2023 | Flips to negative polarity when transmitting from western-most end. No power on that side of line. Nothing out of the ordinary in the TS. (figure 2-4) |
| 0E to 400E | 2013, 2020, 2025 | No signal and severe drifting when transmitting from the eastern-most end. |

Day 2 Events: 9009.002029 – 9009.002119

Events to Watch

| Rx Site ID | NetEvent# | Notes |
| --- | --- | --- |
| Ey400 | 2035, 2052, 2075, 2102 | Drifting Ey |
| Ey2000 | 2041, 2047, 2049, 2050, 2067, 2071, 2073, 2092 | Drifting Ey |
| Ex50 to Ey400 | 2035, 2041, 2043 | No signal when transmitting from the western end of array (figure 2-3) |
| Ex1150 | 2035, 2039, 2041, 2043, 2045, 2047, 2052, 2056, 2061, 2063 | Flips to negative polarity when transmitting from western-most end. No power on that side of line. Nothing out of the ordinary in the TS. (figure 2-4) |
| 0E to 400E | 2037, 2049, 2050, 2054, 2058, 2059,2065, 2075, 2090, 2092 | No signal and severe drifting when transmitting from the eastern-most end. |
| All | 2087 | Unstable transmitter (Figure 2-5) – leak?. Noise present in all stations. Event not repeated |
| 850E to 1350E | 2094, 2095, 2104, 2115, 2117, 2119 | Very unstable dipoles. Wire and infinite were checked for leaks. None found. Event was repeated with more cycles.(Figure 2-6), battery cable changed, etc. |

### Overscale Issues

| Event # | Tx Site ID | Rx Site ID | # Overscales | Notes |
| --- | --- | --- | --- | --- |
| None |  |  |  |  |

Figure 2-2: Event 2008; Tx650W event shows 150E to 250E are drifting

Figure 2-3: Event 2010. When transmitting from west, no signal between 0E and 400E

Figure 2-4: Event 2010; Station Ex1150 flips polarity when transmitting from western-most end

Figure 2-5: Event 2087. Tx shows amplitude jumps – leak?

Figure 2-6: Event 2117. Noisy stations. Transmit signal is OK

## IP Processing Runs

### Common Processing Parameters

| Parameter | Day1 & All Run | Final Run |  |
| --- | --- | --- | --- |
| Method | JEEK v2.1 | JEEK v2.1 |  |
| Stacking |  |  |  |
| Initial ½ Periods to Skip | 2 | 2 |  |
| Stack Depth | 5 | 3 |  |
| Outlier Rejection | Median | Median |  |
| Type | Quad | Quad |  |
| Harmonic | 1 | 1 |  |
| Error Margin | 1.5 | 1.5 |  |
| Vp/Phase Determination Specs. |  |  |  |
| Phase Start Time | 0.8s | 0.8s | Offset from Start |
| Phase End Time | 0.2s | 0.2s | Offset from End |
| Decay Window Type | Tukey | Tukey |  |
| TD Response Filter Type | Hanning | Hanning |  |

### L600N_Day1

Name:		L600N_Day1.IP

Setup:		Emily Data

Date:		August 14, 2009

Setup Notes:

- Processed all events acquired on August 14, 2009 that are marked good by the operator and after checking time series.

### L600N_All

Name:		L600N_All.IP

Date:		August 15, 2009

Setup Notes:

- Processed all events marked good by the operator and after checking time series.

- Events from both days were processed.

Review

Reviewer:	E.D.

| Sensors |  | Description | Observations |
| --- | --- | --- | --- |
| Tx. | Rx. |  |  |
| Tx leading |  |  |  |
| 1050E to 3050E | 50E to 450E | Negative/very large phase in area of very low/negative apparent res values, large errors | TS shows major drifting for these stations, Vp’s are very low (generally <0.1mV), decay curves are bad. |
| 1450E to 1950E | 550E to 1450E | Large errors, apparent res. values are “normal”, phase ~triples in value between 1050E and 1150E, otherwise normal | These are the readings with noisy stations (Figure 2-6). Decay curves are noisy, but still decaying. Vp’s are generally >15mV |
| 2150E, 2850E | 1650E to 1850E | Larger errors in area with very high phase, low res values. | TS for these events look like coupling (very rounded edges), but decay curves look fine. |

| Sensors |  | Description | Observations |
| --- | --- | --- | --- |
| Tx. | Rx. |  |  |
| Tx lagging |  |  |  |
| 650W to 450E | 1150E | Negative apparent resistivities, stripe of lower phase values. Low errors | TS looks good, except reversed polarity. Vp’s are high, decay curves look good. FAMS, LANS, etc. switched out, rod moved, power wire checked for leaks and is >50m away |
| 650W to 150E | 50E to 350E | Area of very low res, high phase values. Larger errors | Lowest Vp’s for Tx lagging (<0.5mV). However, decay curves and TS look fine. |

### L600N_Final

Name:		L600N.IP

Date:		August 15, 2009

Setup Notes:

- Used best events for some repeat readings

- Stack depth reduced to 3 to deal with drifting dipoles.

- This will be final run

Line 600N Pseudo-sections

### Apparent Resistivity (ohm-m) – Current Pole to East/Right of E-Dipoles

Line 600N Pseudo-sections

### Phase (mRads) – Current Pole to East/Right of E-Dipoles

Line 600N Pseudo-sections

### Apparent Resistivity (ohm-m) – Current Pole to West / Left of E-Dipoles

Line 600N Pseudo-sections

### Phase (mRads) – Current Pole to West / Left of E-Dipoles