Assessment of Airborne Magnetics and Radiometrics Surveys at the Corky Prospect

An 87 line km helicopter magnetic and radiometric survey has been completed over the Corky project by New-Sense Geophysics Ltd. (New-Sense) for Strategic Metals Ltd. (Strategic Metals). The survey area surrounds a reportedly drilled Cu-Mo prospect and is located approximately 250 km west-north-west of Whitehorse in the Yukon Territory. Condor Consulting, Inc. (Condor) has been commissioned to assess the data sets and provide a 3D model of the magnetics. Refer to New-Sense’s logistic report (HMR100806) for any additional survey details. Figure 1 shows the location of the survey area and flight path.

Figure 1: Corky airborne magnetics and radiometrics survey flight path. The red dot denotes the location of a reported previously drilled Cu-Mo prospect.

The range of the magnetic response across the survey area is approximately 1,700 nT. The results show two strongly magnetized anomalies dominating the data set and are shown in Figure 2. The northern magnetic anomaly may have been partially tested by drilling as a Cu-Mo prospect according to the Yukon Geologic Surveys’ MINFILE database. Anomalous stream sediment Cu samples of up to 79 ppm were collected to the south of the survey area. It is recommended that the existing drill information be compared to the airborne survey results and the south magnetic anomaly be followed up with a field check and subsequent mapping and sampling, although the source of this anomaly may be located under cover.

Figure 1: Corky total magnetic intensity image.

The University of British Columbia (UBC) 3D magnetic inversion program MAD3D (version 4.0) was used to produce a model of the magnetics data. MAG3D is a program library for carrying out forward modeling and inversion of surface, airborne, and/or borehole magnetic data in the presence of a three dimensional Earth. Data are assumed to be the anomalous magnetic response to buried susceptible material, not including Earth's ambient field. The model is specified using a mesh of rectangular cells, each with a constant value of susceptibility, and topography is included. The magnetic response can be calculated anywhere within the model volume, including above the topography, simulating ground or airborne surveys, and inside the ground simulating borehole surveys. Figure 3 displays the 3D inversion results of the magnetics data at Corky.

Figure 3: Corky 3D magnetic model.

The elevated radiometrics response appears to correlate with an east-west band of exposed rocks through the middle of the survey area off the valley floor.

The following products can be found and downloaded from the Condor ftp site (ftp://ftp.condorconsult.com , user id: archer@condorconsult.com , password: skywalker19):

- Summary report of the assessment

- Registered images of the airborne magnetics and radiometrics (NAD83, Zone 7N)

- 3D magnetic model and associated sections

It is recommended that the results of the airborne magnetics and radiometrics data be compared with any available geologic and geochemical information in order to help advance the exploration program at Corky.

Respectfully submitted;

Mark Goldie

Condor Consulting, Inc.

November 9, 2010

References:

Li, Y. and Oldenburg, D. W., 1996, 3-D Inversion of Magnetic Data: Geophysics, 61, no. 02, 394-408.

Yakovenko, A., Logistics Report for the High Resolution Helicopter Magnetic and Gamma-ray Spectrometric Airborne Geophysical Survey flown over Mint, Nikki, Corky, Meloy, King, and Mars Project Properties, Yukon, from White River Lodge (Mint and Nikki), Burwash Landing (Corky and Meloy), and Braeburn Lodge (King and Mars), Yukon carried out on behalf of Strategic Metals Ltd. by New-Sense geophysics Limited, Project # HMR100806, October 2010.