Will,

The following details outline the survey setup and minor procedural nuances. I apologize for “dumbing” it down I was trying to go through the procedure in my head and didn’t want to forget anything.

Survey Start-up:

- Sync Mag and Base: Make sure the rover has the appropriate time and date before syncing with the base mag. If the rover is not set correctly connect rover to the GPS antenna then sync.

- Connect Base Magnetic Sensor and battery cables: When setting up the base station, use the tethers to secure the staff (with sensor) in a relatively quiet and convenient location. After the mag and base have been synchronized, connect the battery and sensor cables. THE SENSOR CABLE MUST BE CONNECTED TO THE LOWER SENSOR.

- Initiate survey on base: Make sure the base mag has been erased before you start the survey or you’ll be stuck with multiple files at the end of the day. Initiate the survey on the base station mag by pressing A for survey and selecting base.

- Starting location for survey: Make sure the rover being used has been erased (see the aforementioned reason). Press A for Survey  select C for Walk  then press F for position. Locate your starting position using the GPS on the mag. Once at your starting location press A to start  C for Walk. When pausing, for any reason, press A to stop the survey  to continue the survey press F  for ok  then C for Walk.

Reminders:

- Choose a relatively quiet and convenient place to set your base station (Proton Mag #4). According to Francis the base station doesn’t necessarily need to be outside the grid, but must remain in the same location for the duration of the survey; use the tethers to hold the magnetic sensor in place. However, if the base station location is too far from the grid it may be problematic.

- Use rover 3 because it freezes less often. If it does freeze just shut it off and unplug all the cables  reconnect the cables and restart the mag (should fix it).

- I sent you a spare GPS antenna as well as an extra senor cable for the mag/base. I also sent up those small radios. I was thinking if you put it in your back pocket the interference should be minimal.

Good Skill