Silver Quest Resources Ltd.

2010 GEOCHEMICAL REPORT ON THE

FLOW property

Located in the North Dawson Range Area

Whitehorse Mining District

NTS 115N/01, 115O/04

63.08° N Latitude; 140.11° W Longitude

| Grant Numbers | Claim Names |
| --- | --- |
| YD05923 – 05950 YD50801 – 50839 YD50841 – 50862 YD50863 – 50874 YD50875 – 50927 YD50938 – 50964 YD50966, YD50967 YD117501 – 117738 YD09011 – 09139 YD09159 – 09178 YD05873 – 05890 | CAD 1 – CAD 28 CAD 29 – CAD 67 CAD 69 – CAD 90 CAD 93, 95, 97, 99, 101, 103, 105, 107, 109, 111, 113, 115 CAD 117 – CAD 169 CAD 171 – CAD 191 CAD 193, CAD 195 FC 1 – FC 238 FLOW 1 – FLOW 129 FLOW 130 – FLOW 148 WCK 1 – WCK 18 |

Work Conducted June 14 – 16, July 28, August 12 – 18 & 31, September 21 & 22, 2010

-prepared for-

SILVER QUEST RESOURCES LTD.

Suite 1410, 650 West Georgia Street

Vancouver, British Columbia, Canada, V6B 4N8

-prepared by-

Darcy Baker, Ph.D.

Ron Voordouw, Ph.D.

EQUITY EXPLORATION CONSULTANTS LTD.

Suite 200, 900 West Hastings Street

Vancouver, British Columbia, Canada, V6C 1E5

April 26, 2011

TABLE OF CONTENTS

TABLE OF CONTENTS	1

LIST OF APPENDICES	1

LIST OF TABLES	1

LIST OF FIGURES	1

1.0	SUMMARY	2

2.0	INTRODUCTION	2

3.0	PROPERTY DESCRIPTION AND LOCATION	2

4.0	ACCESSIBILITY, CLIMATE, LOCAL RESOURCES, INFRASTRUCTURE, PHYSIOGRAPHY	5

5.0	HISTORY	5

5.1	Previous Work	5

5.2	2010 Exploration Program	6

6.0	REGIONAL GEOLOGY AND MINERALIZATION	6

6.1	Geology	6

6.2	Mineralization	8

7.0	GEOCHEMISTRY	8

7.1	Rock Geochemistry	8

7.2	Soil Geochemistry	9

7.3	Silt Geochemistry	9

8.0	PROPERTY GEOLOGY AND MINERALIZATION	10

9.0	DISCUSSION AND CONCLUSIONS	10

LIST OF APPENDICES

Appendix A: Bibliography

Appendix B: Statement of Expenditures

Appendix C: Laboratory Certificates

Appendix D: Rock Sample Descriptions

Appendix E: Data Disc

Appendix F: Geologist’s Certificates

Appendix G: ALS Minerals Memo

LIST OF TABLES

Table 1: Flow property claims	2

LIST OF FIGURES

Figure 1: Location Map	3

Figure 2: Tenure information	4

Figure 3: Regional geology of the Flow property	7

Figure 4: Soil, Silt and Rock sample locations	Back Pocket

Figure 5a: Surface geochemistry (Au)	Back Pocket

Figure 5b: Surface geochemistry (As)	Back Pocket

Figure 5c: Surface geochemistry (Sb)	Back Pocket

Figure 5d: Surface geochemistry (Mo)	Back Pocket

# SUMMARY

On behalf of Silver Quest Resources Ltd. (“Silver Quest”), Equity Exploration Consultants Ltd. (“Equity”) completed geochemical sampling at the Flow property on June 14 – 16, July 28, August 8 – 12 & 31, and September 21 & 22, 2010. This property is comprised of 582 claims within the White Gold district of the Yukon Territory, located just southeast of the confluence between the White and Ladue rivers.

Work in 2010 comprised twelve days of surface geochemical sampling by crews of between 2 – 5 people, which resulted in the collection of 28 rock, 23 silt and 1018 soil samples.

The most compelling targets within the Flow property are two small Au-in-soil anomalies. The larger of these two comprises a string of three soil samples with high Au and As. The other one is a point anomaly with 343 ppb Au, which is the fifth highest Au-in-soil value in the Dawson Range database (14,000+ analyses). The small size of these anomalies should not be discouraging as discoveries in the White Gold district have been made off point anomalies. Furthermore, both anomalies occur at or near contacts between granitoid and metasedimentary rock units which is a favourable setting for mineralization in the district.

Two reconnaissance ridge line soil traverses that were completed early in the field season returned highly anomalous Au-in-soil. Follow-up sampling, however, did not corroborate the earlier results. This is explained by gold concentrations very close to analytical detection and according to the laboratory, the values are within specifications based on internal QAQC protocols.

# INTRODUCTION

During the 2010 field season, Equity was contracted by Silver Quest to carry out the sampling work on the Flow property as part of a larger exploration program focused at the Boulevard property located 45 km to the southeast. This report describes the work completed and summarizes the results.

# PROPERTY DESCRIPTION AND LOCATION

The Flow property is located in the Dawson Range of the west-central Yukon Territory, with the centre of the property located about 15 km east-southeast of the confluence between the White and Ladue rivers (Figure 1). It comprises 582 contiguous quartz claims that are centered on 63.08° N latitude and 140.11° W longitude (Figure 2).

The property consists of claims bought in 2009 from ATAC Resources and those staked by Silver Quest in late 2009 and 2010, the latter of which comprise the bulk of the property. Those bought from ATAC comprise the CAD 1 – 28 and WCK claims (Table 1).

Table 1: Flow property claims

| Grant Numbers | Claim Names | Recording Date | Staking Date | Expiry Date | Owners |
| --- | --- | --- | --- | --- | --- |
| YD05923 – YD05950 | CAD 1 – CAD 28 | June 26, 2009 | June 20, 2009 | November 26, 2012 | Silver Quest Resources - %100 |
| YD05873 – YD05890 | WCK 1 – WCK 18 | June 26, 2009 | June 20, 2009 | March 22, 2014 | Silver Quest Resources - %100 |
| YD09011 – YD09139 YD09159 – YD09178 | FLOW 1 – FLOW 129 FLOW 130 – FLOW148 | November 26, 2009 | November 16 – 21, 2009 | November 26, 2012 | Silver Quest Resources - %100 |
| YD50801 – YD50839 YD50841 – YD50862 YD50863 – YD50875 YD50876 – YD50967 | CAD 29 – CAD 67 CAD 69 – CAD 90 CAD 93, 95, 97, 99, 101, 103, 105, 107, 109, 111, 115, 117 CAD 118 – CAD 195 | June 23, July 8, 12 &21, 2010 | June 22 – 24, July 1 – 5, 2010 | November 26, 2012 | A mixture of: Silver Quest Resources - %100 Tim Sullivan- %100 |
| YD117501 – YD117738 | FC 1 – FC 238 | November 15, 2010 | October 27 – 31, November 1, 15 2010 | November 15, 2011* | Silver Quest Resources - %100 |

*N.B. expiry date does not include credit for 2010 work

Figure 1: Location Map

Figure 2: Tenure information

# ACCESSIBILITY, CLIMATE, LOCAL RESOURCES, INFRASTRUCTURE, PHYSIOGRAPHY

The Flow property is located 115 km to south-southwest of Dawson City, Yukon. The topography of the area is characterized by rolling hills typical of the Dawson Ranges. Vegetation is sparse, with lower elevations covered by spruce, birch, aspen, alder and willow, whereas higher elevations have thick moss, grasses and scrub birch. Creek valleys are filled with thick willow and alder thickets. North-facing slopes locally exhibit permafrost, which complicates soil sampling. South-facing slopes are more typically vegetated by deciduous trees such as aspen and birch. Drainage systems vary from well channelled, discrete streams to swamps in broader, flat-bottomed valleys.

The area has a sub-arctic continental climate, with cold winters and cool, wet summers. Exploration can be carried out on the property from June until October.

Direct access to the property is via helicopter and several helicopter bases are located in Dawson City. Alternatively, several boat operators in Dawson City and a barge operating out of Minto Landing are available for hire to transport supplies to points along the Yukon River where they can be subsequently mobilized by helicopter to the property. During the 2010 program, personnel were mobilized from the Boulevard camp via Hughes 500D helicopter. The nearest airstrips are Independence and Thistle, both of which are located about 35 km from the property to the south-southeast and east respectively. Fixed wing operators from Dawson City and Whitehorse run Otter, Beaver and Cessna 206 aircraft capable of servicing these airstrips.

# HISTORY

## Previous Work

Parts of the area covered by the currently defined Flow claims were staked and worked at different times. The WCK and CAD 1 – 28 claims have a record of historic work, as described in more detail below. The remaining CAD claims, as well as all Flow and FC claims, were staked and worked for the first time by Silver Quest in late 2009 and 2010.

The area covered by the WCK property was originally staked as the Aries claims in 1969 by Quintana Mineral Corporation following a regional-scale stream sediment survey that identified anomalous gold, arsenic and antimony values from a creek draining the area. Quintana performed grid soil sampling, mapping and bulldozer trenching in 1970. No assessment report was filed and the claims were allowed to lapse following the work.

The area explored by Quintana was restaked as the Eyrie claims by D. Millburn in 1975 and as the Hope claims by O. Davis in 1994; however, there is no public record of work performed. S. Ryan restaked part of the original Aries claims as the CU claims in 1998. Work performed included rock, soil and silt sampling (Ryan, 1999). These claims are still active and lie immediately north of the WCK property.

Prime Properties staked the OHGO claims in 1999. These claims were located two kilometres southeast of the CU claims and included an area that is now part of the WCK property. Prime Properties optioned its claims to Prospector International Resources Inc., which performed cursory geochemical sampling. The highest stream sediment sample values collected from creeks draining the WCK property were 17 ppb Au, 222 ppm As, 4.5 ppm Sb, 45 ppm Cu, 45 ppm Pb and 90 ppm Hg. Peak soil sample values were 12 ppb Au, 52 ppm As, 1.5 ppm Sb, 35 ppm Cu, 69 ppm Hg and 62 ppm Pb (Smith, 2010a). Deltango Gold Limited staked the Au and P claims in 1999 with the P claims adjoined the OHGO claims on the north, east and west sides and the Au claims on the west side. Deltango performed geochemical sampling and geological mapping (Smith, 2010a). The assessment report for this work shows detailed statistical analyses done on the rock and soil samples; however, there are no sample location maps. The OHGO, Au and P claims were subsequently allowed to lapse.

S. Ryan staked the Wolf claims in February 2009. These claims lie immediately west of the WCK property. ATAC staked the WCK property in June 2009 and then sold the property to Silver Quest in December 2009.

ATAC staked the CAD 1 – 28 claims in June 2009 on the basis of a 1986 regional stream sediment survey performed on NTS map sheets 115N and 115O by the Geological Survey of Canada (Friske et al., 1986). Samples were collected from two creeks draining CAD claims 1 – 28. The sample from the more western creek returned 99th percentile mercury (145 ppb) and copper (106 ppm) with 80th percentile gold (6 ppb) and background arsenic (Smith, 2010b). The sample from the eastern creek yielded 99th percentile mercury (120 ppb), 98th percentile arsenic (19 ppm) and background copper and gold. ATAC also sold this property to Silver Quest in December 2009.

A Rimfire/Northgate Joint Venture carried out a regional silt sampling program in selected areas of the Dawson Range in June 2006, directed at locating Pogo-style intrusion-related gold targets that resulted in staking of the Boulevard property (Roberts and Baker, 2007). During the course of this program a total of nine silt samples were collected from areas now encompassed by the Flow property. Five of these silts contained detectable gold, and two of these contained anomalous values of 129 ppb and 32 ppb Au. The silt with 129 ppb Au also contained anomalous As (18 ppm). No ground was staked in the area at the time and thus the work was never filed.

The bedrock geology of the region was most recently compiled by Gordey and Ryan (2005); the section of this map that illustrates the geology surrounding the Flow property is shown in Figure 3.

## 2010 Exploration Program

The Flow property was worked on June 14 – 16, July 28, August 8 – 12 & 31, and September 21 & 22, 2010. On each of these days the work was done by crews of between 2 – 5 soil samplers that were sometimes accompanied by prospectors and/or geologists. In the course of this program 28 new rock samples were collected from the Flow property along with 23 silts and 1018 soils. These samples were added to the existing database of one rock from 2006, nine silts from 2006 and 141 soils from 2009.

The 2010 soil sampling program comprised several contour sampling lines designed to test basins with anomalous Au-in-silt, “ridge-and-spur” type traverse lines along the prominent features underlying the property, and two grids designed to follow-up on anomalies identified through contour and/or ridge-and-spur lines. One of these grids is located in the northwest part of the property whereas the other is located closer to the centre. Sample spacing along all soil sampling lines was mostly 100 m except for some of the ridge-and-spur traverses which had 50 m sample spacing (Figure 4).

Samples were submitted to the ALS Laboratory Group preparation facility in Whitehorse and were analysed for a 51-element high precision suite (ME-MS41) and gold by 30 g fire assay (Au-AA23). Assay certificates of analysis are presented in an Appendix at the end of this report. Soil sample sites are marked with blue and orange flagging and a Tyvek tag, and all sample locations were recorded by hand-held GPS.

A magnetic declination of 22° E was used for all compass measurements. All maps and UTM coordinates are referenced to the 1983 North American Datum (NAD-83; Zone 7).

# REGIONAL GEOLOGY AND MINERALIZATION

## Geology

The Flow property is located within the Yukon-Tanana Terrane in west-central Yukon, a pericratonic terrane of continental affinity that occupies a position between the ancestral continental margin of North America and outboard accreted terranes (Colpron, 2006). The pericratonic terranes were accreted in the Early Jurassic, while subsequent outboard exotic terranes began accreting in the Early to mid-Cretaceous. At this time, the pericratonic package was intruded by a voluminous, northwest-to southeast-trending plutonic suite, referred to as the Dawson Range plutonic belt by Hart et al. (2004) (Klotassin batholith of Templeman-Kluit, 1974), which lies south of the Flow property.

Figure 3: Regional geology of the Flow property

Regional-scale geological compilation maps (e.g. Gordey and Ryan, 2005) show that the Flow property is underlain by several units of country rock schist and granitoids (Figure 3). The oldest unit on the property is Devonian to Mississippian, banded to massive, grey to white quartzite (DMq) that occurs mostly in the eastern part of the property. This unit was thrust overtop of Permian Klondike schist (PKs), which occurs west of the quartzites and typically comprises muscovite-chlorite-quartz-feldpsar schist, chlorite phyllonite and locally deformed lapilli tuff. Both of these predominantly metasedimentary units are intruded by Permian, Jurrassic and Cretaceous granitoids. Granitoids of Permian age (Poga?), which consist mostly of K-feldspar augen gneiss, are the most abundant and underlie much of the central part of the Flow property. The soutwestern part of the property is underlain by the northeastern-most tip of a large Jurassic pluton (EJgd) formed by granodiorite, monzogranite, quartz monzonite and/or quartz monzodiorite. A relatively small Cretaceous intrusion (Kg) - comprising pink to grey, locally porpohyritic, syenogranite to monzogranite - occurs in the northern part of the property. The Cretaceous Carmacks Group (uKCv) overlies Permian to Jurassic rocks in the southern part of the property and consists of andesite and basalt flows and breccias.

## Mineralization

The nearest significant example of gold mineralization to the Flow property is the Coffee property of Kaminak Gold, located just 15 km to the southeast, where significant gold discoveries were announced in spring 2010 (Carpenter, 2011). No resource has been calculated for the Coffee but early drill results include intercepts of 17.1 g/t Au over 15.5 m and 6.3 g/t Au over 35 m within a 15 km long Au in soil anomaly. At Coffee, gold is hosted in both N-S and E-W trending structures (Carpenter, 2011) that overprint regional gneissic and schistose deformation fabrics, indicating that gold is tectonically late. Gold-bearing zones at Coffee are characterised by silicification plus sericite-clay alteration with variable As-Ag-Sb-Ba-Mo enrichment, and the dominant sulphide phase is arsenic-rich pyrite. Gold mineralization at Coffee is hosted by many different rock-types including augen orthogneiss, hornblende-biotite granite, schist and dacite dykes.

No bedrock mineralization is known from the Flow property although prospecting in 2010 did identify several boulders with stibnite- and arsenopyrite-bearing quartz veins, as well as tourmaline-rich conglomerates (see Section 7.1). Furthermore, the presence of Klondike schist on the Flow property, as well as significant lengths of contact zones between granitoid and metasedimentary rocks (Figure 3), defines a favourable geological setting for Coffee-type gold mineralization.

# GEOCHEMISTRY

## Rock Geochemistry

Twenty-seven rock samples were collected from the Flow property in 2010, in four man-days of prospecting. Precious and base metal values for all 27 samples are low. Several samples, however, are cut by sulphide-bearing quartz veins and stringers with trace amounts of Au (6 – 24 ppb) and Ag (2 – 9 ppm), as well as up to 1,985 ppm Cu, 659 ppm Zn, 4,120 ppm As and >10,000 ppm Sb. Sulphide minerals associated with these quartz veins/stringers include chalcopyrite, pyrite, stibnite and galena.

Other rock types collected on the Flow property include tourmaline-rich matrix-supported conglomerate, moderately sericitized and/or chloritized granitoids, chunks of hematite-bearing quartz veins, and pyrite-bearing schists.

## Soil Geochemistry

Soil results for the 1018 samples collected during the 2010 Flow program have been compared with the larger (14,000+) database of soil analyses from numerous Silver Quest properties across the Dawson Range. Based on this larger dataset the Flow property shows several in-soil anomalies of Au and Au-indicator elements that are described below.

The most significant Au-in-soil anomaly occurs in the southwestern corner of the Flow property, and comprises two parallel strings of about 15 samples each that contain up to 53 ppb Au. Two smaller Au-in-soil anomalies occur successively north of this larger anomaly. The more southerly of these, which borders the western margin of the property, consists of three consecutive anomalous Au-in-soils that are coincident with high As. This anomaly is located upslope from a silt sample, collected in 2006, with 129 ppb Au. Even further north lies a point anomaly with 343 ppb Au, which is the fifth highest Au-in-soil value in the Dawson Range database (14,000+ analyses).

There are numerous other apparently anomalous Au-in-soil values that are most likely the result of analytical irregularities from a single analytical certificate (WH10079095). These samples define two lines of samples in the central part of the property, one of which is more east – west trending and the other north – south. The more east – west trending line is most obviously problematic since it was re-tested with a grid of north – south trending sampling lines that returned below detection for Au. The more north – south trending line of Au anomalies was not re-tested, and its southern extent is spatially associated with several other high Au values obtained from separate soil lines. All or parts of this line may be a true anomaly, although further testing with follow-up grids is required for confirmation. According to ALS Minerals, the Au-AA23 analyses all fall within acceptable ranges for these low concentration levels. They recommend the more precise Au-ICP21 method which provides a 1 ppb detection level for Au for future soil samples where low gold concentrations are anticipated (see 2 page investigation report in Appendix G).

The Flow property also shows several soil anomalies of As, Sb and Mo that are considered genuine and possible pathfinders for gold mineralization. The largest such anomaly occurs in the southeastern corner of the Flow property and comprises a large As-in-soil anomaly with coincident but smaller anomalies of Sb and Au. The maximum dimensions of this As-in-soil anomaly are about 1.7 km in a north – south orientation and 1.2 km from east to west. Just a few hundred meters to the north and northeast of this anomaly are two smaller and coincident As- and Sb-in-soil anomalies. There are also two Mo-in-soil anomalies within the central part of the Flow property, with the largest measuring about 800 m in length, in addition to a few small and scattered As ± Mo anomalies in the northwest.

## Silt Geochemistry

Of the 23 silt samples taken from the Flow property in 2010, 11 returned detectable gold and two contained weakly to moderately anomalous Au (26 and 39 ppb respectively). The more strongly anomalous sample is spatially associated with a soil sample containing 30 ppb Au, and occurs in a basin with a few scattered Au-in-soil point anomalies. However, this basin was fairly intensely sampled, since it returned a strong suspect Au-in-soil anomaly (see section 7.2), and the 2010 surveys failed to identify a coherent Au-in-soil anomaly. The weaker of the two anomalous silts, which was collected from a stream in between the two suspect Au-in-soil anomalies, also contains high As. Furthermore, this sample was collected from a basin along with eleven other silts, four of which contain detectable gold.

The 2006 silting program identified two anomalous samples (129 and 32 ppb Au), the weaker of which was re-assessed in the 2010 program. This re-assessment involved collection of four additional silts along the same creek, each of which lacked detectable Au. In addition, sampling of the drainage basin showed just a few scattered moderately high Au-in-soil point anomalies. The creek with 129 ppb Au, which occurs just northwest of the large As-in-soil anomaly, was not re-tested with silts. However, soil sampling shows that one of the small Au-in-soil anomalies occurs just up the ridge from this high Au-in-silt, and that a coincident As-Sb anomaly occurs just downstream of it.

# PROPERTY GEOLOGY AND MINERALIZATION

The significant As-Sb ± Au ± Pb ± Mo ± W ± Bi in-soil anomaly delineated in 2009 (Smith, 2010a) on the WCK claims was prospected in 2010. The area is characterized by steep slopes with abundant blocky talus and poor soil development. These soil samples would be better described as talus fine samples. Nonetheless, mineralization is present in float within parts of the anomaly. Near a significant Sb anomaly a vein with coarse-grained stibnite with euhedral, vuggy quartz was discovered. The vein is about 10 cm wide and is part of a distinct set of veins generally about 1 cm wide with stibnite and minor chalcopyrite. These veins are hosted within an undeformed (likely Cretaceous) granodiorite. In general the veins exposed are abundant enough to explain the soil anomaly and exposure, albeit in talus, is very good.

South of the soil line, the granodiorite is in contact with a distinct conglomerate unit characterized by 1 to 10 cm fragments of white quartz and quartz-muscovite schist. There is no outcrop exposure of the actual contact so its nature is unknown. The conglomerate unit is poorly sorted but locally exhibits crude layering (presumably sedimentary). This unit likely formed during Eocene extension at the time of emplacement of Carmacks Group volcanic rocks (exposed to the north). Potentially significantly, the cement of this unit is dark brown-black and contains abundant tourmaline (Photo 1) however rock samples of this material did not return significant precious metal values.

Photo 1: Tourmaline cemented conglomerate from the WCK claims. Fragments are strongly deformed quartz-muscovite schist and white quartz.

# DISCUSSION AND CONCLUSIONS

Geochemical analysis has defined several impressive Au-in-soil anomalies on the Flow property that, unfortunately, are most likely related to gold concentrations that are very close to the analytical precision. When the values returned from the lab are compared with the regional database, these samples appear to outline coherent anomalies. Follow-up work, however, failed to replicate these values and the anomalies from certificate WH10079095 need to be treated with caution.

The largest anomaly of Au-indicator elements (in this case As ± Sb) occurs in the southeastern portion of the property where it overlies a contact zone between Jurassic granitoids and the Cretaceous Carmacks Group, which is not a known setting for gold mineralization in the White Gold district. Other anomalies of Au-indicator elements occur without enrichment of Au-in-soil.

The most compelling targets within the Flow property are small. These include the three consecutive samples of anomalous Au- and As-in-soil along the southwestern margin of the property, as well as the point anomaly with 348 ppb Au. The three point anomaly occurs uphill from a significant Au-in-silt anomaly identified in the 2006 program (129 ppb). The geology of these two sites is also more favourable, with both occurring at contacts between granitoids and country rock metasediments. Previous discoveries in the White Gold district have been made off of point anomalies so it may be worthwhile to conduct follow-up sampling on detailed grids at these two sites.

Respectfully submitted,

_________________________

Darcy Baker Ph.D., P.Geo.

_________________________

Ron Voordouw Ph.D.

EQUITY EXPLORATION CONSULTANTS LTD.

Vancouver, British Columbia

April 24, 2011

Appendix A: Bibliography

REFERENCES

Carpenter, R., 2011, Geology of New Lode Gold Discoveries in the Coffee Creek Area; White Gold District, West-Central Yukon: Mineral Exploration Roundup 2011, Vancouver, BC, 2011, p. 48-50.

Colpron, M., 2006, Tectonic assembly map of the Yukon -- Tanana and related terranes in Yukon and British Columbia (1:1,000,000), Yukon Geological Survey.

Friske, P. W. B., Hornbrook, E. H. W., Schmitt, H. R., Galletta, A. C., Ellwood, D. J., and  McGurdy, M., 1986, Regional stream sediment and water geochemical reconnaissance data, Yukon. NTS 115N (E1/2), 115O, Geological Survey of Canada.

Gordey, S. P., and  Ryan, J. J., 2005, Geology, Stewart River Area (115N, 115O and part of 115J, Yukon Territory, Geological Survey of Canada Open File 4970.

Hart, J. R., Goldfarb, R., Lewis, L. L., and  Mair, J. L., 2004, The northern Cordilleran mid-Cretaceous plutonic province: Ilmenite/magnetite-series granitoids and intrusion-related mineralization: Resource Geology, v. 54, p. 253-280.

Ryan, S., 1999, Geochemical report on the CU property, Dawson Mining District, p. Assessment Report 094074.

Smith, H., 2010a, Assessment report describing soil geochemical sampling at the WCK property, Archer, Cathro & Associates (1981) Ltd., p. 23.

Smith, H., 2010b, Assessment report describing soil sampling at the CAD property, Archer, Cathro & Associates (1981) Ltd, p. 23.

Templeman-Kluit, D. J., 1974, Reconnaissance geology of Aishihik Lake, Snag and part of Stewart River am-areas, west-central Yukon (115H, 115K-J and 115N-O), Geological Survey of Canada, p. 97.

Appendix B: Statement of Expenditures

Appendix C: Laboratory Certificates

Appendix D: Rock Sample Descriptions

Appendix E: Data Disc

Report text, geochemical databases, drafting and plot files

Appendix F: Geologist’s Certificates

# GEOLOGIST'S CERTIFICATE

# Darcy E.L. Baker

# Suite 114 – 2635 Prince Edward Street,

# Vancouver, BC, Canada

I, Darcy Baker, P.Geo. do hereby certify that:

- I am a Geologist and President of Equity Exploration Consultants Ltd., with offices at Suite 200–900 West Hastings Street in the City of Vancouver, B.C., in the Province of British Columbia.

- I am a member in good standing (#33,448) of the Association of Professional Engineers and Geoscientists of British Columbia.

- I am a graduate of Dalhousie University (1997) with an Honours Bachelor of Science degree in Geology, and am a graduate of the University of Newcastle, Australia (2003) with a Doctor of Philosophy degree in Geology, and I have practiced my profession continuously since 1997.

- Since 1997 I have been involved in mineral exploration for gold, silver, copper, uranium, molybdenum, lead and zinc across Canada and in Alaska, Nevada and Australia.

- I am a Consulting Geologist and principal of Equity Exploration Consultants Ltd. (a geological consulting and contracting firm) and have been so since May 2003.

- I managed the Yukon field exploration campaigns for Silver Quest during 2010.

- I am independent of Silver Quest.

Dated at Vancouver, British Columbia, this 24th day of April, 2011.

___________________________

Darcy Baker, Ph.D., P.Geo.

# GEOLOGIST'S CERTIFICATE

# Ron Voordouw

# Vancouver, BC, Canada

I, Ron Voordouw do hereby certify that:

- I am a Project Geologist with Equity Exploration Consultants Ltd., with offices at Suite 200–900 West Hastings Street in the City of Vancouver, B.C., in the Province of British Columbia.

- I am a graduate of the University of Calgary (1999) with an Honours Bachelor of Science degree in Geology, and am a graduate of the Memorial University (2006) with a Doctor of Philosophy degree in Geology, and I have practiced my profession continuously since 1999.

- I managed the Yukon field exploration campaigns for Silver Quest during 2010.

- I am independent of Silver Quest.

Dated at Vancouver, British Columbia, this 24th day of April, 2011.

___________________________

Ron Voordouw, Ph.D.

Appendix G: ALS Minerals Memo