Assessment of Airborne Magnetics and Radiometrics Surveys at the Mint Prospect

A 272 line km helicopter magnetic and radiometric survey has been completed over the Mint project by New-Sense Geophysics Ltd. (New-Sense) for Strategic Metals Ltd. (Strategic Metals). The survey area covers the previously recognized porphyry Cu-Mo-Au showing (Canyon Mountain) that is located 335 km north-west of Whitehorse in the Yukon Territory. Condor Consulting, Inc. (Condor) has been commissioned to assess the data sets and provide a 3D model of the magnetics. Refer to New-Sense’s logistic report (HMR100806) for any additional survey details. Figure 1 shows the location of the survey area and flight path.

Figure 1: Mint airborne magnetics and radiometrics survey flight path.

The range of the magnetic response across the survey area is approximately 5,200 nT. The response of the magnetics is dominated by a large circular complex (1.5 km in diameter) located in the middle of the survey area survey area as seen in Figure 2 and is coincident with the Canyon Mountain showing. Anomalous stream sediment Cu and As samples (up to 75 and 5 ppm respectively) were collected in the south-east portion of the survey area. It is recommended that the zones corresponding to the interpreted structures be followed up with a field check and subsequent mapping and sampling. Note the probable flight line effects seen as linear east-west features to the north-west of the magnetic complex that corresponds with a pronounced ridge.

Figure 1: Mint total magnetic intensity image and interpreted structural lineaments.

The University of British Columbia (UBC) 3D magnetic inversion program MAD3D (version 4.0) was used to produce a model of the magnetics data. MAG3D is a program library for carrying out forward modeling and inversion of surface, airborne, and/or borehole magnetic data in the presence of a three dimensional Earth. Data are assumed to be the anomalous magnetic response to buried susceptible material, not including Earth's ambient field. The model is specified using a mesh of rectangular cells, each with a constant value of susceptibility, and topography is included. The magnetic response can be calculated anywhere within the model volume, including above the topography, simulating ground or airborne surveys, and inside the ground simulating borehole surveys. Figure 3 displays the 3D inversion results of the magnetics data at Mint.

Figure 3: Mint 3D magnetic model.

The elevated radiometrics response appears for the most part to correlate with higher topographic relief and outcrop, while the lows can be seen associated with the drainages and the valley floor.

The following products can be found and downloaded from the Condor ftp site (ftp://ftp.condorconsult.com , user id: archer@condorconsult.com , password: skywalker19):

- Summary report of the assessment

- Registered images of the airborne magnetics and radiometrics (NAD83, Zone 7N)

- 3D magnetic model and associated sections

- ArcGIS formatted .shp file of the interpreted lineaments derived from the magnetics

It is recommended that the results of the airborne magnetics and radiometrics data be compared with any available geologic and geochemical information in order to help advance the exploration program at Mint.

Respectfully submitted;

Mark Goldie

Condor Consulting, Inc.

December 2, 2010

References:

Li, Y. and Oldenburg, D. W., 1996, 3-D Inversion of Magnetic Data: Geophysics, 61, no. 02, 394-408.

Yakovenko, A., Logistics Report for the High Resolution Helicopter Magnetic and Gamma-ray Spectrometric Airborne Geophysical Survey flown over Mint, Nikki, Corky, Meloy, King, and Mars Project Properties, Yukon, from White River Lodge (Mint and Nikki), Burwash Landing (Corky and Meloy), and Braeburn Lodge (King and Mars), Yukon carried out on behalf of Strategic Metals Ltd. by New-Sense geophysics Limited, Project # HMR100806, October 2010.