Assessment of Airborne Magnetics and Radiometrics Surveys at the DDD Prospect

A 328 line km helicopter magnetic and radiometric survey has been completed over the DDD prospect (Klotassin project) by New-Sense Geophysics Ltd. (New-Sense) for Strategic Metals Ltd. (Strategic Metals). DDD is reported to be a grassroots stockwork (White Gold Type) prospect located approximately 200 km north-west of Whitehorse in the Yukon Territory. Condor Consulting, Inc. (Condor) has been commissioned to assess the data sets and provide a 3D model of the magnetics. Refer to New-Sense’s logistic report (HMR100816) for any additional survey details. Figure 1 shows the location of the survey area and flight path.

Figure 1: DDD airborne magnetics and radiometrics survey flight path.

The range of the magnetic response across the survey area is approximately 1000 nT. The results show several distinct zones of elevated magnetism and interpreted structural breaks that are shown in Figure 2.  The NW trending series of magnetic anomalies appear to correspond with the ultramafic rocks outlined on the Yukon Geologic Survey 1:250,000 geology map, however, the magnetic response does not appear to be as continuous as the mapping suggests. The interpreted lineaments were derived from the total magnetic intensity image and associated filter products (1st and 2nd vertical derivative, tilt derivative, total gradient, upward continuation). Anomalous As and Au (up to 15 ppm and 149 ppb respectively) measurements derived from the regional stream sediment sampling occur in the vicinity of the survey area.

Figure 1: DDD total magnetic intensity image and interpreted structural lineaments.

The University of British Columbia (UBC) 3D magnetic inversion program MAD3D (version 4.0) was used to produce a model of the magnetics data. MAG3D is a program library for carrying out forward modeling and inversion of surface, airborne, and/or borehole magnetic data in the presence of a three dimensional Earth. Data are assumed to be the anomalous magnetic response to buried susceptible material, not including Earth's ambient field. The model is specified using a mesh of rectangular cells, each with a constant value of susceptibility, and topography is included. The magnetic response can be calculated anywhere within the model volume, including above the topography, simulating ground or airborne surveys, and inside the ground simulating borehole surveys. Figure 3 displays the 3D inversion results of the magnetics data at DDD.

Figure 3: DDD 3D magnetic model.

A distinct K low in the radiometrics data set loosely corresponding with the magnetic anomaly centered at 674, 400E and 6,884,200N and is contrary to the general response seen associated with areas of higher relief. It is recommended that this area be followed up with a field visit to check for possible exposed alteration.

The following products can be found and downloaded from the Condor ftp site (ftp://ftp.condorconsult.com , user id: archer@condorconsult.com , password: skywalker19):

- Summary report of the assessment

- Registered images of the airborne magnetics and radiometrics (NAD83, Zone 8N)

- 3D magnetic model and associated sections

- ArcGIS formatted .shp file of the interpreted lineaments derived from the magnetics

It is recommended that the results of the airborne magnetics and radiometrics data be compared with any available geologic and geochemical information in order to help advance the exploration program at DDD.

Respectfully submitted;

Mark Goldie

Condor Consulting, Inc.

November 11, 2010

References:

Li, Y. and Oldenburg, D. W., 1996, 3-D Inversion of Magnetic Data: Geophysics, 61, no. 02, 394-408.

Yakovenko, A., Logistics Report for the High Resolution Helicopter Magnetic and Gamma-ray Spectrometric Airborne Geophysical Survey flown over Klotassin Project Properties: BBB, CCC and DDD Blocks, YT, Canada from Carmacks, Yukon carried out on behalf of Strategic Metals Ltd. by New-Sense geophysics Limited, Project # HMR100816, October 2010.