Assessment of Airborne Magnetics and Radiometrics Surveys at the Meloy Prospect

A 293 line km helicopter magnetic and radiometric survey has been completed over the Meloy project by New-Sense Geophysics Ltd. (New-Sense) for Strategic Metals Ltd. (Strategic Metals). The survey area covers the previously drilled porphyry Cu-Mo-Au prospect (Raft) that is located 190 km north-west of Whitehorse in the Yukon Territory. Condor Consulting, Inc. (Condor) has been commissioned to assess the data sets and provide a 3D model of the magnetics. Refer to New-Sense’s logistic report (HMR100806) for any additional survey details. Figure 1 shows the location of the survey area and flight path.

Figure 1: Meloy airborne magnetics and radiometrics survey flight path.

The range of the magnetic response across the survey area is approximately 1,100 nT. The northern half of the survey area is dominated by a strong magnetic response with a pair of discrete lows in areas of lower topography (Figure 2), while the southern portion of the survey area exhibits a more subdued magnetic response including a series of south-east trending structures. Anomalous stream sediment Cu and As samples (up to 57 and 16 ppm respectively) were collected in the south-west corner of the survey area. It is recommended that the zones corresponding to the interpreted structures be followed up with a field check and subsequent mapping and sampling.

Figure 1: Meloy total magnetic intensity image and interpreted structural lineaments.

The University of British Columbia (UBC) 3D magnetic inversion program MAD3D (version 4.0) was used to produce a model of the magnetics data. MAG3D is a program library for carrying out forward modeling and inversion of surface, airborne, and/or borehole magnetic data in the presence of a three dimensional Earth. Data are assumed to be the anomalous magnetic response to buried susceptible material, not including Earth's ambient field. The model is specified using a mesh of rectangular cells, each with a constant value of susceptibility, and topography is included. The magnetic response can be calculated anywhere within the model volume, including above the topography, simulating ground or airborne surveys, and inside the ground simulating borehole surveys. Figure 3 displays the 3D inversion results of the magnetics data at Meloy.

Figure 3: Meloy 3D magnetic model.

The elevated radiometrics response appears for the most part to correlate with higher topographic relief and outcrop, while the lows can be seen associated with the drainages and the valley floor.

The following products can be found and downloaded from the Condor ftp site (ftp://ftp.condorconsult.com , user id: archer@condorconsult.com , password: skywalker19):

- Summary report of the assessment

- Registered images of the airborne magnetics and radiometrics (NAD83, Zone 7N)

- 3D magnetic model and associated sections

- ArcGIS formatted .shp file of the interpreted lineaments derived from the magnetics

It is recommended that the results of the airborne magnetics and radiometrics data be compared with any available geologic and geochemical information, in particular with the historic drilling, in order to help advance the exploration program at Meloy.

Respectfully submitted;

Mark Goldie

Condor Consulting, Inc.

December 2, 2010

References:

Li, Y. and Oldenburg, D. W., 1996, 3-D Inversion of Magnetic Data: Geophysics, 61, no. 02, 394-408.

Yakovenko, A., Logistics Report for the High Resolution Helicopter Magnetic and Gamma-ray Spectrometric Airborne Geophysical Survey flown over Mint, Nikki, Corky, Meloy, King, and Mars Project Properties, Yukon, from White River Lodge (Mint and Nikki), Burwash Landing (Corky and Meloy), and Braeburn Lodge (King and Mars), Yukon carried out on behalf of Strategic Metals Ltd. by New-Sense geophysics Limited, Project # HMR100806, October 2010.