Aldrin Resource Corporation

2010 GEOCHEMICAL REPORT ON THE IND PROJECT

| YC36103 – YC36112 | IND 1 – IND 10 |
| --- | --- |
| YC44987 – YC44996 | IND 11 – IND 20 |
| YC61018 – YC61039 | IND 21 – IND 42 |
| YC95578 – YC95579 | IND 105 – IND106 |
| YC96101 – YC96200 | IND 43 – IND 103 IND 107 – IND 130 |

Located in the Dawson Area

Dawson Mining District

NTS 115O/13

63°50’ N Latitude; 139°33’ W Longitude

Work Performed September 23rd – October 1st, 2010

-prepared for-

ALDRIN RESOURCE CORPORATION

Suite 2020-401 West Georgia Street

Vancouver, British Columbia, Canada, V6B 5A1

-prepared by-

David Swanton, M.Sc.

EQUITY EXPLORATION CONSULTANTS LTD.

Suite 200, 900 West Hastings Street

Vancouver, British Columbia, Canada, V6C 1E5

January 20, 2011

TABLE OF CONTENTS

TABLE OF CONTENTS	1

LIST OF APPENDICES	1

LIST OF TABLES	1

LIST OF FIGURES	1

1.0	SUMMARY	2

2.0	INTRODUCTION	2

3.0	RELIANCE ON OTHER EXPERTS	2

4.0	PROPERTY DESCRIPTION AND LOCATION	2

5.0	ACCESSIBILITY, CLIMATE, LOCAL RESOURCES, INFRASTRUCTURE, PHYSIOGRAPHY	6

6.0	REGIONAL AND PROPERTY GEOLOGY	6

7.0	HISTORY	6

7.1	Previous Work	6

7.2	2010 Program	7

8.0	GEOCHEMISTRY	9

9.0	THIN SECTION PETROGRAPHY	9

10.0	DISCUSSION AND CONCLUSIONS	13

LIST OF APPENDICES

Appendix A: Bibliography

Appendix B: Statement of Expenditures

Appendix C: Trench sample descriptions and weighted averages

Appendix D: Petrographic Report

Appendix E: Certificates of Analysis

Appendix F: Compact Disc

Appendix G: Geologist’s Certificate

LIST OF TABLES

Table 1: Tenure details for the IND Claims	3

Table 2: Percentile values for 2010 trench samples	9

Table 3: Correlation matrix for 2010 trench samples	9

Table 4: Summary of significant assays from Trench 4	9

LIST OF FIGURES

Figure 1: Location Map	4

Figure 2: Tenure information	5

Figure 3: Regional Geology of the IND area	8

Figure 4a: 2010 trench locations	10

Figure 4b: 2010 trench 4 sample locations and gold values	11

Figure 4c: 2010 trench 5 sample locations and gold values	12

# SUMMARY

The IND property lies approximately 25 km south-southwest of Dawson City, within the Dawson Mining district of the Yukon Territory.  The property consists of 136 claims totalling approximately 3000 hectares, which have been optioned from Ryanwood Exploration by Aldrin Resource Corp.

Access to the property is possible via maintained secondary roads from Dawson City.  The area has a sub-arctic climate with discontinuous permafrost.  Elevation in the region ranges from 1100 m on ridge tops to 330 m at drainage base, the Yukon River.  The claim block lies within the Yukon-Tanana Terrane, and is underlain by metamorphic rocks of the Devonian-Mississippian Nasina Assemblage and the Permian Jim Creek Pluton.

Previous work in the area has been limited.  Between 2005 and 2008, Ryanwood staked the IND claims, conducted extensive soil sampling and a ground-based geophysical survey.  The results of these surveys were sufficiently encouraging to justify a mapping and rock sampling program in 2009 by Jean Pautler, P.Geo.

The 2010 program consisted of geochemical and petrographic analysis of rock samples from two trenches with a total length of 685m.  The eastern trench (TR10-4) contained several zones with highly anomalous gold grades (up to 12 g/t over 30 cm and 2 g/t over 10 m), hosted within granite interpreted to be part of the Jim Creek Pluton.  Six samples were submitted for petrographic analysis, showing evidence of hydrothermal alteration both within the granite of the pluton and in the country rock.  Based on the results of the programs of this and previous years, it is concluded that the IND property has sufficient potential for widespread gold mineralization to warrant additional exploration.

# INTRODUCTION

Equity Exploration Consultants Ltd (“Equity”) was contracted by Aldrin Resource Corporation (“Aldrin”) to compile the results of a trenching and petrographic analysis program carried out by Aldrin personnel on the IND property in September, 2010.  The purpose of the work was early stage exploration of the property for gold and/or other economic metals.  The information used in the compilation of this report consists of assessment reports filed with the Yukon Department of Mines, Energy and Resources, maps and reports from the Geological Survey of Canada and private information supplied by Aldrin.

# RELIANCE ON OTHER EXPERTS

The author has not relied on a report, opinion or statement of an expert for information concerning legal, environmental, political or other issues.

# PROPERTY DESCRIPTION AND LOCATION

The IND property is located approximately 25 km south-southwest of Dawson City and 5 km east of the Yukon River (Figure 1).  The claim block is centred at 63°50’ north latitude and 139°33’ east longitude on NTS map sheet 115O/13.

The IND claim block consists of 136 adjoining quartz claims with a total area of approximately 3000 hectares (Table 1, Figure 2).  Claims were originally staked and are still owned by Ryanwood Exploration, subject to an option agreement under which Aldrin can earn a 100% interest through work commitments and payments of cash and shares to Ryanwood.

There are no known environmental liabilities on the property.

Work in 2010 was conducted under a Class 1 Quartz Mining Land Use permit.

Table 1: Tenure details for the IND Claims

| Grant Numbers | Claim Names | Recording Date | Staking Date | Expiry Date |
| --- | --- | --- | --- | --- |
| YC36103 – YC36112 | IND 1 – IND 10 | 6/2/2005 | 5/26/2005 | 3/15/2015 |
| YC44987 – YC44996 | IND 11 – IND 20 | 10/3/2006 | 9/22/2006 | 3/15/2014 |
| YC61018 – YC61039 | IND 21 – IND 42 | 6/4/2007 | 5/28/2007 | 3/15/2015 |
| YC95578 – YC95579 | IND 105 – IND106 | 6/25/2009 | 6/16/2009 | 3/15/2013 |
| YC96101 – YC96200 | IND 43 – IND 103 IND 107 – IND 130 | 6/25/2009 | 6/16/2009 | 3/15/2013 |

Figure 1: Location Map

Figure 2: Tenure information

# ACCESSIBILITY, CLIMATE, LOCAL RESOURCES, INFRASTRUCTURE, PHYSIOGRAPHY

The IND property lies approximately 25 km southwest of Dawson city, near the confluence of the Yukon and Indian Rivers.  There is a well-developed network of good secondary roads which allow road access from Dawson City to the property.  Dawson City is the nearest supply and population centre, with all equipment and personnel mobilized by road from there to the IND property for the 2010 work program.

The climate of the area is classified as sub-arctic, with long cold winters and short cool summers.  Permafrost is widespread.  The area is generally snow-free from early June until late September, making field work outside of this time period difficult.  The claims straddle a gently sloping ridge with peaks up to 1100 m elevation, drained by creeks which flow into either the Yukon or Indian rivers.  The Yukon River forms a local base level at 330 m.  Vegetation in the region consists of conifer and deciduous forests in valleys and on ridge-flanks, with sub-alpine scrub vegetation on the ridge-tops.  North facing slopes are generally frozen year round and poorly drained.

# REGIONAL AND PROPERTY GEOLOGY

The IND property lies within the Yukon-Tanana Terrane, a series of mid-Paleozoic to Mid-Mesozoic continental arc assemblages built on Lower Palaeozoic and possibly older continental basement.  The terrane is generally composed of variably deformed metamorphic rocks including pelitic and quartzofeldspathic schist and paragneiss, felsic orthogneiss, and mafic to felsic metavolcanic and metaplutonic rocks, all of which are intruded by plutonic suites that range in age from Paleozoic to Neogene (Mortensen 1992).  In the Stewart River area (the map sheet on which the IND property is located) bedrock geology is dominated by Devonian to Carboniferous metasedimentary units intruded by Permian to Cretaceous igneous bodies.  Underlying the IND claim block itself (Figure 3) are quartzite, metapelite and marble of the Devonian-Mississippian Nasina Assemblage intruded by the ~252 Ma Jim Creek Pluton, a coarse-grained unfoliated biotite-bearing granite to quartz monzonite (Gordey and Ryan 2005).

Of particular interest when considering potential gold mineralization on the IND property is proximity to the Klondike placer deposits.  Eldorado, Hunker and Bonanza creeks, all of which have supported placer operations for over a century, lie within 25 km to the northeast.  Pautler (2009) has also suggested that mineralization on the IND property shows similarities to the nearby White Gold deposit.

# HISTORY

## Previous Work

Very little historical work on the immediate IND area is recorded with the Yukon Department of Mines, Energy and Resources.  A small drill program (7 holes totalling 189 feet) for Tamark Inc. on Ensley Creek directly north of the IND block is recorded by Beets (1986) but no results are reported.

The first work reported directly on the IND claims is the geophysical and geochemical program described by Ryan (2008).  At the time of the work, IND 1 – 42 (see Table 1) were held by Ryanwood Exploration.  The rest of the IND block had yet to be staked.  Soil sampling in 2005 and 2006 had identified a gold-in-soil anomaly, and the purpose of the 2007 program was to better define and extend this anomaly.  Geophysical work consisted of a ground-based magnetic survey which delineated two broad magnetic highs, the westernmost of which is roughly co-incident with the previously mapped extent of the Jim Creek Pluton.  The eastern magnetic high does not coincide with the previously mapped extent of the pluton (Gordey and Ryan 2005), however Ryan (2008) states that the pluton may be more extensive than previously shown, and thus the eastern magnetic high may also represent igneous bedrock.  Gold-in-soil anomalies with co-incident lanthanum and bismuth highs are located primarily over these magnetic highs.

This program was followed up with a two day geological and geochemical evaluation by Jean Pautler, described fully in Pautler (2009).  The author produced a 1:7,500 scale map of, and took grab rock samples from, the geochemically anomalous area identified by Ryan (2008).  Results confirm that the Jim Creek Pluton has outliers not recorded on the map of Gordey and Ryan (2005).  Primarily within the granite but extending into country rock, Pautler (2009) identified minor pyrite, limonite after pyrite, hematite, silicification, sericite alteration and quartz stockwork veining.  It was recommended that a trenching and property-wide mapping program be conducted in order to identify promising drill targets.  Based on these recommendations, three trenches were excavated and left unfilled.

## 2010 Program

The 2010 program consisted of the digging of two new trenches with a total length of 685 m.  Equipment was mobilized out of Dawson City along the network of dirt roads running over the property.  A total of 145 samples were generally taken over 5m intervals along the trenches (every attempt was made to obtain an even distribution of material from the entire 5m sample length), though smaller intervals were sampled where warranted.  Samples were submitted to ACME Labs in Vancouver, BC for fire assay fusion followed by ICP-ES analysis for Au (ALS code G601) and Aqua Regia digestion followed by ICP-MS analysis for a 37-element suite (ALS code 1DX15).  Certificates of analysis are presented in Appendix C.  Six samples were submitted to Dr. Tim Liverton for petrographic analysis, the results of which are presented in Appendix D. The trenches were not backfilled upon completion of mapping and sampling.

Figure 3: Regional Geology of the IND area

# GEOCHEMISTRY

145 rock samples were collected from the two trenches dug during the 2010 program.  Percentiles and a correlation matrix for elements of interest from these samples are presented in Tables 2 and 3. There is extremely good correlation between Au and Ag, and between Bi and Au, suggesting that both these elements may be useful as pathfinders of gold on the IND property.  Trench locations are shown on Figure 4a and sample locations with gold analyses on Figures 4b and 4c. Detailed descriptions with assay values for Au, As, Ag, Sb and Bi are presented in Appendix C.  As shown in Figure 4c, Trench 5 encountered little in the way of gold mineralization; however, Trench 4 intersected several zones (all within the granite of the Jim Creek Pluton) of good gold grades (Figure 4b).  Table 4 summarizes the most interesting zones, most notably a 10 m stretch averaging 2.1 g/t Au and one 30 cm sample from a quartz vein which assayed 12.5 g/t Au.

Table 2: Percentile values for 2010 trench samples

|  | Au (ppm) | As (ppm) | Ag (ppm) | Sb (ppm) | Bi (ppm) |
| --- | --- | --- | --- | --- | --- |
| Max Value | 12.46 | 261 | 4.7 | 0.4 | 25 |
| Min Value | 0.005 | 0.7 | 0.1 | 0.1 | 0.1 |
| 98th | 1.43 | 122 | 1.4 | 0.3 | 7.9 |
| 95th | 0.40 | 72 | 0.8 | 0.3 | 2.7 |
| 90th | 0.21 | 50 | 0.7 | 0.3 | 1.5 |
| 80th | 0.11 | 20 | 0.5 | 0.2 | 0.8 |
| 50th | 0.03 | 7.5 | 0.3 | 0.2 | 0.2 |

Table 3: Correlation matrix for 2010 trench samples

|  | Au | As | Ag | Sb |
| --- | --- | --- | --- | --- |
| As | 0.14 |  |  |  |
| Ag | 0.81 | 0.02 |  |  |
| Sb | 0.03 | 0.34 | -0.02 |  |
| Bi | 0.90 | 0.24 | 0.69 | 0.18 |

Table 4: Summary of significant assays from Trench 4

| From | To | Length | Au (g/t) |
| --- | --- | --- | --- |
| 0 | 5 | 5 | 1.186 |
| 56 | 66 | 10 | 1.02 |
| 56 | 76 | 20 | 0.587 |
| 58.7 | 59 | 0.3 | 12.46 |
| 205 | 220 | 15 | 0.821 |
| 280 | 285 | 5 | 4.05 |
| 280 | 290 | 10 | 2.09 |

# THIN SECTION PETROGRAPHY

Six samples were selected for thin section petrographic analysis by Dr. Tim Liverton. The full petrographic report is presented in Appendix D.

Figure 4a: 2010 trench locations

Figure 4b: 2010 trench 4 sample locations and gold values

Figure 4c: 2010 trench 5 sample locations and gold values

# DISCUSSION AND CONCLUSIONS

Though the scope of work in 2010 was limited, it confirmed that significant gold grades are present in bedrock underneath zones of the IND property which had previously yielded anomalous gold-in-soil values.  Mineralization appears to be hosted within the Jim Creek Pluton, and may be related to quartz veining of unknown age and genesis.  Determining the extent of this mineralization and veining should be a priority for any future work.

Respectfully submitted,

_________________________

David Swanton, M.Sc.

EQUITY EXPLORATION CONSULTANTS LTD.

Vancouver, British Columbia

January 21, 2011

Appendix A: Bibliography

REFERENCES

Gordey, S.P. and Ryan, J.J.  2005.  Geology, Steward River Area (115N, 115O and part of 115J), Yukon Territory; Geological Survey and Canada, Open File 4970, scale 1:250,000.

Mortensen, J.K.  1992.  Pre-mid-Mesozoic tectonic evolution of the Yukon-Tanana terrane, Yukon and Alaska.  Tectonics, 11: 836 – 853.

Pautler, J.  2009.  Technical Report on the Indian River Project, Dawson, Yukon Territory.

Ryan, Shawn.  2008.  Geophysical-geochemical report, IND claims.  Yukon Mines, Energy and Resources Report YMIP 2007-047.

Appendix B: Statement of Expenditures

# IND Project – Statement of 2010 Expenditures

# Fieldwork: September 23 – October 1, 2010

2010 exploration work completed on the IND claim block involved excavating two trenches, and a petrographic study and associated support work including:

- Mobilization of excavator from Dawson City to IND claims and excavation of two trenches totaling 685-meters in length averaging approximately 1.5-meters in depth.  Cost:  $3,200

- Fee for consultant Ted Archibald:  $2,500

- Lodging for consultant:  $1,500

- Fee for sample helper:  $1,000

- Petrographic study and report:  $1,200

- Cost of multiple element assays for 147 samples:  $5,145

- Shipping:  $250

- Helicopter for sample pickup:  $350

- Field supplies: $200

- Vehicle rental for 10-days:  $1,000

___________________________________________

Summary of Expenses:

| # Excavator: | # $ 3,200 |
| --- | --- |
| # Consultant and helper: | # 5,000 |
| # Petrographic study and report: | # 1,200 |
| # Assays: | # 5,145 |
| # Shipping and helicopter: | # 550 |
| # Field Supplies: | # 200 |
| # Truck rental: | # 1,000 |
| # Total: | # $ 16,295 |

Appendix C: Trench sample descriptions and weighted averages

|  |  |  |  |  | IND Project |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2010 TRENCH SAMPLE DESCRIPTIONS AND RESULTS |  |  |  |  |  |  |
| SAMPLE # | TRENCH # | LOCATION (m) |  |  |  | Au | Ag | As | Sb | Bi |
|  |  | Start | Finish | Type | DESCRIPTION | g/t | ppm | ppm | ppm | ppm |
| 116201 | TR10-4 | 0 | 5 | rock | Granite, often weakly limonitic 571033/7080213 | 1.186 | 0.4 | 6.5 | 0.3 | 7.8 |
| 116202 | TR10-4 | 5 | 10 | rock | Granite, often weakly limonitic | 0.127 | 0.2 | 5.1 | 0.2 | 1.2 |
| 116203 | TR10-4 | 10 | 15 | rock | Granite, often weakly limonitic | 0.094 | 0.2 | 4.2 | 0.2 | 0.8 |
| 116204 | TR10-4 | 15 | 20 | rock | Granite, often weakly limonitic | 0.161 | 0.2 | 2.4 | 0.1 | 1.4 |
| 116205 | TR10-4 | 20 | 25 | rock | Granite, often weakly limonitic | 0.244 | 0.3 | 4.3 | 0.2 | 1.8 |
| 116206 | TR10-4 | 25 | 28 | rock | Silicified intrusive, grayish quartz with finely disseminated sulphides | 0.289 | 0.5 | 2.6 | 0.1 | 1.7 |
| 116207 | TR10-4 | 28 | 31 | rock | Silicified intrusive, grayish quartz with finely disseminated sulphides | 0.058 | 0.4 | 20.3 | 0.1 | 0.5 |
| 116208 | TR10-4 | 31 | 36 | rock | Granite, often weakly limonitic | 0.407 | 0.3 | 22.6 | 0.2 | 2.9 |
| 116209 | TR10-4 | 36 | 41 | rock | Granite, often weakly limonitic | 0.207 | 0.3 | 83.4 | 0.3 | 1.6 |
| 116210 | TR10-4 | 41 | 46 | rock | Granite, often weakly limonitic | 0.139 | 0.3 | 74.4 | 0.2 | 0.8 |
| 116211 | TR10-4 | 46 | 51 | rock | Granite, often weakly limonitic | 0.087 | 0.2 | 57.4 | 0.3 | 0.8 |
| 116212 | TR10-4 | 51 | 56 | rock | Granite, often weakly limonitic 570989/7080188 | 0.085 | 0.2 | 46.9 | 0.3 | 1 |
| 116213 | TR10-4 | 56 | 61 | rock | Granite, often weakly limonitic | 1.359 | 1.2 | 90.4 | 0.3 | 5.8 |
| 116214 | TR10-4 | 61 | 66 | rock | Granite, often weakly limonitic | 0.681 | 0.7 | 54.5 | 0.3 | 3.6 |
|  |  | 56 | 66 |  | weighted average | 1.02 Au over 10m |  |  |  |  |
| 116215 | TR10-4 | 66 | 71 | rock | Granite, often weakly limonitic | 0.109 | 0.4 | 19.2 | 0.1 | 0.9 |
| 116216 | TR10-4 | 71 | 76 | rock | Granite, often weakly limonitic | 0.199 | 0.4 | 35.4 | 0.2 | 1.2 |
|  |  | 56 | 76 |  | weighted average | 0.587 Au over 20m |  |  |  |  |
| 116217 | TR10-4 | 76 | 81 | rock | Granite, often weakly limonitic | 0.042 | 0.2 | 46.6 | 0.3 | 0.5 |
| 116218 | TR10-4 | 81 | 86 | rock | Granite, often weakly limonitic | 0.236 | 0.2 | 14.2 | 0.2 | 1.2 |
| 116219 | TR10-4 | 86 | 90.5 | rock | Silicified intrusive, grayish quartz with finely disseminated sulphides | 0.208 | 0.3 | 14.8 | 0.2 | 1.5 |
| 116220 | TR10-4 | 90.5 | 95 | rock | Silicified intrusive, grayish quartz with finely disseminated sulphides | 0.063 | 0.3 | 3.6 | <0.1 | 0.4 |
| 116221 | TR10-4 |  |  | rock | Blank | <0.005 | <0.1 | 1.5 | 0.2 | <0.1 |
| 116222 | TR10-4 |  |  | rock | Standard | 1.051 | 0.2 | 1578.1 | 1.5 | 0.1 |
| 116223 | TR10-4 | 95 | 100 | rock | Granite, often weakly limonitic | 0.09 | 0.2 | 12.1 | 0.2 | 0.6 |
| 116224 | TR10-4 | 100 | 105 | rock | Granite, often weakly limonitic 570944/7080172 | 0.054 | 0.2 | 38.6 | 0.2 | 0.3 |
| 116225 | TR10-4 | 105 | 110 | rock | Granite. often weakly limonitic | 0.198 | 0.4 | 12.5 | 0.2 | 1.2 |
| 116226 | TR10-4 | 110 | 115 | rock | Granite, often weakly limonitic | 0.073 | 0.3 | 8.6 | 0.1 | 0.6 |
| 116227 | TR10-4 | 115 | 120 | rock | Granite, often weakly limonitic | 0.26 | 0.4 | 20.8 | 0.1 | 1.6 |
| 116228 | TR10-4 | 120 | 125 | rock | Granite, often weakly limonitic | 0.092 | 0.1 | 261.5 | 0.2 | 0.5 |
| 116229 | TR10-4 | 125 | 130 | rock | Granite, often weakly limonitic | 0.059 | 0.1 | 136 | 0.2 | 0.2 |
| 116230 | TR10-4 | 130 | 135 | rock | Granite, often weakly limonitic | 0.026 | <0.1 | 10.9 | <0.1 | 0.2 |
| 116231 | TR10-4 | 135 | 140 | rock | Granite, often weakly limonitic | 0.007 | <0.1 | 6.3 | <0.1 | <0.1 |
| 116232 | TR10-4 | 140 | 145 | rock | Quartzite 290/flat lying | 0.017 | 0.1 | 10.1 | 0.1 | 0.1 |
| 116233 | TR10-4 | 145 | 150 | rock | Quartzite | 0.041 | 0.1 | 9.5 | 0.1 | <0.1 |
| 116234 | TR10-4 | 150 | 155 | rock | Quartzite 570897/7080153 | 0.019 | <0.1 | 3 | <0.1 | 0.2 |
| 116235 | TR10-4 | 155 | 160 | rock | Quartzite | 0.037 | 0.2 | 4.2 | <0.1 | 0.2 |
| 116236 | TR10-4 | 160 | 165 | rock | Quartzite | 0.21 | 0.1 | 2.9 | <0.1 | 1.2 |
| 116237 | TR10-4 | 165 | 170 | rock | Quartzite | 0.032 | <0.1 | 8.6 | 0.2 | 0.2 |
| 116238 | TR10-4 | 170 | 175 | rock | Granite | 0.033 | 0.1 | 5.3 | 0.1 | 0.3 |
| 116239 | TR10-4 | 175 | 180 | rock | Granite | 0.015 | <0.1 | 5.4 | 0.2 | 0.1 |
| 116240 | TR10-4 | 180 | 185 | rock | Granite | 0.007 | <0.1 | 3 | <0.1 | <0.1 |
| 116241 | TR10-4 | 185 | 190 | rock | Granite | 0.007 | <0.1 | 9.8 | 0.1 | <0.1 |
| 116242 | TR10-4 | 190 | 195 | rock | Granite | 0.03 | 0.1 | 11.5 | 0.1 | 0.2 |
| 116243 | TR10-4 | 195 | 200 | rock | Granite | 0.021 | 0.1 | 10.9 | 0.2 | 0.1 |
| 116244 | TR10-4 | 200 | 205 | rock | Granite 570850/7080136 | 0.032 | 0.1 | 24 | 0.2 | 0.3 |
| 116245 | TR10-4 |  |  | rock | Blank | <0.005 | <0.1 | 1.7 | 0.2 | <0.1 |
| 116246 | TR10-4 |  |  | rock | Standard | 0.989 | 0.2 | 1534.5 | 1.3 | 0.1 |
| 116247 | TR10-4 | 205 | 210 | rock | Granite | 0.203 | 0.2 | 15.5 | 0.2 | 1.3 |
| 116248 | TR10-4 | 210 | 215 | rock | Granite | 1.956 | 0.7 | 23.7 | 0.3 | 8.7 |
| 116249 | TR10-4 | 215 | 220 | rock | Granite | 0.305 | 0.4 | 12.9 | 0.2 | 1.9 |
|  |  |  |  |  | weighted average | 0.821 over 15m |  |  |  |  |
| 116250 | TR10-4 | 220 | 225 | rock | Granite | 0.071 | <0.1 | 5 | 0.1 | 0.6 |
| 116251 | TR10-4 | 225 | 230 | rock | Granite | 0.014 | <0.1 | 4.8 | <0.1 | 0.2 |
| 116252 | TR10-4 | 230 | 235 | rock | Granite | 0.211 | 0.2 | 15.2 | 0.1 | 0.9 |
| 116253 | TR10-4 | 235 | 240 | rock | Granite | 0.393 | 0.3 | 9.9 | 0.2 | 2 |
| 116254 | TR10-4 | 240 | 245 | rock | Granite | 0.041 | <0.1 | 8.7 | 0.1 | 0.3 |
| 116255 | TR10-4 | 245 | 250 | rock | Granite | 0.124 | 0.1 | 17.2 | 0.2 | 0.8 |
| 116256 | TR10-4 | 250 | 255 | rock | Granite 570804/7080122 | 0.61 | 0.4 | 12.3 | 0.1 | 3 |
| 116257 | TR10-4 | 255 | 260 | rock | Granite | 0.13 | 0.3 | 28.7 | 0.2 | 1.1 |
| 116258 | TR10-4 | 260 | 265 | rock | Granite | 0.131 | 0.3 | 23.4 | 0.3 | 0.8 |
| 116259 | TR10-4 | 265 | 270 | rock | Granite | 0.109 | 0.2 | 16.7 | 0.2 | 0.7 |
| 116260 | TR10-4 | 270 | 275 | rock | Granite | 0.098 | 0.2 | 12.7 | 0.2 | 0.6 |
| 116261 | TR10-4 | 275 | 280 | rock | Granite | 0.079 | 0.2 | 15.7 | 0.2 | 0.6 |
| 116262 | TR10-4 | 280 | 285 | rock | Granite | 4.053 | 1.4 | 120.4 | 0.4 | 22.2 |
| 116263 | TR10-4 | 285 | 290 | rock | Granite | 0.135 | 0.3 | 53.2 | 0.3 | 0.8 |
|  |  |  |  |  | weighted average | 2.094 over 10m |  |  |  |  |
| 116264 | TR10-4 | 290 | 295 | rock | Granite | 0.08 | 0.2 | 17.5 | 0.2 | 0.7 |
| 116265 | TR10-4 | 295 | 300 | rock | Granite | 0.06 | 0.2 | 33.7 | 0.2 | 0.5 |
| 116266 | TR10-4 | 300 | 305 | rock | Granite 570750/7080110 | 0.085 | 0.2 | 146 | 0.3 | 0.8 |
| 116267 | TR10-4 | 27m |  | rock | Grab Bert J. Siliceous granite, grey quartz with finely disseminated sulphides. 571011/7080200 | 0.055 | 0.7 | 5.1 | 0.2 | 0.5 |
| 116268 | TR10-4 | 66m |  | rock | Grab Bert J. Granite with a few weathered out pyrites | 0.086 | 0.6 | 13.3 | 0.2 | 0.8 |
| 116269 | TR10-4 | 90.5m |  | rock | Grab Bert J. Siliceous granite, grey quartz with finely disseminated sulphides. 570952/7080171 | 0.089 | 0.5 | 3.2 | 0.1 | 0.7 |
| 116270 | TR10-4 | 45.7m |  | rock | Grab, Ted A. Granite, quartz rich with majority of feldspars turned to clay 570996/7080193 | 0.048 | 0.3 | 78.7 | 0.2 | 0.4 |
| 116271 | TR10-4 | 59m |  | rock | Grab. Ted A. Quartz vein, located on both sides of trench, 5-10 cms wide, 145/90(?), rock badly fractured/broken up 570982/7080186 | 12.46 | 4.7 | 32.1 | <0.1 | 25 |
| 116272 | TR10-5 | 0 | 5 | rock | Granite crumble 570375/7080214 | 0.024 | 0.2 | 62.6 | 0.2 | 0.2 |
| 116273 | TR10-5 | 5 | 10 | rock | Granite crumble | 0.034 | 0.2 | 64.5 | 0.2 | 0.2 |
| 116274 | TR10-5 | 10 | 15 | rock | Granite crumble | 0.028 | 0.1 | 28.7 | 0.2 | 0.2 |
| 116275 | TR10-5 | 15 | 19 | rock | Granite crumble | 0.025 | 0.3 | 16.2 | 0.2 | 0.2 |
| 116276 | TR10-5 | 19 | 23 | rock | Granite crumble | 0.019 | 0.3 | 12.9 | 0.2 | 0.2 |
| 116277 | TR10-5 | 23 | 28 | rock | Quartzite (overlying granite) | 0.008 | 0.4 | 23.9 | 0.3 | 0.2 |
| 116278 | TR10-5 | 28 | 33 | rock | Quartzite (overlying granite) | 0.016 | 0.3 | 20.2 | 0.4 | 0.2 |
| 116279 | TR10-5 | 33 | 38 | rock | Quartzite | 0.01 | 0.4 | 16.6 | 0.3 | 0.2 |
| 116280 | TR10-5 | 38 | 43 | rock | Quartzite | 0.008 | 0.7 | 13.9 | 0.3 | 0.1 |
| 116281 | TR10-5 | 43 | 48 | rock | Quartzite | 0.01 | 0.6 | 14.8 | 0.2 | 0.1 |
| 116282 | TR10-5 | 48 | 53 | rock | Quartzite 570327/7080202 | 0.024 | 0.3 | 10.4 | 0.2 | 0.2 |
| 116283 | TR10-5 | 53 | 58 | rock | Quartzite | 0.008 | 0.7 | 13 | 0.3 | 0.1 |
| 116284 | TR10-5 | 58 | 63 | rock | Quartzite | 0.044 | 0.6 | 10 | 0.2 | 0.4 |
| 116285 | TR10-5 | 63 | 68 | rock | Quartzite | 0.01 | 0.4 | 6.9 | 0.2 | 0.2 |
| 116286 | TR10-5 | 68 | 73 | rock | Quartzite | 0.012 | 0.2 | 4.8 | 0.2 | 0.2 |
| 116287 | TR10-5 | 78m |  | rock | Quartzite, start channel sampling | <0.005 | 0.3 | 4.1 | 0.2 | 0.1 |
| 116288 | TR10-5 | 83m |  | rock | Quartzite | 0.012 | 0.3 | 4.7 | 0.2 | 0.1 |
| 116289 | TR10-5 | 88m |  | rock | Quartzite | 0.013 | 0.4 | 8.6 | 0.3 | 0.2 |
| 116290 | TR10-5 | 93m |  | rock | Quartzite | <0.005 | 0.4 | 8.8 | 0.3 | 0.1 |
| 116291 | TR10-5 | 98m |  | rock | Quartzite 570278/7080191 | 0.012 | 0.4 | 8.8 | 0.2 | 0.2 |
| 116292 | TR10-5 | 103m |  | rock | Quartzite | <0.005 | 0.4 | 3.5 | 0.2 | <0.1 |
| 116293 | TR10-5 | 108m |  | rock | Quartzite | 0.012 | 0.7 | 3.6 | 0.2 | 0.2 |
| 116294 | TR10-5 | 113m |  | rock | Quartzite | 0.018 | 0.6 | 4.2 | 0.2 | 0.2 |
| 116295 | TR10-5 | 118m |  | rock | Quartzite | 0.089 | 0.5 | 7.5 | 0.3 | 0.1 |
| 116296 | TR10-5 | 123m |  | rock | Quartzite | 0.034 | 0.4 | 3.8 | 0.2 | 0.1 |
| 116297 | TR10-5 | 128m |  | rock | Quartzite | 0.007 | 0.6 | 1.8 | 0.1 | 0.1 |
| 116298 | TR10-5 | 133m |  | rock | Quartzite | 0.013 | 0.4 | 2.6 | 0.2 | 0.2 |
| 116299 | TR10-5 | 136 | 141 | rock | Graphitic quartzite | 0.007 | 0.2 | 1.9 | 0.2 | 0.1 |
| 116300 | TR10-5 | 141 | 146.5 | rock | Graphitic quartzite | <0.005 | 0.3 | 0.9 | 0.1 | <0.1 |
| 116301 | TR10-5 | 146.5 | 150.3 | rock | Granite | 0.028 | 0.3 | 2.1 | 0.2 | 0.2 |
| 116302 | TR10-5 | 150.3 | 155 | rock | Quartzite, somewhat sheared 570228/7080181 | 0.007 | 0.3 | 2.1 | 0.1 | 0.1 |
| 116303 | TR10-5 | 160m |  | rock | Quartzite, often micaceous, occasionally schistose | <0.005 | 0.2 | 2.1 | 0.3 | <0.1 |
| 116304 | TR10-5 | 165m |  | rock | Quartzite, often micaceous, occasionally schistose | 0.01 | 0.3 | 4.1 | 0.3 | 0.1 |
| 116305 | TR10-5 | 170 | 173.5 | rock | Quartzite, often micaceous, occasionally schistose | 0.012 | 0.3 | 2.9 | 0.2 | <0.1 |
| 116306 | TR10-5 | 173.5 | 178.5 | rock | Granite | 0.006 | 0.3 | 3 | 0.2 | 0.1 |
| 116307 | TR10-5 | 178.5 | 183.5 | rock | Granite | 0.009 | 0.3 | 3.2 | 0.2 | 0.2 |
| 116308 | TR10-5 | 183.5 | 188.5 | rock | Granite | 0.012 | 0.4 | 2.9 | 0.2 | 0.1 |
| 116309 | TR10-5 | 188.5 | 193.5 | rock | Granite | 0.007 | 0.5 | 2.7 | 0.1 | 0.2 |
| 116310 | TR10-5 | 193.5 | 198.5 | rock | Granite | 0.009 | 0.5 | 2.4 | 0.2 | 0.2 |
| 116311 | TR10-5 | 198.5 | 204 | rock | Granite | 0.018 | 0.4 | 4.9 | 0.2 | 0.2 |
| 116312 | TR10-5 | 204 | 209 | rock | Quartzite | <0.005 | 0.8 | 1.7 | 0.1 | 0.1 |
| 116313 | TR10-5 | 214m |  | rock | Quartzite, start 1.5m channel sampling | 0.005 | 0.6 | 1.8 | 0.2 | <0.1 |
| 116314 | TR10-5 | 219m |  | rock | Quartzite | <0.005 | 0.9 | 2.3 | 0.2 | <0.1 |
| 116315 | TR10-5 | 224m |  | rock | Quartzite | <0.005 | 0.6 | 3.9 | 0.2 | <0.1 |
| 116316 | TR10-5 | 229m |  | rock | Quartzite | <0.005 | 0.8 | 1.4 | 0.1 | <0.1 |
| 116317 | TR10-5 | 234m |  | rock | Quartzite | <0.005 | 0.8 | 0.8 | <0.1 | <0.1 |
| 116318 | TR10-5 | 239m |  | rock | Quartzite | <0.005 | 0.9 | 0.7 | 0.1 | 0.1 |
| 116319 | TR10-5 | 244m |  | rock | Quartzite | <0.005 | 0.7 | 1.3 | 0.2 | <0.1 |
| 116320 | TR10-5 | 249m |  | rock | Quartzite | <0.005 | 1.7 | 0.8 | <0.1 | <0.1 |
| 116321 | TR10-5 | 254 | 259 | rock | Quartzite | <0.005 | 2.0 | 1.9 | 0.1 | 0.1 |
| 116322 | TR10-5 | 259 | 264 | rock | Granite | 0.01 | 0.3 | 3.5 | 0.2 | 0.1 |
| 116323 | TR10-5 | 264 | 269 | rock | Granite | 0.006 | 0.5 | 1.7 | 0.1 | <0.1 |
| 116324 | TR10-5 | 269 | 274 | rock | Granite | 0.012 | 0.2 | 1.9 | 0.2 | <0.1 |
| 116325 | TR10-5 | 274 | 279 | rock | Granite | 0.008 | 0.2 | 1.9 | 0.1 | 0.1 |
| 116326 | TR10-5 | 279 | 284 | rock | Granite | 0.018 | 0.3 | 2.8 | 0.1 | 0.2 |
| 116327 | TR10-5 | 284 | 289 | rock | Granite | 0.02 | 0.2 | 5.6 | 0.2 | 0.2 |
| 116328 | TR10-5 | 289 | 294 | rock | Granite | 0.058 | 0.3 | 55.9 | 0.2 | 0.3 |
| 116329 | TR10-5 | 294 | 299 | rock | Granite | 0.043 | 0.3 | 4.2 | 0.2 | 0.3 |
| 116330 | TR10-5 | 299 | 304 | rock | Granite 570081/7080141 | 0.027 | 0.1 | 59.8 | 0.2 | 0.2 |
| 116331 | TR10-5 | 304 | 309 | rock | Granite | 0.03 | 0.2 | 15.2 | 0.2 | 0.2 |
| 116332 | TR10-5 | 309 | 314 | rock | Granite | 0.034 | 0.2 | 18.8 | 0.2 | 0.2 |
| 116333 | TR10-5 | 314 | 319 | rock | Granite | 0.033 | 0.3 | 3.7 | 0.2 | 0.2 |
| 116334 | TR10-5 | 319 | 324 | rock | Granite | 0.043 | 0.3 | 18.6 | 0.2 | 0.2 |
| 116335 | TR10-5 | 324 | 329 | rock | Granite | 0.04 | 0.2 | 4.5 | 0.2 | 0.3 |
| 116336 | TR10-5 | 329 | 334 | rock | Granite | 0.029 | 0.2 | 6.4 | 0.2 | 0.2 |
| 116337 | TR10-5 | 334 | 339 | rock | Granite | 0.033 | 0.2 | 5.2 | 0.2 | 0.2 |
| 116338 | TR10-5 | 339 | 344 | rock | Granite | 0.014 | 0.2 | 3.9 | 0.2 | 0.1 |
| 116339 | TR10-5 | 344 | 347 | rock | Quartzite | <0.005 | 0.2 | 6 | 0.2 | <0.1 |
| 116340 | TR10-5 | 347 | 350 | rock | Quartzite | <0.005 | 0.2 | 7.5 | 0.2 | <0.1 |
| 116341 | TR10-5 | 350 | 355 | rock | Granite 570028/7080134 | 0.027 | 0.1 | 3.2 | <0.1 | 0.2 |
| 116342 | TR10-5 | 355 | 360 | rock | Granite | 0.018 | <0.1 | 3.3 | 0.1 | 0.1 |
| 116343 | TR10-5 | 360 | 365 | rock | Granite | 0.063 | 0.3 | 2.2 | <0.1 | 0.4 |
| 116344 | TR10-5 | 365 | 370 | rock | Granite | 0.054 | 0.2 | 6.9 | 0.1 | 0.4 |
| 116345 | TR10-5 | 370 | 375 | rock | Granite | 0.028 | 0.2 | 16.7 | 0.2 | 0.2 |
| 116346 | TR10-5 | 375 | 380 | rock | Granite 569999/7080129 | 0.04 | 0.2 | 4.4 | 0.1 | 0.3 |
| 116347 | TR10-5 | 382m |  | rock | Grab 569998/7080129 Just before the very end of the trench there is a narrow short finger of quartzite so sampled seperately | <0.005 | <0.1 | 1.5 | <0.1 | <0.1 |

Appendix D: Petrographic Report

Appendix E: Certificates of Analysis

Appendix F: Compact Disc

Report text, geochemical and drill databases, geophysical files, drafting and plot files, photographs

Appendix G: Geologist’s Certificate

GEOLOGIST'S CERTIFICATE

David A. Swanton

402-1595 East 6th Avenue

Vancouver, British Columbia

daves@equityexploration.com

I, David Swanton, do herby certify that:

- I am a Project Geologist with Equity Exploration Consultants Ltd, with offices at Suite 200-900 West Hastings St., Vancouver, British Columbia.

- I am a graduate of the University of British Columbia (2004) with a Bachelor of Science degree and of Acadia University (2010) with a Master of Science degree in Geology.

- I have been involved in the mineral exploration industry since 2006.

- I have prepared this report based on data provided to me by Aldrin Resource Corporation.

Dated at Vancouver, British Columbia, this___20__day of ___January__________, 2011.

___________________________

David Swanton, M.Sc.