May, 2010

The following summarizes the sampling and QA/QC protocols to be followed for the duration of the 2010 diamond drill program on the Keystone property.

Drill Hole Numbering Scheme:

- Drill hole numbering is to incorporate the 3-letter project code, year, and sequential number, for example:

KEY10-01

Quality Control:

Quality Control Procedures for Rock, Trench and Drill Core Samples:

| 01 | Field Duplicate 1 |
| --- | --- |
| 02 | Field Duplicate 2 |
| 03 | Routine Sample |
| 04 | Routine Sample |
| 05 | Routine Sample |
| 06 | Routine Sample |
| 07 | Routine Sample |
| 08 | Routine Sample |
| 09 | Preparation Duplicate |
| 10 | Standard/Blank |
| 11 | Routine Sample |
| 12 | Routine Sample |
| 13 | Routine Sample |
| 14 | Routine Sample |
| 15 | Routine Sample |
| 16 | Routine Sample |
| 17 | Routine Sample |
| 18 | Routine Sample |
| 19 | Routine Sample |
| 20 | Routine Sample |

- Standards are inserted 1/20 (sample numbers are pre-determined as 10,30, 50, 70, 90; skip the number in the sample sequence for later insertion of the standard reference material); blanks may be used in lieu of a standard, at a rate of 1 in 3 standards, and/or inserted after any highly mineralized sample.  See the table at the end of the document for a list of standards available.

- Preparation duplicates are inserted 1/20 (sample numbers are pre-determined as 09, 29, 49, 69, 89; submit an empty bag with a ticket number, alerting the preparation lab to prepare the duplicate from the previous sample).

- Field duplicates are inserted at a rate of 1/20 (sample numbers are pre-determined as 01/02, 21/22, 41/42, etc.) and in the case of drilling will be of quartered core. Additional field duplicates will be selected after the analyses are returned in order to select high grade intersections for duplicate samples.

Analytical Procedure:

- Samples are to be prepared in the standard manner by coarse crushing and pulverization (R-150)

- All samples will be analysed by:

    - ME-ICP-41 (4-acid digestion with an ICP finish)

    - Au-AA24 (50g low level Au fire assay)

- Overlimits:

    - AA62 for base metals >10,000 ppm and Ag >100 ppm

    - Ag-GRA22 (gravimetric fire assay) for Ag >1000 ppm

    - Au GRA22 (gravimetric fire assay)for Au >1 ppm

    - Au-SCR22 (metallic screen) for Au >10ppm

Analytical Batch Numbering Scheme:

- Is the following scheme

B2010KEY001

YEAR 3-LETTER PROJECT CODE SHIPMENT NUMBER