DDD-TR-11-01

Lithological descriptions

- (limonite) SCHIST

  - Orange weathering, flakey, friable medium green-grey schist with quartz and muscovite (on partings), well foliated.  Locally weak hematite or chlorite alteration.  Limonite (orange) coating common on fracture surfaces and partings, becoming pervasive near contact.  Sharp fault contact (light green-ish grey gouge) ~260/75N

@1.5m: Foliation 210/15N and Foliation 245/65N

- (sericite) SCHIST

  - Light green-grey broken rock with bands of fault gouge and breccia.  Rock is a very fine grained to fine grained, pale green-white.  Frequent patches of saccharoidal quartz or limonite or muscovite.  Pervasive sericite and patchy sheared alteration has made identifying protolith difficult.

@0.8m: A narrow band of gouge ~230/25N

@2.4m: Gradational contact

- (hydrothermal) BRECCIA

  - Orange rubbly, monolithic breccia with dark grey quartz matrix.  Angular, occasional jigsaw fit, pale green-grey clasts up to 10cm.  Matrix becomes limonitic near contacts.  Clasts all pervasively sericite altered.  Drusy quartz common.  Dark grey colour of massive quartz matrix may be due to very fine grained disseminated sulphides.

@3.4m: Irregular contact with sericite schist.  Approximately vertical.

- (altered rock) SCHIST(?)

  - Irregularly blocky, light brown, relatively competent, but still quite fractured.  Fissle.  Very fine grained, tan with stockwork of drusey quartz and/or hematite veinlets.  Pervasive clay(?) alteration.  Local bands and patches of disseminated very fine grained black mineral with occasional metallic silver fleck (arsenopyrite?).

@7.5m: Foliation 045/35SE.  Limonite common as fracture coating, patches and bands

@8.6-8.8m: Broken gouge zone ~250/65N

- (hematite and/or chlorite + sericite) SCHIST

  - Similar to (4) except manganese oxide staining on fractures gives this interval a brown and black colour.  Instead of dull tan-clay alteration, the rock is usually dark brown-ish, red and grey or pale green (sub-ordinate).  This is due to pervasive hematite or chlorite and sericite.  Fewer quartz and/or hematite veinlets.  Well foliated (especially where hematite is present).

@12m: Foliation 325/12NE.  Band and patches of limonite less common

@17m: Sharp fault gouge (narrow ~1cm) contact

- (sheared) SCHIST

  - Broken, orange schist with zone of orange fault gouge.  Hematite, limonite cover fracture surfaces.

@18.4m: A 20cm zone of fault gouge ~225/25

- (manganese oxide and hematite) SCHIST (and lesser tuff)

  - Broken, black and orange stained schist.  Blocky sections probably tuff.  Pervasive hematite usually colours the rock a purple-brown colour.

@21.6m: 345/30NE (tuff bed?)

- (carbonate) SCHIST and TUFF

  - Broken, orange>black weathering schist and tuff layers.  Schist usually dull green-ish brown and red-ish brown bands.  Moderate carbonate.  Probably moderately clay (weathering a possible factor as well).

@24m: 310/30N (tuff bedding)

@28.4-32.9m: Very broken mix of clay and bedrock and up to 1% very fine grained, disseminated black mineral locally present.

- (carbonate) TUFF(?)

  - Broken, manganese oxide stained tuff with patchy orange carbonate alteration.  Tuff is grey, hard and aphanitic when unaltered.  Locally strongly hematite altered.

@32.9-33.7m: ~25cm zone of pale grey gouge 305/35NE.

Rock highly fractured and strongly altered.  Rubble/blockiness suggests protolith is a tuff.

@36.4m: 305/30NE

- FAULT and TUFF

  - Orange gouge at the start and end of this interval with very broken tuff between

@41.7-42.3m: Orange gouge 240/20N

@45m: Gouge on west rib difficult to find on east rib.

- (manganese oxide stained, broken) TUFF and SCHIST(?)

  - Very broken, blocky (irregularly) black stained tuff and schist(?).  The rock is usually a dull fine grained, granular green-brown.  Clay alteration.  Variably foliated.  Local hematite.

*After this interval, frozen ground prevented us from reaching bedrock

- COLLUVIUM

  - Mainly sub-angular schist and tuff clasts mixed with soil.

@54.7-55.7m: Colluvium dominated by irregular clasts of orange and yellow weathered grey quartz vein.

Quartz vein clasts usually sachrodal and drusy quartz.  Fine grained disseminated pyrite (<1%) and very fine grained disseminated black and silver mineral (arsenopyrite? ~1%) were ovserved in quartz vein clast from trench castings.

*Quartz vein clasts probably nearly in situ.

**Occur at an east-west trending recessive slope break.

@56-69m: Clasts mainly muscovite schist but limonite stained white quartz vein and silicified schist occur frequently.  Rare chlorite schist also present.

@70-74.5m: Colluvium dominated by variously silicified and limonite altered muscovite schist (usually unaltered or weakly altered).  Quartz veins fragments with limonite staining and occasional drusy quartz occur frequently.

@71-74m: White quartz vein clast common in colluvium.  Orange limonite staining frequently on fractures.  Locally cut by drusy quartz veinlets.

***Patchy pervasive and/or stockwork veinlet hematite occurs locally.

- (limonite) SCHIST

  - Similar to (1)

@74.5-75.5m: Blue, tan and orange fault gouge 245/45N (approximate)

@76m: Foliation 280/40N in schist

- COLLUVIUM

  - @70-107m: Colluvium dominated by muscovite schist and tuff, commonly with limonite fracture staining.

- (green) SCHIST

  - Irregular-platy, black (manganese oxide) stained schist.  Fresh surface usually moderately foliated aphanitic grey-ish green.

- (green) DIORITE(?)

  - Irregularly blocky, black stained diorite.  Fresh surface dull, medium green with 10-15% subhedral to anhedral yellow-ish white feldspar phenocrysts.  5-10% anhedral hornblende (locally larger lathes up to 5mm). ~1% biotite and rare quartz.  Matrix is dull green (Feldspathic).  Hornblend often chloritorized.  Green colouring likely reflects pervasive chlorite alteration.

@122-124.5m: Rock is very broken, especially near the contact.

@129.5-130m: Rock is silicified (frequent quartz veinlets).

- BIOTITE-MUSCOVITE SCHIST

  - Platy, well foliated, black biotite-muscovite schist.

@109.5m: Foliation 275/15N

@136.5m: Gradational contac with (18)

- QUARTZ-BIOTITE QUARTZITE

  - Blocky, medium grey quartzite composed mainly of fine grained grey quartz and varying percentage of fine grained biotite in bands defining a weak schistosity.

@175.5m: Foliation 340/30E

- COLLUVIUM

  - Dominated by quartz-biotite quartzite clasts.

@194-195.5m: Clasts often strongly silicified.  Rock flooded and/or replaced by massive to granular white quartz.

- (green) MAFIC

  - Blocky green mafic intrusive.  Various phases, first outcrop at 196.5m is a fine grained dark green rock with approximately equal proportions of green-ish feldspar (anhedral) and an anhedral black mafic mineral with one strong cleavage.  Locally features up to 2% clots (<5mm) of fine grained pyrite.  Also occurs as an aphanitic dull green phase.

- COLLUVIUM

  - Dominated by clasts of (20).

- QUARTZITE

  - Broken, poorly exposed quartz-biotite quartzite.

- COLLUVIUM

  - Dominated by clasts of (20).

- (limonite-muscovite) SCHIST

  - Very broken and poorly exposed, dull green, limonite-muscovite schist.  Dull orange-brown patches of limonite common.

- COLLUVIUM

  - Dominated by quartzite > biotite schist > green mafic.  White quartz vein also present

@226.5-230m: orange moderate-strong iron carbonate clasts become common.

- (green) MAFIC

  - Very broken, poorly exposed green mafic.  Limonite and manganese oxide stain noted on fracture surfaces.  Locally silicified (veinlets).

- COLLUVIUM

  - Dominated by biotite-muscovite schist and biotite quartzite.

@231.9-235m: Iron carbonate clasts common.

- (silicified and iron carbonate) ANDESITE(?)

  - Rubbly, dull olive andesite cut by frequent, irregular quartz veinlets and iron carbonate patches.  Andesitic composition(?)

- (clast poor) OVERBURDEN

  - Possible fault gouge(?) and/or decomposed shear zone.  Variously coloured and textured clays.

- (black) MAFIC

  - Rubble, probably ultramafic, composed of interlocking crystals of a platy, green-ish black mineral with one strong cleavage.  Broken thin edges of this mineral are a translucent green.

- (altered) SCHIST(?)

  - Black and orange, very broken, iron carbonate schist(?).  Well foliated.

@253.4-254.2m: Rock is composed of decomposed chips.

- (altered) INTRUSIVE(?)

  - Aphanitic medium green rock with an irregular stockwork of quartz veinlets and patchy iron carbonate.  Very broken.

- ANDESITE

  - Irregularly blocky, dull, olive, Feldspathic matrix.  ~2% 1mm quartz eyes and ~5% anhedral, dull black mafic mineral (<2mm).  intrusive or flow of andesitic(?) compositioni.

- QUARTZ-MUSCOVITE SCHIST

  - Platy, green quartz-muscovite schist

@277.6m Foliation 290/35N

- COLLUVIUM

  - Dominated by quartz-muscovite schist

- QUARTZ-MUSCOVITE SCHIST

  - Sam	e as (34)

- (hematite, manganese oxide, iron carbonate) QUARTZITE(?)

  - Altered quartzite(?) with manganese oxide, hematite or iron carbonate on various fracture surfaces.  Very broken.

DDD-TR-11-02

Lithological descriptions

- QUARTZ-MUSCOVITE SCHIST

  - Platy, fissile, shiny light grey to orange weathering.  Muscovite + quartz in a lesser dull green, aphanitic matrix.  Difficult to get strike and dip due to slumping.  Increasing clay +/- hematite towards the contact with (2).

@4m: Narrow discontinuous zone of gouge (~2cm wide, oriented 225/25NW) marks the contact between alteration schist and (2)

- (clay) TUFF(?)

  - Angular broken and rubbly, aphanitic, pale tan to dull earthy light green-ish brown.  Matrix of clay and fine grained quartz with a sheen of muscovite on some partings.  Increasingly siliceous towards (3) with irregular veinlets of clear or drusy quartz common.

@7.1m: Foliation 285/40N

- QUARTZ VEIN

  - Very broken, white saccharoidal and drusy quartz vein.  Light grey, fine grained.

@8.5m: Shear zone 280/50N

- (siliceous and sericite) SCHIST and TUFF

  - Broekn, lithg grey, aphanitic siliceous sericite schist and tuff

@9.3m: Foliation (bedding) 325/35NE.  Strongly silicified tuff (+ moderate sericite) overlain by moderate-strong silica and strong sericite schist.  Band of schist overlies tuff.

@9.9m: Gouge – 2cm band of orange gouge 250/60N

- (sericite and silica) TUFF

  - Broken, light grey rock composed mainly of very fine grained, granular, grey quartz.  Probably silica and sericite altered tuff.  Where rock is more broken, there is less quartz and more sericite.

@11.2m: Fracture set ~160/80 and 220/80

@13.8-14.8m: Resistant hump of competent rock.  May be hydrothermal breccia.  Locally (uncommon) disseminated very fine grained pyrite (~2%) and very fine grained black mineral (~1%)

@14.8m: Contact between foliated rock and unfoliated (siliceous) rock is ~245/25N (siliceous tuff to schist contact?)

- (clay-muscovite) SCHIST

  - Very broken, orange weathering, dull tan, very fine grained clay-muscovite schist

@16m: Foliation 310/10 NE

@17.5m: (west rib), irregular patch of quartz vein material.  Pale grey-ish white, massive quartz with orange grey weak limonite stain on fractures.  Drusy, orange stained vugs ~3% of the rock

@18m: Poorly exposed transition to blocky tuff – probably shallow dip

- (brown stained) TUFF

  - Irregularly blocky (angular) dark brown stained tuff.  Clear (white), fine grained granular, weakly foliated quartz composes this rock.

- MUSCOVITE-SCHIST and TUFF

  - Decomposed and broken rock characterizes this interval of interbedded schist and tuff.

@53.1m: Saccharoidal quartz with sericite tuff in trench castings.

@55.1-56.1m: Orange and yellow strained moderate sericite and silica altered tuff.

@59.1-59.6m: Muscovite schist is moderately hematite altered.

- BIOTITE-MUSCOVITE SCHIST

  - Variable hematite altered biotite-muscovite schist or muscovite schist

@60.1m: Foliation 340/30E

@61.1-63.5m: Muscovite schist with increasing hematite and limonite

- COLLUVIUM

  - Dominant clast types are tuff and muscovite schist.  Usually orange stained.

@74.6-79.1m: Water filled depression that is probably the source of the frequent quartz vein clasts in the trench casting.  Generally white or clear granular quartz, but some drusy veinlets and patches and possible saccharoidal quartz present.

@91.1-94.6m: Water filled pit.

@98-103m: Water filled pit.  Small piece of yellow and orange stained possible epithermal quartz in castings.  Light grey, saccharoidal(?).

@108.3-114.5m: Water filled pit

@115.1m: Small piece of light green (chlorite) rock on trench floor.

- (green, muscovite?) QUARTZ SCHIST

  - Poorly exposed, rubbly section of olive green (clay?)  Muscovite-quartz schist.  Locally clear to white quartz vein (up to 4cm).

- COLLUVIUM

  - Tuff and various intrusive rock dominate

@129.4-137.6m: Trench bottom cuts probably decomposed in situ (11).  Various clast types

@134.1m: Dark grey, very fine grained matrix basalt with ~3% subhedral white feldspar (<3mm) and <1% biotite(<1mm).  Minor light green epidote(?) in irregular veinlet.

- (green) QUARTZITE

  - Blocky, pale green-ish white, fine grained, granular quartz with muscovite-rich (+ minor biotite) partings.  Weakly foliated

- COLLUVIUM

  - Dominant clast is weakly foliated, pale green quartzite

- QUARTZITE + SCHIST

  - @159.1-162.1m: Pale green-white quartzite

@162.1-165.1m: Black (manganese oxide?) stained muscovite schist

@162.6m: Foliation 265/20N

@165.1-169.1m: Biotite-quartz schist

- COLLUVIUM

  - Dominantly biotite-quartz-schist and quartzite

- QUARTZITE and SCHIST

  - @175.1-176.6m: Weakly foliated, pale green, granular quartzite

@176.6-183.1m: Biotite-quartz schist

@183.1-187.1m: Green muscovite schist

- COLLUVIUM

  - @187.1-196.1m: Green muscovite schist or quartzite

@196.1-240m: Dominantly biotite quartzite clasts

- (black) MAFIC

  - Black medium grained mafic composed largely of black mafic mineral.  Locally up to 3% fine grained pyrite (clots <5mm).  Frequently iron carbonate.

@242.8-249.8m: Poorly exposed, decomposed bedrock.

@253.3-255.8m: Poorly exposed, very broken, strong iron carbonate altered black mafic.

- COLLUVIUM

  - Clasts dominantly black mafic, then increasingly (to the south) an aphanitic, hard, white felsic rock

- (black) MAFIC and ANDESITE (iron carbonate and silica)

  - @263-264m: Black mafic

@264-266.5m: Very fine grained, quartz porphyry (dull olive)

Both intervals variably iron carbonate and silica altered.

- COLLUVIUM

  - Light grey, hard, aphanitic, variably iron carbonate and local clots of pyrite (fine grained, <2%).  Increasing andesite (dull olive, fine grained) clasts moving southwards.

- (green muscovite) SCHIST

  - Poorly exposed and broken green muscovite schist.  Variably iron carbonate and silica altered, especially for last 1.5m.

- OVERBURDEN

DDD-TR-11-03

Lithological descriptions

- GREEN MUSCOVITE SCHIST

  - Well foliated platy green muscovite schist, where relatively unaltered

@4m: Foliation 033/30SE

- (altered) SCHIST

  - Generally orange (limonite) or hematite stained schist

@6.0-9.1m: Broken, orange stained, silicified and seritized, manganese oxide, limonite schist and tuff

@12-17m: Very broken, orange stained, seritized schist (with minor tuff)

@15.8m: Foliation 280/75N

- (sheared) altered SCHIST

  - Similar to (2) but very broken

@18.8-19.4m: Broken white to clear quartz vein with drusy vugs and orange stained fractures

- (silicified, seritized) BRECCA/TUFF/QUARTZ VEIN

  - Rubbly, unfoliated pale grey very fine grained saccharoidal quartz +sericite + irregular drusy quartz, often appears indistinctly brecciated.

@23.5m: Irregular contact between broken, foliated rock and rubbly, unfoliated rock 240/50N

@26.6m: Fracture (dominant) 200/50W

@28.6m: Poorly exposed irregular(?) contact between breccia and tuff

- BANDED TUFF

  - Augularly blocky tuff with dark brown and orange staining on fractures

@32.5-35.1m: Tuff, more broken and limonite stained

- MUSCOVITE-QUARTZ SCHIST

  - Orange stained muscovite quartz schist.  Well foliated

@41.1m: Foliation 290/45N

@43.6m: Contact 315/25N

- TUFF (similar to (5))

  - Orange and brown stained, less strong than (5)

@63.2m: Sheared contact ~255/60 – difficult to measure as it is defined by several fracturing planes

- Sericite TUFF

  - Very broken, orange stained, pale green and grey, very fine grained tuff.  Moderate-strongly seritized

@65.6m: Moderately north dipping contact(?)

- (orange stained) TUFF and SCHIST

  - Broken, interbedded tuff and schist.  Orange stained, variable silicification (locally, several crackled quartz veins, patchy pervasive silicification) and patchy hematite, especially in schist.  Schist is green muscovite schist

@69.5m: Broken/gouge zone, ~280/45N

- (sericite) TUFF

  - Moderate-strong sericite tuff.  Orange stained, broken, increasing silicification to south end of interval.  Some drusy quartz patches.

- COLLUVIUM

  - @77.3-83.2m: Overburden dominated by silicified and seritized tuff, quartz vein (saccharoidal and drusy) and breccia with quartz vein matrix.  Often an apple green tint

@77.3-79.7m: Very broken orange stained muscovite schist.  Essentially in-situ

@79.7-80.2m: Essentially in-situ altered turf and quartz vein

@83.2-89.7m: Overburden dominated by irregularly blocky, orange and brown stained tuff.  Variously altered clasts common (limonite, sericite, hematite)

- TUFF and hematite SCHIST

  - Broken and poorly exposed weak sericite +/- silica tuff and strong hematite and manganese oxide stained schist

@93.7m: 290/30N

- COLLUVIUM

  - Dominated by brown stringers, irregularly blocky, medium grey, very fine grained quartzite.  Lesser muscovite schist.

@132.2m: Biotite schist clasts begin to occur.  Quartzite still dominant

@133.2-138.2m: Occasional seritized clast

@138.2-145.2m: Some hematite and silica clasts

@152.8-172.8m: Dominantly schist

DDD-TR-11-04

Lithological descriptions

- QUARTZITE

  - Blocky, essentially in-situ, quartzite clasts form the overburden here.  No clearly coherent bedrock reached.  Commonly dark brown stained.  Locally moderately seritized.

@4.5-5.6m: Very broken, poorly exposed orange stained quartzite

@6.5-8.5m: Poorly exposed manganese oxide and hematite or limonite quartzite.  Silicified and seritized by end of interval

- QUARTZ VEIN

  - Large, probably in-situ, blocks with sections of poorly exposed rock or clay between

@8.7-10.6m: Irregularly broken grey saccharoidal or white granular quartz vein with drusy cavities.  Also intensely silicified and seritized rock (quartzite?)

@10.6-11.4m: Solid, resistance block of white granular quartz vein.

@11.4-12.8m: Overburden - quartz vein not exposed

@12.8-13.6m: Resistant block of somewhat granular white quartz with orange stained fractures

@13.6-13.8m: Glassy grey quartz with very fine grained speckling of black mineral (~1%).  Within black speckling is the occasional silvery metallic fleck (arsenopyrite?)

- (Silicified and seritized) QUARTZITE

  - Large (~1m), probably in-situ, block of intensely silicified and seritized quartzite(?).  Features fine grained “blobs” of quartzite

- OVERBURDEN

  - Moderately silicified and seritized quartzite and quartz vein clasts dominate

- FAULT BRECCIA

  - Broken zone of fault breccia with some larger clasts of quartz vein or limonite-rich material

- QUARTZITE (orange stained)

  - Broken, orange stained quartzite

@19.7m: Crude fracture fabric ~295/55N

@19.8-20.2m: Medium grey quartz vein (very fine grained to aphanitic quartz) with <1% fine grained pyrite and possible trace arsenopyrite.  Thick coating of limonite on fractures.  Oriented ~290/55N

- OVERBURDEN

  - Dominant clasts type is blocky manganese oxide stained quartzite and lesser muscovite schist.  White quartz with orange stained fractures common.

- Altered SCHIST and QUARTZ VEIN BRECCIA

  - @40-41m: (Poorly exposed in water filled section of trench.)  Quartz vein breccia with limonite matrix orange stained fractures and drusy quartz vugs in white quartz vein.

@34-40m: Moderately silicified and seritized schist(?) and orange stained white quartz veins.  (Again, poorly exposed in water fill section.)

- OVERBURDEN

  - Variably altered quartzite.  Moderately silicification and seritization common.  Often manganese oxide stained.

DDD-TR-11-05

Lithological descriptions

- OVERBURDEN (COLLUVIUM)

  - Dominantly biotite schist and quartzite

- SCHIST > QUARTZITE

  - Poorly exposed and broken dull olive schists and lesser quartzite.  Patchy sericite and/or silica (quartz veinlets?).  Weak-moderate foliation

- (manganese oxide stained with strong hematite) QUARTZITE(?)

  - Rubbly, black stained (manganese oxide), strong hematite quartzite(?).  Rare fresh surface, mainly granular quartz, unfoliated

- OVERBURDEN

  - Dominantly clasts from (3) and (5)

- QUARTZITE/SCHIST (interbedded)

  - @14m: Foliation 310/70NE.  Broken, poorly exposed.  Dull grey to green siliceous grey, interbedded quartzite and schist.  Black stain (manganese oxide)

@11-12.6m: Moderate hematite

@12.6-28.9m: Very rubbly, sheared

@19.5m: Foliation/fracture 280/70N

- COLLUVIUM

  - Dominantly small clasts of quartzite and minor schist

@31-32.5m: Interval of orange overburden with clasts of sugary white quartz containing ~10% subrounded clasts of clear quartz (<1cm)

- HORNBLENDE DIORITE

  - Angular blocky to rubbly fractured, black stained, medium pinkish grey, medium grained hornblende diorite.  Hornblende phenocrysts are subhedral to euhedral and chlorite altered.  Feldspathic aphanitic groundmass.  Hornblende ~5% and <2mm

- OVERBURDEN

  - Dominantly diorite clasts

- SCHIST

  - @45-47.7m: Green muscovite schist

@47.7-50.6m: Black biotite muscovite schist

@46.1m: 270/30N

@49.7m: 300/35N

- OVERBURDEN

  - Light grey-ish green quartzite +/- biotite and/or muscovite dominant clast type.

@68.9m: Muscovite schist common clast type

@65.7m: Frequent quartzite clasts with hematite or iron carbonate stained vugs.  Often drusy quartz lined

@86.9-89.9m: Frequent strong iron carbonate stained quartzite and infrequent weak sericite quartzite

- QUARTZ-MUSCOVITE-BIOTITE SCHIST

  - Light-dark grey quartz-muscovite-biotite schist (orange quartz-muscovite-biotite schist).  Flaggy.

@95.9m: Foliation 205/15W

@99.5m: Shear, ~40cm of broken rock and bands of gouge ~275/15N.  The bottom (south) of this zone is the contact with (12)

- (black green) MAFIC ROCK

  - Very fine grained, greenish black mafic rock with irregular rubbly fracture.  Usually 1-5% very fine to fine pyrite in clots and disseminated.  Locally up to 1% possible very fine arsenopyrite

@102.9-109.4m: Poorly exposed and decomposed black green mafic rock, commonly limonite stained

- QUARTZ-MUSCOVITE-BIOTITE SCHIST

  - Similar to (11) though more broken.

- OVERBURDEN

  - Dominantly quartz-muscovite-schist with lesser black green mafic rock with sulphides.

DDD-TR-11-06

Lithological descriptions

- (orange stained green) MUSCOVITE SCHIST

  - Dull green muscovite schists.  Increasing pervasive orange staining towards contact with (2).  Moderate sericite near contact

@0.4m: Foliation 285/40N

@2.2m: Very broken zone marks vertical(?) contact.  Strong orange and dark brown stringers.

- (limonite) Breccia

  - Rubble, intensiely blocky, brown and orange stained breccia(?).  Rare fresh surfaces show breccia similar to DDD-TR-11-01.  Grey quartz and very fine grained sulphides in patches within a very limonitic rock.  Also clear saccharoidal quartz vein material and drusy quartz vugs.

- (sericite) SCHIST

  - Broken to platy, poorly exposed moderate-strong sericitic schist.  Pale greenish white with granular quartz laminae (as seen elsewhere).  Brown to dark brown strained.  Two intervals show intense brown-black staining and frequent irregular quartz veins.

@ 3.7m: Foliation 330/45NE

@4.4-5.4m: Intense brown-black stringers and quartz veinlets

@7.9-9.3m: Intense brown-black stringers and quartz veinlets and patches of limonite breccia

- (green) MUSCOVITE SCHIST (and quartz vein)

  - Broekn, poorly exposed dull green quartz-muscovite schist

@ 11.5m: Very broken with intense back stringers

@ 12.3-12.7m: Variably broken white quartz vein with orange and brown stain on fractures

  - Quartz vein ~30cm thick and parallel to foliation

@12.9m: Cotnact parallesl foliation at 245/20N

- QUARTZ-MUSCOVITE-BIOTITE SCHIST/QUARTZITE

  - Platy, dark grey to dark olive quartz-muscovite-schist/quartzite

@12.9-13.5m: Dark grey quartz-muscovite-biotite schist

@21.4m: Quartz-muscovite-biotite schist

- OVERBURDEN

  - Colluvium dominated by quartz-muscovite schist

@23m: Float – limonite breccia(?)

- QUARTZ-MUSCOVITE SCHIST

  - Quartz-muscovite +/- biotite schist as per (5).  Minor banded quartzite.

- OVERBURDEN

  - Dominantly dark grey or green-grey quartz-muscovite schist

@39-42.9m: Orange stained quartzite dominates.

- (banded) QUARTZITE

  - Blocky, orange stained fractures, light grey banded quartzite

- OVERBURDEN

  - Dominantly banded quartzite clasts as per (9)

@61.6: Dark brown to black stringers limonite and quartz breccia clasts in float

@43.6-50.6m: Frequent small chips of orange stain or dull pale greenish-yellow (clay?).  Very fine grained rock (altered schist?)

- QUARTZ-MUSCOVITE SCHIST

  - Small flaggy pieces of green-grey quartz-muscovite schist.  In situ, but likely slumped therefore no structural measurements taken

- OVERBURDEN

  - @74.8-83.6m: Quartz-muscovite schist and light grey quartzite dominates

@75m: Float of dark to light grey quartz locally faintly banded with drusy cavities ~1% very fine grained sulphides (pyrite + ?)

@83-100.8m: Frozen ground with <1m penetration.  Same clasts as before

- BIOTITE QUARTZITE

  - Poorly exposed, dark grey biotite quartzite.

- OVERBURDEN

  - Clasts mainly dark grey biotite quartzite and green or dark grey biotite schist

@102.2m: Strong sercitized quartzite float

- QUARTZITE-BIOTITE-MUSCOVITE SCHIST

  - Poorly exposed quartz-bioitite-muscovite-schist

- OVERBURDEN

  - Dominated by muscovite schist and biotite quartzite

- MUSCOVITE-SCHIST AND BIOTITE QUARTZITE

  - @155.7-152.1m:Dull green muscovite schist

@152.1-154.5m: Dark grey biotite quartzite.  Both fairly broken.  Difficult to get a structural measurement as they look somewhat slumped.

- OVERBURDEN

  - Colluvium dominated by schist and quartzite clasts.

@164.6-224.6m: Trench ~1m deep