Data Archive Description:

Survey Details:

Survey Area Name:	Apollo Area

Project number:	14013

Client:	Kaminak Gold Corp.

Survey Company Name:	CGG

Flown Dates:	March 22 to March 24, 2014

Archive Creation Date:	April 10, 2014

Geodetic Information for map products:

Datum:	NAD83

Ellipsoid:	GRS80

Projection:	UTM Zone 7N

Central meridian:	141° West

False Easting:	500000 metres

False Northing:	0 metres

Scale factor:	0.9996

WGS84 to Local Conversion:	Molodensky

Dx,Dy,Dz:	0, 0, 0

Grid Archive:

Geosoft Grids:

| File | Description | Units |
| --- | --- | --- |
| mag | Horizontal Gradient Enhanced Total Magnetic Field | nT |
| cvg | Calculated Vertical Magnetic Gradient | nT/m |
| mhg | Measured Transverse Magnetic Gradient | nT/m |

Linedata Archive:

Geosoft Database Layout:

| Field | Variable | Description | Units |
| --- | --- | --- | --- |
| 1 | X | Easting NAD83 | m |
| 2 | Y | Northing NAD83 | m |
| 3 | fid | fiducial | - |
| 4 | longitude | Longitude WGS84 | degrees |
| 5 | latitude | Latitude WGS84 | degrees |
| 6 | flight | Flight number | - |
| 7 | date | Flight date | ddmmyy |
| 8 | altrad_heli | Helicopter height above surface from radar altimeter | m |
| 9 | altlas_heli | Helicopter height above surface from laser altimeter | m |
| 10 | gpsz | Helicopter height above geoid | m |
| 11 | dtm | Digital terrain model (above geoid) | m |
| 12 | diurnal | Measured ground magnetic intensity | nT |
| 13 | magport _raw | Total magnetic field, port sensor – spike rejected | nT |
| 14 | magstar_raw | Total magnetic field, starboard sensor – spike rejected | nT |
| 15 | magport _comp | Total magnetic field, port sensor - compensated | nT |
| 16 | magstbd_comp | Total magnetic field, starboard sensor - compensated | nT |
| 17 | mag_ave | Total magnetic field, average of port and starboard sensors | nT |
| 18 | mag_ave_lag | Total magnetic field, average of port and starboard sensors - corrected for lag | nT |
| 19 | mag_tmi | Total magnetic intensity | nT |
| 20 | transgrad | Measured transverse horizontal magnetic gradient | nT/m |
| 21 | fx | Fluxgate magnetometer, component 1 | nT |
| 22 | fy | Fluxgate magnetometer, component 2 | nT |
| 23 | fz | Fluxgate magnetometer, component 3 | nT |

Note – The null values in the GDB and XYZ archives are displayed as *.

Report:

A logistics and processing report for Project #14013 in PDF format:

R14013.pdf