140813CarcorPrelimInterp.docx

INTERPRETATIONS FOR CARLINCORE RESOURCES LTD.
OF RECCE GEOCHEMICAL SOIL DATA,
CL & HJ CLAIM BLOCKS AND 390KEAST & 490KEAST,
RACKLA BELT, EAST-CENTRAL YUKON

Colin I. Godwin, PhD, PEng, PGeo
Godwin Consultants Ltd.
660 Nootka Way, Port Moody, BC, Canada  V3H 0B7
cigodwin@yahoo.com Tel 604-939-6507
2014-08-13

Contents

INTRODUCTION	4

OVERALL SUMMARY	4

IDENTIFICATION OF ANOMALOUS AREAS ON GEOLOGICAL MAP PROVIDED	5

Figure 1. Main project area with identification of six significant anomalous areas (in order of interest from 1 through 6).	5

TABLE 1. Values for samples that have sum-totals (R&D added) greater than 40 (anomalous samples); sum-totals greater than 50 are particularly significant.	6

TABLE 2. Summary of R and D results showing number of samples for each element as related to R-values.	8

SUMMARY: ANOMALOUS AREA 1	9

SUMMARY: ANOMALOUS AREA 2	9

SUMMARY: ANOMALOUS AREA 3	9

SUMMARY: ANOMALOUS AREA 4	9

SUMMARY: ANOMALOUS AREA 5	9

SUMMARY: ANOMALOUS AREA 6	10

SUMMARY: ANOMALOUS AREA 390KEAST	10

SUMMARY: ANOMALOUS AREA 490KEAST	10

ANOMALOUS AREA 1 MAPS AND PROFILES	10

Figure 2. Anomalous Area 1 with rock interpretation from geochemical data at each sample site.	10

Figure 3. Anomalous Area 1 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.	11

Figure 4. Anomalous Area 1 contoured (inverse distance, rainbow red high) for silver (AgPpmAi); black triangles are proportional to combined silver and gold (DAgAu).	12

Figure 5. Anomalous Area 1 contoured (inverse distance, rainbow red high) for gold (fire assay: AuPpmCi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).	13

Figure 6. Anomalous Area 1: Profile 7126750N-7126250N	14

Figure 7. Anomalous Area 1: Profile 7126250N-7125750N	14

Figure 8. Anomalous Area 1: Profile 7125750N-7125250N	15

Figure 9. Anomalous Area 1: Profile 7125250N-7124750N	15

ANOMALOUS AREA 1: SUMMARY INTERPRETATION	15

TABLE 3. Anomalous Area 1: best samples with sum-totals (R&D added) greater than 40; sum-totals greater than 50 are particularly significant.	16

ANOMALOUS AREA 2 MAPS AND PROFILES	17

Figure 10. Anomalous Area 2 with rock interpretation from geochemical data at each sample site.	17

Figure 11. Anomalous Area 2 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.	18

Figure 12. Anomalous Area 2 contoured (inverse distance, rainbow red high) for silver (AgPpmAi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).	19

Figure 13. Anomalous Area 2 contoured (inverse distance, rainbow red high) for gold (fire assay: AuPpmCi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).	20

Figure 14. Anomalous Area 2: Profile 7132750N-7132250N	20

Figure 15. Anomalous Area 2: Profile 7132250N-7131750	21

ANOMALOUS AREA 2: SUMMARY INTERPRETATION	21

TABLE 4. Anomalous Area 2: best samples with sum-totals (R&D added) greater than 40.	21

ANOMALOUS AREA 3 MAPS	22

Figure 16. Anomalous Area 3 with rock interpretation from geochemical data at each sample site.	22

Figure 17. Anomalous Area 3 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.	23

Figure 18. Anomalous Area 3 contoured (inverse distance, rainbow red high) for silver (AgPpmAi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).	24

Figure 19. Anomalous Area 3 contoured (inverse distance, rainbow red high) for gold (fire assay: AuPpmCi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).	25

ANOMALOUS AREA 3: SUMMARY INTERPRETATION	25

TABLE 5. Anomalous Area 3: best samples with sum-totals (R&D added) greater than 40.	26

ANOMALOUS AREA 4 MAPS	27

Figure 20. Anomalous Area 4 with rock interpretation from geochemical data at each sample site.	27

Figure 21. Anomalous Area 4 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.	28

ANOMALOUS AREA 4: SUMMARY INTERPRETATION	28

TABLE 6. Anomalous Area 4: best samples with sum-totals (R&D added) greater than 40; sum-totals greater than 50 are particularly significant.	29

ANOMALOUS AREA 5 MAPS	30

Figure 22. Anomalous Area 5 with rock interpretation from geochemical data at each sample site.	30

Figure 23. Anomalous Area 5 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.	31

ANOMALOUS AREA 5: SUMMARY INTERPRETATION	31

TABLE 7. Anomalous Area 5: best samples with sum-totals (R&D added) greater than 40.	32

ANOMALOUS AREA 6 MAPS	33

Figure 24. Anomalous Area 6 with rock interpretation from geochemical data at each sample site.	33

Figure 25. Anomalous Area 6 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.	34

ANOMALOUS AREA 6: SUMMARY INTERPRETATION	34

TABLE 8. Anomalous Area 6: best samples with sum-totals (R&D added) greater than 40.	35

ANOMALOUS AREA 390KEAST MAPS	36

Figure 26. Anomalous Area 390kEast with rock interpretation from geochemical data at each sample site.	36

Figure 27. Anomalous Area 390kEast with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.	37

Figure 28. Anomalous Area 390kEast contoured (inverse distance, rainbow red high) for silver (AgPpmAi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).	38

Figure 29. Anomalous Area 390kEast contoured (inverse distance, rainbow red high) for gold (AuPpmCi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).	39

ANOMALOUS AREA 390kEAST: SUMMARY INTERPRETATION	39

TABLE 9. Anomalous Area 390kEast: best samples with sum-totals (R&D added) greater than 40.	40

ANOMALOUS AREA 490KEAST MAPS	41

Figure 30. Anomalous Area 490kEast with rock interpretation from geochemical data at each sample site.	41

Figure 31. Anomalous Area 490kEast with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.	42

Figure 32. Anomalous Area 390kEast contoured (inverse distance, rainbow red high) for silver (AgPpmAi); black triangle marks anomalous combined silver and gold (DAgAu >= 4.0).	43

Figure 33. Anomalous Area 390kEast contoured (inverse distance, rainbow red high) for gold (AuPpmCi); black triangle marks anomalous combined silver and gold (DAgAu >= 4.0).	44

ANOMALOUS AREA 490kEAST: SUMMARY INTERPRETATION	44

TABLE 10. Anomalous Area 490kEast: only samples with sum-total greater than 40.	45

CONCLUSIONS	45

SIGNATURE SIGNOFF	45

APPENDIX A: RvDfS OVERVIEW	46

Figure A1. Visual definition of background and anomalous populations.	47

Main D-factors used to evaluate silver gold potential from the Carlincore Resources Limited data are:	47

## INTRODUCTION

Soil geochemical data for portions of the Rackla Belt of east-central Yukon was submitted by Morgan Li of Carlincore Resources Limited. The evaluation below is based on a new geological-statistical approach developed by the writer called RvDfS or Relative Value Discrimination Factor Systematics. This technique is outlined in Appendix 1.

## OVERALL SUMMARY

Anomalous Area 390kEast is judged to be the most significant of areas examined in this report. Anomalous Area 1 is also important. Anomalous Area 4 might rank next, especially if additional assays in the area turn out to be significant. All other areas are of lessor significance based on sampling to date.

It is recommended that immediate work be focused on: Anomalous Area 390kEast, and Anomalous Area 1. Anomalous Area 4 should be followed up pending additional results. If additional results are not available additional recce soil sampling in Anomalous Area 4 is justified.

Work should begin by prospecting of those sample sites with the highest “Sum” numbers. Presence of the minerals quartz, orpiment and realgar should be looked for, noted and sampled because of their obvious significance to discovery of Carlin type mineralization. Discovery of such indicators should be used to guide and prioritize soil sampling on systematic grids.

## IDENTIFICATION OF ANOMALOUS AREAS ON GEOLOGICAL MAP PROVIDED

### Figure 1. Main project area with identification of six significant anomalous areas (in order of interest from 1 through 6).

Colored symbols represent anomalous samples as follows: red stars = Carlin signature anomalous, black up-triangles = gold plus silver anomalous, purple dots = mercury plus thallium anomalous, yellow dots = Precious metal deposit signature anomalous.

Additional to Figure 1 there are two other areas with anomalous samples. These are called: (i) 390kEast (386,000E to 392,000E and 7,188,000N to 7,196,000N), and (ii) 490kEast (482,000E to 498,000E and 7,245,000E and 7,256,000E).

The best results for all Anomalous Areas are summarized in Table 1. The sample sites are in decreasing order of “Sum”, which is a summary of a number of factors related to Carlin type precious metal deposits. Only samples considered to be anomalous (i.e., Value >= 40) are presented. Samples with Value >= 50 are markedly anomalous.

### TABLE 1. Values for samples that have sum-totals (R&D added) greater than 40 (anomalous samples); sum-totals greater than 50 are particularly significant.

| StaNo | Nad83E | Nad83N | Sum | AgPpm | AsPct | HgPpm | SPct | SbPpm | TePpm | TlPpm | AuPpm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HJ1042 | 627373 | 7122446 | 75.6 | 3.01 | 353.00 | 0.39 | 1.06 | 4.53 | 0.58 | 0.21 | 0.034 |
| HJ1074 | 627273 | 7122232 | 56.8 | 0.78 | 131.00 | 0.28 | 1.21 | 2.98 | 0.20 | 0.26 | 0.006 |
| CL0776 | 607458 | 7126444 | 56.3 | 0.71 | 74.90 | 0.23 | 0.27 | 2.37 | 0.17 | 0.31 | 0.042 |
| CL0652 | 606835 | 7126538 | 56.2 | 2.03 | 84.70 | 0.01 | 0.57 | 0.19 | 1.94 | 0.26 | 0.020 |
| CL0763 | 607655 | 7125925 | 54.9 | 0.42 | 79.40 | 0.11 | 1.03 | 2.21 | 0.17 | 0.71 | 0.022 |
| HJ0109 | 620044 | 7124526 | 54.5 | 1.66 | 5.00 | 0.01 | 1.88 | 0.22 | 0.39 | 0.10 | 0.110 |
| CL0669 | 607024 | 7126227 | 53.3 | 0.55 | 114.00 | 0.06 | 1.16 | 3.43 | 0.17 | 0.40 | 0.003 |
| CL0733 | 607236 | 7125356 | 52.6 | 0.65 | 111.00 | 0.14 | 0.41 | 3.13 | 0.20 | 0.46 | 0.006 |
| HJ0024 | 619475 | 7124843 | 52.3 | 0.38 | 67.70 | 0.80 | 0.09 | 2.47 | 0.19 | 1.07 | 0.009 |
| HJ1231 | 623887 | 7122576 | 52.0 | 0.41 | 135.00 | 0.19 | 0.13 | 3.61 | 0.33 | 0.10 | 0.013 |
| CL0660 | 606940 | 7126541 | 51.3 | 0.49 | 84.70 | 0.04 | 1.04 | 2.93 | 0.26 | 0.53 | 0.005 |
| CL0759 | 607624 | 7125747 | 51.2 | 1.31 | 87.80 | 0.15 | 0.24 | 2.41 | 0.15 | 0.38 | 0.012 |
| CL0628 | 607015 | 7125596 | 50.7 | 0.36 | 128.00 | 0.10 | 1.63 | 2.92 | 0.16 | 0.25 | 0.009 |
| HJ1073 | 627315 | 7122234 | 50.4 | 0.81 | 115.00 | 0.09 | 0.14 | 3.18 | 0.30 | 0.21 | 0.024 |
| CL0683 | 607273 | 7125834 | 50.3 | 0.47 | 86.70 | 0.11 | 0.87 | 2.95 | 0.16 | 0.65 | 0.006 |
| CL0617 | 614292 | 7128977 | 49.6 | 0.45 | 33.60 | 1.07 | 0.10 | 1.43 | 0.09 | 0.88 | 0.009 |
| HJ1080 | 627050 | 7122021 | 49.5 | 2.40 | 20.10 | 0.25 | 0.11 | 6.38 | 0.08 | 0.50 | 0.009 |
| CL0720 | 607437 | 7125887 | 48.5 | 0.59 | 107.00 | 0.08 | 0.90 | 3.31 | 0.16 | 0.30 | 0.008 |
| CL0887 | 611030 | 7121341 | 48.0 | 0.62 | 17.80 | 0.13 | 0.05 | 1.44 | 0.17 | 0.28 | 0.110 |
| CL0760 | 607598 | 7125783 | 47.9 | 0.78 | 76.00 | 0.18 | 0.31 | 2.79 | 0.14 | 0.38 | 0.008 |
| CL0066 | 614451 | 7132079 | 47.5 | 0.41 | 20.90 | 0.40 | 0.12 | 1.52 | 0.18 | 0.15 | 0.084 |
| CL0684 | 607242 | 7125775 | 47.4 | 0.69 | 60.00 | 0.17 | 0.32 | 2.12 | 0.16 | 0.36 | 0.006 |
| CL0767 | 607602 | 7126094 | 47.3 | 0.29 | 112.00 | 0.06 | 0.59 | 2.92 | 0.21 | 0.58 | 0.007 |
| HJ1075 | 627231 | 7122188 | 47.2 | 1.08 | 122.00 | 0.08 | 0.32 | 3.28 | 0.17 | 0.20 | 0.005 |
| CL0122 | 614284 | 7132201 | 47.0 | 0.15 | 26.80 | 0.97 | 0.05 | 6.00 | 0.27 | 0.11 | 0.039 |
| HJ1037 | 619853 | 7123288 | 47.0 | 0.16 | 70.70 | 2.34 | 0.09 | 2.49 | 0.09 | 0.69 | 0.007 |
| CL0855 | 610363 | 7120341 | 46.8 | 0.48 | 13.90 | 0.28 | 0.16 | 0.48 | 0.14 | 0.41 | 0.075 |
| HJ1056 | 627016 | 7123023 | 46.3 | 1.89 | 16.00 | 0.24 | 0.13 | 4.41 | 0.07 | 0.51 | 0.010 |
| CL0768 | 607555 | 7126102 | 45.9 | 0.38 | 79.80 | 0.06 | 0.62 | 2.24 | 0.16 | 0.43 | 0.011 |
| HJ0233 | 624233 | 7126224 | 45.8 | 3.47 | 30.30 | 0.33 | 0.06 | 0.97 | 0.01 | 0.24 | 0.019 |
| HJ0023 | 619453 | 7124878 | 45.6 | 0.23 | 60.20 | 0.60 | 0.19 | 1.85 | 0.28 | 0.36 | 0.009 |
| CL0724 | 607413 | 7125688 | 45.5 | 0.79 | 87.10 | 0.07 | 0.24 | 3.33 | 0.19 | 0.29 | 0.005 |
| HJ120A | 619374 | 7124706 | 45.2 | 0.12 | 70.90 | 0.47 | 0.24 | 1.94 | 0.30 | 0.30 | 0.008 |
| CL0010 | 614070 | 7132626 | 45.2 | 0.46 | 9.00 | 0.14 | 0.09 | 0.63 | 0.22 | 0.20 | 0.148 |
| CL0136 | 614683 | 7132641 | 45.1 | 0.28 | 45.70 | 0.69 | 0.08 | 3.77 | 0.27 | 0.12 | 0.013 |
| HJ1033 | 619839 | 7123489 | 45.0 | 0.20 | 76.50 | 0.47 | 0.15 | 2.39 | 0.19 | 0.50 | 0.010 |
| HJ0092 | 619709 | 7125017 | 44.8 | 0.28 | 58.00 | 1.05 | 0.08 | 1.60 | 0.22 | 0.34 | 0.008 |
| HJ1191 | 624736 | 7121019 | 44.7 | 0.48 | 73.80 | 0.07 | 0.23 | 2.34 | 0.24 | 0.27 | 0.008 |
| CL0734 | 607203 | 7125325 | 44.6 | 0.48 | 70.30 | 0.09 | 0.49 | 2.09 | 0.16 | 0.48 | 0.006 |
| CL0880 | 610854 | 7121234 | 44.6 | 0.45 | 17.90 | 1.49 | 0.07 | 3.59 | 0.12 | 0.12 | 0.026 |
| CL0123 | 614277 | 7132250 | 44.4 | 0.16 | 22.40 | 1.26 | 0.03 | 14.20 | 0.18 | 0.09 | 0.017 |
| CL0117 | 614125 | 7132047 | 44.1 | 0.14 | 20.20 | 1.64 | 0.04 | 6.08 | 0.34 | 0.08 | 0.016 |
| CL0886 | 611035 | 7121356 | 44.1 | 0.63 | 13.90 | 0.14 | 0.09 | 0.84 | 0.17 | 0.30 | 0.075 |
| CL0104 | 613641 | 7132075 | 43.8 | 0.15 | 15.50 | 1.45 | 0.05 | 6.89 | 0.28 | 0.10 | 0.024 |
| CL0116 | 614070 | 7131966 | 43.8 | 0.10 | 12.80 | 1.38 | 0.08 | 5.54 | 0.25 | 0.07 | 0.018 |
| CL0341 | 613531 | 7128582 | 43.6 | 0.40 | 8.00 | 0.22 | 0.04 | 0.31 | 0.24 | 0.23 | 0.114 |
| CL0687 | 607224 | 7125648 | 43.6 | 0.67 | 69.80 | 0.15 | 0.24 | 2.17 | 0.17 | 0.19 | 0.003 |
| CL0473 | 613714 | 7128887 | 43.4 | 0.24 | 59.20 | 0.53 | 0.06 | 1.28 | 0.12 | 0.47 | 0.027 |
| CL0661 | 606973 | 7126518 | 43.3 | 0.21 | 82.80 | 0.02 | 1.26 | 2.57 | 0.30 | 0.30 | 0.003 |
| CL0719 | 607447 | 7125921 | 43.1 | 0.71 | 65.70 | 0.05 | 0.22 | 2.14 | 0.13 | 0.34 | 0.006 |
| CL0533 | 612954 | 7128475 | 43.1 | 0.14 | 42.70 | 0.94 | 0.01 | 11.00 | 0.17 | 0.17 | 0.010 |
| CL0121 | 614269 | 7132146 | 42.9 | 0.16 | 14.90 | 0.93 | 0.07 | 8.07 | 0.29 | 0.07 | 0.018 |
| CL0723 | 607371 | 7125730 | 42.8 | 0.61 | 65.60 | 0.12 | 0.19 | 2.44 | 0.14 | 0.35 | 0.005 |
| CL0644 | 606810 | 7126198 | 42.6 | 0.84 | 44.00 | 0.18 | 0.22 | 1.41 | 0.18 | 0.18 | 0.008 |
| CL0348 | 613801 | 7128465 | 42.6 | 0.32 | 114.00 | 0.10 | 0.08 | 0.78 | 0.26 | 0.37 | 0.010 |
| CL0688 | 607239 | 7125584 | 42.1 | 0.32 | 90.50 | 0.06 | 0.40 | 2.58 | 0.15 | 0.38 | 0.005 |
| CL0806 | 607778 | 7126194 | 42.0 | 0.67 | 53.20 | 0.17 | 0.22 | 1.88 | 0.11 | 0.27 | 0.007 |
| CL0658 | 606872 | 7126618 | 41.8 | 0.56 | 68.00 | 0.09 | 0.16 | 2.34 | 0.18 | 0.24 | 0.006 |
| CL0882 | 610996 | 7121558 | 41.7 | 0.14 | 55.60 | 0.23 | 0.06 | 3.79 | 0.25 | 0.37 | 0.004 |
| HJ0568 | 625985 | 7121985 | 41.7 | 0.33 | 70.00 | 0.17 | 0.10 | 0.83 | 0.27 | 0.29 | 0.004 |
| CL0615 | 614274 | 7128878 | 41.6 | 0.26 | 19.60 | 0.86 | 0.06 | 0.92 | 0.16 | 1.23 | 0.010 |
| CL0725 | 607454 | 7125640 | 41.5 | 0.37 | 101.00 | 0.05 | 0.28 | 2.48 | 0.12 | 0.45 | 0.005 |
| CL0728 | 607375 | 7125532 | 41.3 | 0.25 | 88.70 | 0.03 | 0.95 | 2.57 | 0.12 | 0.55 | 0.005 |
| CL0534 | 613008 | 7128486 | 41.2 | 0.30 | 44.20 | 0.20 | 0.13 | 1.15 | 0.19 | 0.29 | 0.021 |
| HJ1072 | 627358 | 7122271 | 41.1 | 0.77 | 50.00 | 0.12 | 0.09 | 1.44 | 0.29 | 0.14 | 0.005 |
| HJ0021 | 619427 | 7124975 | 41.1 | 0.32 | 40.80 | 0.36 | 0.07 | 1.77 | 0.23 | 0.17 | 0.009 |
| HJ1078 | 627119 | 7122089 | 40.9 | 1.20 | 58.10 | 0.13 | 0.15 | 2.01 | 0.09 | 0.15 | 0.006 |
| HJ1055 | 627069 | 7122986 | 40.7 | 1.15 | 17.00 | 0.19 | 0.08 | 3.48 | 0.06 | 0.70 | 0.007 |
| HJ0029 | 619701 | 7124741 | 40.7 | 0.50 | 43.30 | 0.12 | 0.09 | 2.33 | 0.19 | 0.30 | 0.004 |
| CL0075 | 614657 | 7132415 | 40.4 | 0.08 | 118.00 | 0.31 | 0.02 | 2.60 | 0.20 | 0.11 | 0.009 |
| CL0769 | 607520 | 7126124 | 40.3 | 0.42 | 52.80 | 0.06 | 0.62 | 1.73 | 0.14 | 0.23 | 0.012 |
| CL0648 | 606752 | 7126364 | 40.3 | 0.68 | 31.50 | 0.17 | 0.13 | 1.31 | 0.17 | 0.26 | 0.005 |
| CL0761 | 607633 | 7125829 | 40.3 | 0.34 | 80.20 | 0.08 | 0.29 | 2.08 | 0.12 | 0.46 | 0.006 |
| CL0777 | 607436 | 7126497 | 40.2 | 0.36 | 65.30 | 0.16 | 0.17 | 1.95 | 0.13 | 0.31 | 0.004 |
| CL0766 | 607627 | 7126053 | 40.0 | 0.30 | 68.80 | 0.04 | 0.50 | 1.89 | 0.15 | 0.49 | 0.005 |

Overall results are summarized in Table 2. Because the emphasis is on identifying Carlin type silver-gold precious metal deposits those R-values and D-factors that are the most relevant are outlined in red. These all reflect elements commonly associated with Carlin deposits and are specifically: RAuCi = R-value for fire assay gold, RAuBest = R-value for the best gold in either INNA or fire assay analysis, DAgAu = D-factor for silver plus gold, DAsSb = D-factor for arsenic plus antimony, DHgTl = D-factor for mercury plus thallium, DSSe = D-factor for sulfur plus selenium, DCarl = D-factor for Carlin type deposits and DPrec = D-factor for precious metal type deposits. The formulas used for these factors are in Appendix A. The other entries in Table 2, also detailed in Appendix A, represent D-factors for rock types and major deposit types. Interpretation of the Value categories are as follows: R >=7 records the number of samples that are extremely anomalous; R >= 4 records the number of samples that are anomalous; and R >=3 records the number of samples that are marginally anomalous.

### TABLE 2. Summary of R and D results showing number of samples for each element as related to R-values.

| N=3060 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VALUE | RAuCi | RAuBest | DAgAu | DAlGa | DAsSb | DBGe | DBaP | DCaSr |
| >=7 | 19 | 25 | 8 | 335 | 20 | 2 | 13 | 206 |
| >=4 | 256 | 144 | 103 | 2465 | 384 | 294 | 2477 | 2470 |
| >3 | 256 | 144 | 166 | 2841 | 563 | 660 | 3033 | 2772 |
| VALUE | DCeLa | DCeTh | DCoNi | DCrNi | DCuMo | DCuZn | DGaGe | DGeSn |
| >=7 | 54 | 2 | 28 | 5 | 7 | 18 | 13 | 2 |
| >=4 | 595 | 391 | 331 | 179 | 245 | 933 | 504 | 166 |
| >3 | 777 | 604 | 475 | 320 | 595 | 1706 | 977 | 307 |
| VALUE | DHfZr | DHgTl | DKRb | DLaY | DLiNa | DMgTi | DMoW | DNbTa |
| >=7 | 22 | 15 | 94 | 38 | 4 | 5 | 4 | 9 |
| >=4 | 305 | 163 | 2567 | 699 | 123 | 1488 | 144 | 153 |
| >3 | 552 | 293 | 2938 | 1256 | 271 | 1946 | 240 | 279 |
| VALUE | DPV | DPbZn | DSSe | DSnW | DThU | DCarb | DFels | DMafc |
| >=7 | 26 | 4 | 22 | 7 | 12 | 126 | 2 | 2 |
| >=4 | 2334 | 151 | 718 | 209 | 367 | 2704 | 340 | 506 |
| >3 | 3031 | 229 | 1131 | 307 | 549 | 3037 | 1637 | 1611 |
| VALUE | DShal | DCarl | DCbhs | DCrbt | DPorp | DPrec | DPrsk | DSudb |
| >=7 | 0 | 1 | 0 | 0 | 0 | 3 | 2 | 6 |
| >=4 | 793 | 87 | 358 | 148 | 36 | 62 | 30 | 385 |
| >3 | 1973 | 649 | 1406 | 1301 | 328 | 240 | 450 | 907 |
| VALUE | DSxvl | DMetl |  |  |  |  |  |  |
| >=7 | 2 | 0 |  |  |  |  |  |  |
| >=4 | 201 | 48 |  |  |  |  |  |  |
| >3 | 646 | 600 |  |  |  |  |  |  |

The summaries below are comments on each of the Anomalous Areas detailed in the figures that follow. Anomalous Areas 1 to 6 are located generally in Figure 1. The areas in Figure 1 are presented in order of apparent interest—Anomalous Area 1 being of the most interest. Some data from Anomalous Area 4 is apparently missing, consequently, review of additional data from this area could enhance its relative significance among the Anomalous Areas. Anomalous Areas 390kEast and 490kEast are outside of Figure 1; only detailed maps are presented for these areas.

## SUMMARY: ANOMALOUS AREA 1

Anomalous Area 1 (Figs. 1-6, Table 2) is apparently has the best Carlin type precious metal deposit targets within Figure 1. Follow-up of this area requires, first of all, prospecting of all anomalous sample sites in Table 1. Results of this could dictate where sample grids would be best located. However, a detailed soil grid in the area of the rectangle in Anomalous Area 1 area is justified. Grid sampling must be done with a consistent sampling method and the field presence of quartz, pyrite, orpiment and realgar should be systematically recorded when found.

## SUMMARY: ANOMALOUS AREA 2

Follow-up of Anomalous Area 2 requires, first of all, prospecting of all ten sample sites in Table 4. Results of this could dictate where sample grids would be best located. However, a detailed soil grid in the areas of the two rectangles in the Anomalous Area 2 is justified. Grid sampling must be done with a consistent sampling method and the field presence of quartz, pyrite, orpiment and realgar should be systematically recorded when found.

## SUMMARY: ANOMALOUS AREA 3

Follow-up of Anomalous Area 3 requires, first of all, prospecting of all seven sample sites in Table 3. Results of this could dictate where sample grids would be best located. However, a detailed soil grid in the areas of the two rectangles in the Anomalous Area 2 is marginally justified.

## SUMMARY: ANOMALOUS AREA 4

Follow-up of Anomalous Area 4 requires, first of all, obtaining all data. Following this, prospecting of all seven sample sites in Table 6 (and other anomalous samples as determined from additional data) should be carried out. Results of this could dictate where sample grids would be best located.

## SUMMARY: ANOMALOUS AREA 5

Follow-up of Anomalous Area 5 requires, first of all, prospecting of the five sample sites in Table 7. Results of this could dictate where sample grids would be best located. However, a detailed soil grid in the areas of the two rectangles in the Anomalous Area 5 is marginally justified.

## SUMMARY: ANOMALOUS AREA 6

Follow-up of Anomalous Area 6 requires, first of all, prospecting of all nine sample sites in Table 8. Results of this could dictate where sample grids would be best located. However, a detailed soil grid in the areas of the two rectangles in the Anomalous Area 5 is marginally justified.

## SUMMARY: ANOMALOUS AREA 390KEAST

Follow-up of Anomalous Area 390kEast is of the highest priority on the areas examined in the report. Prospecting of the sample sites in Table 9 could dictate where sample grids should first be extended. In fact, a detailed soil grid over as much of the property as possible is probably justified.

## SUMMARY: ANOMALOUS AREA 490KEAST

This area is of the least interest in the areas examined in this report. Follow-up of Anomalous Area 490kEast requires, first of all, prospecting of the one sample site in Table 10. Results of this could dictate whether or not a sample grid is justified.

## ANOMALOUS AREA 1 MAPS AND PROFILES

### Figure 2. Anomalous Area 1 with rock interpretation from geochemical data at each sample site.

Colored dots indicate potential underlying rock type as follows: black dots = shale signature, blue dots = carbonate signature, yellow-orange dots = possible felsic rock signature. Grey crosses represent other sample points.

### Figure 3. Anomalous Area 1 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.

Colored symbols represent anomalous samples as follows: red stars = Carlin signature anomalous, black up-triangles = gold plus silver anomalous, purple northeast pointing arrows = mercury plus thallium anomalous, yellow southwest pointing arrows = precious metal deposit signature anomalous. Grey crosses represent other sample points.

### Figure 4. Anomalous Area 1 contoured (inverse distance, rainbow red high) for silver (AgPpmAi); black triangles are proportional to combined silver and gold (DAgAu).

### Figure 5. Anomalous Area 1 contoured (inverse distance, rainbow red high) for gold (fire assay: AuPpmCi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).

### Figure 6. Anomalous Area 1: Profile 7126750N-7126250N

Three samples have value total peaks greater than 40.

### Figure 7. Anomalous Area 1: Profile 7126250N-7125750N

Seven samples have value total peaks greater than 40.

### Figure 8. Anomalous Area 1: Profile 7125750N-7125250N

Two samples have value total peaks greater than 40.

### Figure 9. Anomalous Area 1: Profile 7125250N-7124750N

No samples have peaks with a value total greater than 40.

## ANOMALOUS AREA 1: SUMMARY INTERPRETATION

The area of principal interest in Anomalous Area 1 is outlined by a rectangle in Figures 1 to 5. Several anomalous areas exist at the northern end of the data in the northwestern and north-central part of these figures. Both of these areas have a rock-geochemical signature that has felsic characteristics. The nine samples judged to be the most significant (Sum = RAg + RAu + RAs + RHg + RS + RSb + RTl + RTe + DAgAu + DCarl + DPrec) are in Table 3. These Value sums that are greater than 40 potentially mark significant gold and silver mineralization. Silver and gold values are contoured in Figures 4 and 5, and detailed east-west stacked-bar profiles are in Figures 6 to 9. Anomalous areas in all of the above figures clearly define follow-up areas. Note that the absolute values of silver (AgPpm) and gold by fire assay (AuPpm) are low in absolute terms. The best silver is 2 grams per tonne and gold does not approach half a gram per tonne. These moderate silver and gold values might be a reflection of the soil sample medium collected in the field.

### TABLE 3. Anomalous Area 1: best samples with sum-totals (R&D added) greater than 40; sum-totals greater than 50 are particularly significant.

| StaNo | Nad83E | Nad83N | Sum | AgPpm | AsPct | HgPpm | SPct |  | SbPpm | TePpm | TlPpm | AuPpm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CL0776 | 607458 | 7126444 | 56.3 | 0.71 | 74.90 | 0.23 | 0.27 |  | 2.37 | 0.17 | 0.31 | 0.042 |
| CL0652 | 606835 | 7126538 | 56.2 | 2.03 | 84.70 | 0.01 | 0.57 |  | 0.19 | 1.94 | 0.26 | 0.020 |
| CL0763 | 607655 | 7125925 | 54.9 | 0.42 | 79.40 | 0.11 | 1.03 |  | 2.21 | 0.17 | 0.71 | 0.022 |
| CL0669 | 607024 | 7126227 | 53.3 | 0.55 | 114.00 | 0.06 | 1.16 |  | 3.43 | 0.17 | 0.40 | 0.003 |
| CL0733 | 607236 | 7125356 | 52.6 | 0.65 | 111.00 | 0.14 | 0.41 |  | 3.13 | 0.20 | 0.46 | 0.006 |
| CL0660 | 606940 | 7126541 | 51.3 | 0.49 | 84.70 | 0.04 | 1.04 |  | 2.93 | 0.26 | 0.53 | 0.005 |
| CL0759 | 607624 | 7125747 | 51.2 | 1.31 | 87.80 | 0.15 | 0.24 |  | 2.41 | 0.15 | 0.38 | 0.012 |
| CL0628 | 607015 | 7125596 | 50.7 | 0.36 | 128.00 | 0.10 | 1.63 |  | 2.92 | 0.16 | 0.25 | 0.009 |
| CL0683 | 607273 | 7125834 | 50.3 | 0.47 | 86.70 | 0.11 | 0.87 |  | 2.95 | 0.16 | 0.65 | 0.006 |
| CL0720 | 607437 | 7125887 | 48.5 | 0.59 | 107.00 | 0.08 | 0.90 |  | 3.31 | 0.16 | 0.30 | 0.008 |
| CL0760 | 607598 | 7125783 | 47.9 | 0.78 | 76.00 | 0.18 | 0.31 |  | 2.79 | 0.14 | 0.38 | 0.008 |
| CL0684 | 607242 | 7125775 | 47.4 | 0.69 | 60.00 | 0.17 | 0.32 |  | 2.12 | 0.16 | 0.36 | 0.006 |
| CL0767 | 607602 | 7126094 | 47.3 | 0.29 | 112.00 | 0.06 | 0.59 |  | 2.92 | 0.21 | 0.58 | 0.007 |
| CL0768 | 607555 | 7126102 | 45.9 | 0.38 | 79.80 | 0.06 | 0.62 |  | 2.24 | 0.16 | 0.43 | 0.011 |
| CL0724 | 607413 | 7125688 | 45.5 | 0.79 | 87.10 | 0.07 | 0.24 |  | 3.33 | 0.19 | 0.29 | 0.005 |
| CL0734 | 607203 | 7125325 | 44.6 | 0.48 | 70.30 | 0.09 | 0.49 |  | 2.09 | 0.16 | 0.48 | 0.006 |
| CL0687 | 607224 | 7125648 | 43.6 | 0.67 | 69.80 | 0.15 | 0.24 |  | 2.17 | 0.17 | 0.19 | 0.003 |
| CL0661 | 606973 | 7126518 | 43.3 | 0.21 | 82.80 | 0.02 | 1.26 |  | 2.57 | 0.30 | 0.30 | 0.003 |
| CL0719 | 607447 | 7125921 | 43.1 | 0.71 | 65.70 | 0.05 | 0.22 |  | 2.14 | 0.13 | 0.34 | 0.006 |
| CL0723 | 607371 | 7125730 | 42.8 | 0.61 | 65.60 | 0.12 | 0.19 |  | 2.44 | 0.14 | 0.35 | 0.005 |
| CL0644 | 606810 | 7126198 | 42.6 | 0.84 | 44.00 | 0.18 | 0.22 |  | 1.41 | 0.18 | 0.18 | 0.008 |
| CL0688 | 607239 | 7125584 | 42.1 | 0.32 | 90.50 | 0.06 | 0.40 |  | 2.58 | 0.15 | 0.38 | 0.005 |
| CL0806 | 607778 | 7126194 | 42.0 | 0.67 | 53.20 | 0.17 | 0.22 |  | 1.88 | 0.11 | 0.27 | 0.007 |
| CL0658 | 606872 | 7126618 | 41.8 | 0.56 | 68.00 | 0.09 | 0.16 |  | 2.34 | 0.18 | 0.24 | 0.006 |
| CL0725 | 607454 | 7125640 | 41.5 | 0.37 | 101.00 | 0.05 | 0.28 |  | 2.48 | 0.12 | 0.45 | 0.005 |
| CL0728 | 607375 | 7125532 | 41.3 | 0.25 | 88.70 | 0.03 | 0.95 |  | 2.57 | 0.12 | 0.55 | 0.005 |
| CL0769 | 607520 | 7126124 | 40.3 | 0.42 | 52.80 | 0.06 | 0.62 |  | 1.73 | 0.14 | 0.23 | 0.012 |
| CL0648 | 606752 | 7126364 | 40.3 | 0.68 | 31.50 | 0.17 | 0.13 |  | 1.31 | 0.17 | 0.26 | 0.005 |
| CL0761 | 607633 | 7125829 | 40.3 | 0.34 | 80.20 | 0.08 | 0.29 |  | 2.08 | 0.12 | 0.46 | 0.006 |
| CL0777 | 607436 | 7126497 | 40.2 | 0.36 | 65.30 | 0.16 | 0.17 |  | 1.95 | 0.13 | 0.31 | 0.004 |
| CL0766 | 607627 | 7126053 | 40.0 | 0.30 | 68.80 | 0.04 | 0.50 |  | 1.89 | 0.15 | 0.49 | 0.005 |

Follow-up of Anomalous Area 1 requires, first of all, prospecting of all anomalous sample sites in Table 3. Results of this could dictate where sample grids would be best located. However, a detailed soil grid in the area of the rectangle in Anomalous Area 1 area is justified. Grid sampling must be done with a consistent sampling method and the field presence of quartz, pyrite, orpiment and realgar should be systematically recorded when found.

## ANOMALOUS AREA 2 MAPS AND PROFILES

### Figure 10. Anomalous Area 2 with rock interpretation from geochemical data at each sample site.

Colored dots indicate potential underlying rock type as follows: black dots = shale signature, blue dots = carbonate signature, yellow-orange dots = possible felsic rock signature.

### Figure 11. Anomalous Area 2 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.

Colored symbols represent anomalous samples as follows: red stars = Carlin signature anomalous, black up-triangles = gold plus silver anomalous, purple northeast pointing arrows = mercury plus thallium anomalous, yellow southwest pointing arrows = precious metal deposit signature anomalous. Grey crosses represent other sample points.

### Figure 12. Anomalous Area 2 contoured (inverse distance, rainbow red high) for silver (AgPpmAi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).

### Figure 13. Anomalous Area 2 contoured (inverse distance, rainbow red high) for gold (fire assay: AuPpmCi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).

### Figure 14. Anomalous Area 2: Profile 7132750N-7132250N

Three peaks have Value greater than 40.

### Figure 15. Anomalous Area 2: Profile 7132250N-7131750

Five peaks have Value greater than 40.

## ANOMALOUS AREA 2: SUMMARY INTERPRETATION

The area of principal interest in Anomalous Area 1 is outlined by the two rectangles in Figures 10 to 13. The nine samples judged to be the most significant (Sum = RAg + RAu + RAs + RHg + RS + RSb + RTl + RTe + DAgAu + DCarl + DPrec) are in Table 2 (see also stacked-bar profiles of Figs. 14 & 15). These Value sums that are greater than 40 potentially mark significant gold and silver mineralization, but note that these values are less than those associated with Anomalous Area 1 (Table 3). Note that the absolute values of silver (AgPpm) and gold by fire assay (AuPpm) are low in absolute terms. The best silver is a little less than half a gram per tonne and gold does not approach half a gram per tonne. These moderate silver-gold results might be a reflection of the soil sample medium collected in the field.

### TABLE 4. Anomalous Area 2: best samples with sum-totals (R&D added) greater than 40.

| StaNo | Nad83E | Nad83N | Sum | AgPpm | AsPct | HgPpm | SPct | SbPpm | TePpm | TlPpm | AuPpm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CL0066 | 614451 | 7132079 | 47.5 | 0.41 | 20.90 | 0.40 | 0.12 | 1.52 | 0.18 | 0.15 | 0.084 |
| CL0122 | 614284 | 7132201 | 47.0 | 0.15 | 26.80 | 0.97 | 0.05 | 6.00 | 0.27 | 0.11 | 0.039 |
| CL0010 | 614070 | 7132626 | 45.2 | 0.46 | 9.00 | 0.14 | 0.09 | 0.63 | 0.22 | 0.20 | 0.148 |
| CL0136 | 614683 | 7132641 | 45.1 | 0.28 | 45.70 | 0.69 | 0.08 | 3.77 | 0.27 | 0.12 | 0.013 |
| CL0123 | 614277 | 7132250 | 44.4 | 0.16 | 22.40 | 1.26 | 0.03 | 14.20 | 0.18 | 0.09 | 0.017 |
| CL0117 | 614125 | 7132047 | 44.1 | 0.14 | 20.20 | 1.64 | 0.04 | 6.08 | 0.34 | 0.08 | 0.016 |
| CL0104 | 613641 | 7132075 | 43.8 | 0.15 | 15.50 | 1.45 | 0.05 | 6.89 | 0.28 | 0.10 | 0.024 |
| CL0116 | 614070 | 7131966 | 43.8 | 0.10 | 12.80 | 1.38 | 0.08 | 5.54 | 0.25 | 0.07 | 0.018 |
| CL0121 | 614269 | 7132146 | 42.9 | 0.16 | 14.90 | 0.93 | 0.07 | 8.07 | 0.29 | 0.07 | 0.018 |
| CL0075 | 614657 | 7132415 | 40.4 | 0.08 | 118.00 | 0.31 | 0.02 | 2.60 | 0.20 | 0.11 | 0.009 |

Follow-up of Anomalous Area 2 requires, first of all, prospecting of all ten sample sites in Table 4. Results of this could dictate where sample grids would be best located. However, a detailed soil grid in the areas of the two rectangles in the Anomalous Area 2 is justified. Grid sampling must be done with a consistent sampling method and the field presence of quartz, pyrite, orpiment and realgar should be systematically recorded when found.

## ANOMALOUS AREA 3 MAPS

### Figure 16. Anomalous Area 3 with rock interpretation from geochemical data at each sample site.

Colored dots indicate potential underlying rock type as follows: black dots = shale signature, blue dots = carbonate signature, yellow-orange dots = possible felsic rock signature.

### Figure 17. Anomalous Area 3 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.

Colored symbols represent anomalous samples as follows: red stars = Carlin signature anomalous, black up-triangles = gold plus silver anomalous, purple northeast pointing arrows = mercury plus thallium anomalous, yellow southwest pointing arrows = precious metal deposit signature anomalous. Grey crosses represent other sample points.

### Figure 18. Anomalous Area 3 contoured (inverse distance, rainbow red high) for silver (AgPpmAi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).

### Figure 19. Anomalous Area 3 contoured (inverse distance, rainbow red high) for gold (fire assay: AuPpmCi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).

## ANOMALOUS AREA 3: SUMMARY INTERPRETATION

The area of principal interest in Anomalous Area 3 is outlined by a rectangle in Figures 16 and 17. A smaller anomaly exists at the central-northern boundary of the area. The anomalous samples are generally sparsely distributed throughout this area. The seven samples judged to be the most significant (Sum = RAg + RAu + RAs + RHg + RS + RSb + RTl + RTe + DAgAu + DCarl + DPrec) are in Table 5. These Value sums that are greater than 40 potentially mark significant gold and silver mineralization. Note that the absolute values of silver (AgPpm) and gold by fire assay (AuPpm) are low in absolute terms. The best silver is less than half a gram per tonne and gold does not approach half a gram per tonne. These moderate values might be a reflection of the soil sample medium collected in the field.

Follow-up of Anomalous Area 3 requires, first of all, prospecting of all seven sample sites in Table 5. Results of this could dictate where sample grids would be best located. However, a detailed soil grid in the areas of the two rectangles in the Anomalous Area 2 is marginally justified. Grid sampling must be done with a consistent sampling method and the field presence of quartz, pyrite, orpiment and realgar should be systematically recorded when found.

### TABLE 5. Anomalous Area 3: best samples with sum-totals (R&D added) greater than 40.

| StaNo | Nad83E | Nad83N | Sum | AgPpm | AsPct | HgPpm | SPct | SbPpm | TePpm | TlPpm | AuPpm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CL0617 | 614292 | 7128977 | 49.6 | 0.45 | 33.60 | 1.07 | 0.10 | 1.43 | 0.09 | 0.88 | 0.009 |
| CL0341 | 613531 | 7128582 | 43.6 | 0.40 | 8.00 | 0.22 | 0.04 | 0.31 | 0.24 | 0.23 | 0.114 |
| CL0473 | 613714 | 7128887 | 43.4 | 0.24 | 59.20 | 0.53 | 0.06 | 1.28 | 0.12 | 0.47 | 0.027 |
| CL0533 | 612954 | 7128475 | 43.1 | 0.14 | 42.70 | 0.94 | 0.01 | 11.00 | 0.17 | 0.17 | 0.010 |
| CL0348 | 613801 | 7128465 | 42.6 | 0.32 | 114.00 | 0.10 | 0.08 | 0.78 | 0.26 | 0.37 | 0.010 |
| CL0615 | 614274 | 7128878 | 41.6 | 0.26 | 19.60 | 0.86 | 0.06 | 0.92 | 0.16 | 1.23 | 0.010 |
| CL0534 | 613008 | 7128486 | 41.2 | 0.30 | 44.20 | 0.20 | 0.13 | 1.15 | 0.19 | 0.29 | 0.021 |

## ANOMALOUS AREA 4 MAPS

### Figure 20. Anomalous Area 4 with rock interpretation from geochemical data at each sample site.

Colored dots indicate potential underlying rock type as follows: black dots = shale signature, blue dots = carbonate signature, yellow-orange dots = possible felsic rock signature. Grey crosses represent other sample points.

### Figure 21. Anomalous Area 4 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.

Colored symbols represent anomalous samples as follows: red stars = Carlin signature anomalous, black up-triangles = gold plus silver anomalous, purple northeast pointing arrows = mercury plus thallium anomalous, yellow southwest pointing arrows = precious metal deposit signature anomalous. Grey crosses represent other sample points.

## ANOMALOUS AREA 4: SUMMARY INTERPRETATION

The area of principal interest in Anomalous Area is outlined by a rectangle in Figures 20 and 21. The anomalous samples are concentrated in one part of the line samples. Other maps (Li, pers. comm, 14-08-12) show that there are additional lines—four of which are in the area of the rectangle. Assay values for these lines are not known and this data is critical to overall evaluation of this area. However, for the data at hand, the seven samples judged to be the most significant (Sum = RAg + RAu + RAs + RHg + RS + RSb + RTl + RTe + DAgAu + DCarl + DPrec) are in Table 6. These Value sums that are greater than 40 potentially mark significant gold and silver mineralization. Note that the absolute values of silver (AgPpm) and gold by fire assay (AuPpm) are low in absolute terms. The best silver is about 3 grams per tonne and gold does not approach half a gram per tonne. These moderate values might be a reflection of the soil sample medium collected in the field.

Follow-up of Anomalous Area 4 requires, first of all, obtaining all data. Following this, prospecting of all seven sample sites in Table 6 (and other anomalous samples as determined from additional data) should be carried out. Results of this could dictate where sample grids would be best located. Grid sampling must be done with a consistent sampling method and the field presence of quartz, pyrite, orpiment and realgar should be systematically recorded when found.

### TABLE 6. Anomalous Area 4: best samples with sum-totals (R&D added) greater than 40; sum-totals greater than 50 are particularly significant.

| StaNo | Nad83E | Nad83N | Sum | AgPpm | AsPct | HgPpm | SPct | SbPpm | TePpm | TlPpm | AuPpm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HJ1042 | 627373 | 7122446 | 75.6 | 3.01 | 353.00 | 0.39 | 1.06 | 4.53 | 0.58 | 0.21 | 0.034 |
| HJ1074 | 627273 | 7122232 | 56.8 | 0.78 | 131.00 | 0.28 | 1.21 | 2.98 | 0.20 | 0.26 | 0.006 |
| HJ1073 | 627315 | 7122234 | 50.4 | 0.81 | 115.00 | 0.09 | 0.14 | 3.18 | 0.30 | 0.21 | 0.024 |
| HJ1080 | 627050 | 7122021 | 49.5 | 2.40 | 20.10 | 0.25 | 0.11 | 6.38 | 0.08 | 0.50 | 0.009 |
| HJ1075 | 627231 | 7122188 | 47.2 | 1.08 | 122.00 | 0.08 | 0.32 | 3.28 | 0.17 | 0.20 | 0.005 |
| HJ1072 | 627358 | 7122271 | 41.1 | 0.77 | 50.00 | 0.12 | 0.09 | 1.44 | 0.29 | 0.14 | 0.005 |
| HJ1078 | 627119 | 7122089 | 40.9 | 1.20 | 58.10 | 0.13 | 0.15 | 2.01 | 0.09 | 0.15 | 0.006 |

## ANOMALOUS AREA 5 MAPS

### Figure 22. Anomalous Area 5 with rock interpretation from geochemical data at each sample site.

Colored dots indicate potential underlying rock type as follows: black dots = shale signature, blue dots = carbonate signature, yellow-orange dots = possible felsic rock signature.

### Figure 23. Anomalous Area 5 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.

Colored symbols represent anomalous samples as follows: red stars = Carlin signature anomalous, black up-triangles = gold plus silver anomalous, purple northeast pointing arrows = mercury plus thallium anomalous, yellow southwest pointing arrows = precious metal deposit signature anomalous. Grey crosses represent other sample points.

## ANOMALOUS AREA 5: SUMMARY INTERPRETATION

The area of principal interest in Anomalous Area 5 is outlined by a rectangle in Figures 22 and 23. The anomalous samples are sparsely distributed throughout this area. The five samples judged to be the most significant (Sum = RAg + RAu + RAs + RHg + RS + RSb + RTl + RTe + DAgAu + DCarl + DPrec) are in Table 7. Those sum-totals sums that are greater than 40 potentially mark significant gold and silver mineralization. Note that the absolute values of silver (AgPpm) and gold by fire assay (AuPpm) are low in absolute terms. The best silver is about half a gram per tonne and gold does not approach half a gram per tonne. This is not encouraging, but might be a reflection of the soil sample medium collected in the field.

Follow-up of Anomalous Area 5 requires, first of all, prospecting of the five sample sites in Table 7. Results of this could dictate where sample grids would be best located. However, a detailed soil grid in the areas of the two rectangles in the Anomalous Area 5 is marginally justified. Grid sampling must be done with a consistent sampling method and the field presence of quartz, pyrite, orpiment and realgar should be systematically recorded when found.

### TABLE 7. Anomalous Area 5: best samples with sum-totals (R&D added) greater than 40.

| StaNo | Nad83E | Nad83N | Sum | AgPpm | AsPct | HgPpm | SPct | SbPpm | TePpm | TlPpm | AuPpm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CL0887 | 611030 | 7121341 | 48.0 | 0.62 | 17.80 | 0.13 | 0.05 | 1.44 | 0.17 | 0.28 | 0.110 |
| CL0855 | 610363 | 7120341 | 46.8 | 0.48 | 13.90 | 0.28 | 0.16 | 0.48 | 0.14 | 0.41 | 0.075 |
| CL0880 | 610854 | 7121234 | 44.6 | 0.45 | 17.90 | 1.49 | 0.07 | 3.59 | 0.12 | 0.12 | 0.026 |
| CL0886 | 611035 | 7121356 | 44.1 | 0.63 | 13.90 | 0.14 | 0.09 | 0.84 | 0.17 | 0.30 | 0.075 |
| CL0882 | 610996 | 7121558 | 41.7 | 0.14 | 55.60 | 0.23 | 0.06 | 3.79 | 0.25 | 0.37 | 0.004 |

## ANOMALOUS AREA 6 MAPS

### Figure 24. Anomalous Area 6 with rock interpretation from geochemical data at each sample site.

Colored dots indicate potential underlying rock type as follows: black dots = shale signature, blue dots = carbonate signature, yellow-orange dots = possible felsic rock signature.

### Figure 25. Anomalous Area 6 with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.

Colored symbols represent anomalous samples as follows: red stars = Carlin signature anomalous, black up-triangles = gold plus silver anomalous, purple northeast pointing arrows = mercury plus thallium anomalous, yellow southwest pointing arrows = precious metal deposit signature anomalous. Grey crosses represent other sample points.

## ANOMALOUS AREA 6: SUMMARY INTERPRETATION

The area of principal interest in Anomalous Area 6 is outlined by two rectangles in Figures 24 and 25. The anomalous samples are sparsely distributed throughout this area. The nine samples judged to be the most significant (Sum = RAg + RAu + RAs + RHg + RS + RSb + RTl + RTe + DAgAu + DCarl + DPrec) are in Table 8. Those sum-totals that are greater than 40 potentially mark significant gold and silver mineralization. Note that the absolute values of silver (AgPpm) and gold by fire assay (AuPpm) are low in absolute terms. The best silver is about one and a half grams per tonne and gold does not approach half a gram per tonne. These moderate silver and gold results might be a reflection of the soil sample medium collected in the field.

Follow-up of Anomalous Area 6 requires, first of all, prospecting of all nine sample sites in Table 8. Results of this could dictate where sample grids would be best located. However, a detailed soil grid in the areas of the two rectangles in the Anomalous Area 5 is marginally justified. Grid sampling must be done with a consistent sampling method and the field presence of quartz, pyrite, orpiment and realgar should be systematically recorded when found.

### TABLE 8. Anomalous Area 6: best samples with sum-totals (R&D added) greater than 40.

| StaNo | Nad83E | Nad83N | Sum | AgPpm | AsPct | HgPpm | SPct | SbPpm | TePpm | TlPpm | AuPpm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HJ0109 | 620044 | 7124526 | 54.5 | 1.66 | 5.00 | 0.01 | 1.88 | 0.22 | 0.39 | 0.10 | 0.110 |
| HJ0024 | 619475 | 7124843 | 52.3 | 0.38 | 67.70 | 0.80 | 0.09 | 2.47 | 0.19 | 1.07 | 0.009 |
| HJ1037 | 619853 | 7123288 | 47.0 | 0.16 | 70.70 | 2.34 | 0.09 | 2.49 | 0.09 | 0.69 | 0.007 |
| HJ0023 | 619453 | 7124878 | 45.6 | 0.23 | 60.20 | 0.60 | 0.19 | 1.85 | 0.28 | 0.36 | 0.009 |
| HJ120A | 619374 | 7124706 | 45.2 | 0.12 | 70.90 | 0.47 | 0.24 | 1.94 | 0.30 | 0.30 | 0.008 |
| HJ1033 | 619839 | 7123489 | 45.0 | 0.20 | 76.50 | 0.47 | 0.15 | 2.39 | 0.19 | 0.50 | 0.010 |
| HJ0092 | 619709 | 7125017 | 44.8 | 0.28 | 58.00 | 1.05 | 0.08 | 1.60 | 0.22 | 0.34 | 0.008 |
| HJ0021 | 619427 | 7124975 | 41.1 | 0.32 | 40.80 | 0.36 | 0.07 | 1.77 | 0.23 | 0.17 | 0.009 |
| HJ0029 | 619701 | 7124741 | 40.7 | 0.50 | 43.30 | 0.12 | 0.09 | 2.33 | 0.19 | 0.30 | 0.004 |

## ANOMALOUS AREA 390KEAST MAPS

### Figure 26. Anomalous Area 390kEast with rock interpretation from geochemical data at each sample site.

Colored dots indicate potential underlying rock type as follows: black dots = shale signature, blue dots = carbonate signature, yellow-orange dots = possible felsic rock signature. Grey crosses represent other sample points.

### Figure 27. Anomalous Area 390kEast with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.

Colored symbols represent anomalous samples as follows: red stars = Carlin signature anomalous, black up-triangles = gold plus silver anomalous, purple northeast pointing arrows = mercury plus thallium anomalous, yellow southwest pointing arrows = precious metal deposit signature anomalous. Black dots represent other sample points.

### Figure 28. Anomalous Area 390kEast contoured (inverse distance, rainbow red high) for silver (AgPpmAi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).

### Figure 29. Anomalous Area 390kEast contoured (inverse distance, rainbow red high) for gold (AuPpmCi); the size of the black triangles are proportional to combined silver and gold (bubble plot for DAgAu).

## ANOMALOUS AREA 390kEAST: SUMMARY INTERPRETATION

Most of the areas sampled in Anomalous Area 390kEast (Figs. 26 to 29) are of interest. There are a large number of anomalous samples in this area. The thirty-nine samples judged to be the most significant (Sum = RAg + RAu + RAs + RHg + RS + RSb + RTl + RTe + DAgAu + DCarl + DPrec) are in Table 9. Thse sum-totals that are greater than 40 potentially mark significant gold and silver mineralization. Note that the absolute values of silver (AgPpm) and gold by fire assay (AuPpm) are low in absolute terms. However, the absolute values for two samples have silver (AgPpm) silver values greater than 10 grams per tonne—they are the best silver values encountered in this project. Gold is less than half a gram per tonne. This area has the best results of any area reported here.

Follow-up of Anomalous Area 390kEast is of the highest priority on the areas examined in the report. Prospecting of the sample sites in Table 9 could dictate where sample grids should first be extended. In fact, a detailed soil grid over as much of the property as possible is probably justified. Grid sampling must be done with a consistent sampling method and the field presence of quartz, pyrite, orpiment and realgar should be systematically recorded when found.

### TABLE 9. Anomalous Area 390kEast: best samples with sum-totals (R&D added) greater than 40.

| StaNo | Nad83E | Nad83N | Sum | AgPpm | AsPct | HgPpm | SPct | SbPpm | TePpm | TlPpm | AuPpm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DJ0048 | 390968 | 7190061 | 92.4 | 12.60 | 107.00 | 0.90 | 0.44 | 46.00 | 1.15 | 2.07 | 0.057 |
| DJ0046 | 390924 | 7189971 | 80.6 | 11.40 | 56.80 | 0.81 | 0.17 | 25.30 | 0.33 | 2.68 | 0.027 |
| DJ1081 | 390070 | 7190380 | 71.3 | 3.47 | 53.40 | 0.23 | 0.13 | 22.70 | 0.35 | 0.71 | 0.062 |
| DJ0310 | 388291 | 7191502 | 64.9 | 3.77 | 31.10 | 0.35 | 0.25 | 15.80 | 0.09 | 1.08 | 0.013 |
| DJ0047 | 390955 | 7189998 | 61.3 | 1.68 | 43.60 | 0.20 | 0.09 | 17.90 | 0.36 | 1.13 | 0.006 |
| DJ0293 | 388739 | 7191649 | 54.0 | 1.86 | 14.00 | 0.14 | 0.16 | 3.17 | 0.04 | 0.30 | 0.169 |
| DJ0060 | 391456 | 7190148 | 53.7 | 0.87 | 32.30 | 0.33 | 0.60 | 4.16 | 0.02 | 1.22 | 0.013 |
| DJ1052 | 386948 | 7188743 | 51.5 | 1.70 | 22.80 | 0.21 | 0.15 | 5.22 | 0.05 | 1.76 | 0.011 |
| DJ0312 | 388268 | 7191404 | 49.7 | 1.76 | 21.30 | 0.32 | 0.13 | 5.41 | 0.04 | 0.45 | 0.014 |
| DJ0058 | 391391 | 7190225 | 49.5 | 1.22 | 24.30 | 0.32 | 0.40 | 3.69 | 0.10 | 0.42 | 0.009 |
| DJ0398 | 388945 | 7194109 | 48.9 | 0.76 | 12.20 | 0.10 | 0.11 | 2.48 | 0.03 | 0.26 | 0.363 |
| DJ1005 | 386802 | 7189291 | 48.8 | 2.23 | 21.10 | 0.11 | 0.21 | 5.69 | 0.05 | 0.45 | 0.019 |
| DJ0288 | 388799 | 7191446 | 48.7 | 1.68 | 9.50 | 0.11 | 0.06 | 2.31 | 0.03 | 0.19 | 0.475 |
| DJ1082 | 390051 | 7190420 | 47.3 | 1.82 | 19.70 | 0.16 | 0.18 | 5.66 | 0.17 | 0.27 | 0.013 |
| DJ0241 | 389134 | 7191115 | 47.2 | 1.54 | 17.80 | 0.30 | 0.13 | 2.78 | 0.07 | 0.42 | 0.019 |
| DJ0295 | 388657 | 7191708 | 47.1 | 1.52 | 16.00 | 0.14 | 0.12 | 3.75 | 0.04 | 4.78 | 0.004 |
| DJ0397 | 388951 | 7194048 | 46.9 | 0.86 | 12.60 | 0.10 | 0.15 | 3.21 | 0.03 | 0.29 | 0.145 |
| DJ0045 | 390877 | 7189969 | 46.5 | 0.88 | 11.90 | 0.31 | 0.18 | 2.51 | 0.05 | 0.36 | 0.043 |
| DJ1014 | 387201 | 7189080 | 46.0 | 2.13 | 15.90 | 0.17 | 0.11 | 5.31 | 0.04 | 0.35 | 0.011 |
| DJ0311 | 388257 | 7191446 | 45.1 | 1.63 | 20.80 | 0.13 | 0.14 | 4.98 | 0.06 | 0.67 | 0.012 |
| DJ0419 | 389254 | 7194014 | 44.9 | 1.12 | 12.70 | 0.14 | 0.19 | 3.56 | 0.04 | 0.30 | 0.048 |
| DJ0294 | 388693 | 7191684 | 44.7 | 1.86 | 16.60 | 0.20 | 0.39 | 4.52 | 0.04 | 0.58 | 0.008 |
| DJ1007 | 386902 | 7189249 | 44.3 | 1.97 | 25.00 | 0.14 | 0.15 | 6.92 | 0.05 | 0.62 | 0.004 |
| DJ0420 | 389256 | 7194056 | 43.5 | 4.81 | 16.40 | 0.14 | 0.19 | 4.27 | 0.03 | 0.28 | 0.007 |
| DJ1080 | 390095 | 7190320 | 43.4 | 0.82 | 12.00 | 0.30 | 0.25 | 1.62 | 0.07 | 0.42 | 0.013 |
| DJ1013 | 387147 | 7189099 | 43.2 | 2.09 | 25.10 | 0.09 | 0.12 | 6.34 | 0.05 | 0.35 | 0.005 |
| DJ1004 | 386758 | 7189309 | 43.1 | 2.60 | 20.00 | 0.07 | 0.21 | 6.69 | 0.05 | 0.52 | 0.008 |
| DJ0242 | 389076 | 7191116 | 43.0 | 1.52 | 13.60 | 0.17 | 0.10 | 2.95 | 0.04 | 0.37 | 0.010 |
| DJ0057 | 391344 | 7190248 | 41.8 | 1.22 | 12.40 | 0.17 | 0.15 | 2.88 | 0.05 | 0.45 | 0.007 |
| DJ1006 | 386857 | 7189250 | 41.4 | 2.00 | 21.10 | 0.11 | 0.17 | 4.91 | 0.05 | 0.38 | 0.007 |
| DJ0055 | 391256 | 7190282 | 41.4 | 1.36 | 12.70 | 0.07 | 0.15 | 3.23 | 0.03 | 1.00 | 0.004 |
| DJ1061 | 387103 | 7188494 | 40.9 | 0.72 | 13.70 | 0.10 | 0.17 | 1.55 | 0.08 | 0.40 | 0.027 |
| DJ1002 | 386662 | 7189363 | 40.8 | 1.91 | 19.10 | 0.09 | 0.15 | 4.80 | 0.04 | 0.43 | 0.010 |
| DJ0285 | 388665 | 7191363 | 40.6 | 0.89 | 10.10 | 0.06 | 0.07 | 2.04 | 0.06 | 0.43 | 0.062 |
| DJ0276 | 388748 | 7191038 | 40.5 | 1.01 | 14.40 | 0.17 | 0.11 | 2.90 | 0.04 | 0.51 | 0.006 |
| DJ1003 | 386707 | 7189328 | 40.4 | 1.44 | 17.80 | 0.13 | 0.11 | 4.44 | 0.03 | 0.33 | 0.006 |
| DJ0297 | 388654 | 7191822 | 40.3 | 1.55 | 13.50 | 0.14 | 0.14 | 3.16 | 0.04 | 0.35 | 0.005 |
| DJ0243 | 389029 | 7191120 | 40.2 | 1.30 | 12.00 | 0.13 | 0.10 | 2.36 | 0.06 | 0.42 | 0.011 |
| DJ0309 | 388324 | 7191515 | 40.0 | 1.58 | 17.30 | 0.07 | 0.11 | 6.69 | 0.06 | 0.48 | 0.005 |

## ANOMALOUS AREA 490KEAST MAPS

### Figure 30. Anomalous Area 490kEast with rock interpretation from geochemical data at each sample site.

Colored dots indicate potential underlying rock type as follows: black dots = shale signature, blue dots = carbonate signature, yellow-orange dots = possible felsic rock signature.

### Figure 31. Anomalous Area 490kEast with symbols representing precious metal related (silver and gold related) factors based on geochemical data at each sample site.

Colored symbols represent anomalous samples (D-factors >= 4.0) as follows: black up-triangles = gold plus silver, purple thick plus = mercury plus thallium; Carlin and precious metal deposit anomalous signatures are not present.

### Figure 32. Anomalous Area 390kEast contoured (inverse distance, rainbow red high) for silver (AgPpmAi); black triangle marks anomalous combined silver and gold (DAgAu >= 4.0).

### Figure 33. Anomalous Area 390kEast contoured (inverse distance, rainbow red high) for gold (AuPpmCi); black triangle marks anomalous combined silver and gold (DAgAu >= 4.0).

## ANOMALOUS AREA 490kEAST: SUMMARY INTERPRETATION

The area of principal interest in Anomalous Area 490kEast is in the southwestern corner of Figures 30 to 33. Strongly anomalous samples are rare. The one samples judged to be significant (Sum = RAg + RAu + RAs + RHg + RS + RSb + RTl + RTe + DAgAu + DCarl + DPrec) is in Table 10. The one sum-totals greater than 40 potentially marks significant gold and silver mineralization. Note that the absolute values of silver (AgPpm) and gold by fire assay (AuPpm) is low in absolute terms (both much less than half a gram per tonne). This might in part be a reflection of the soil sample medium collected in the field.

This area is of the least interest in the areas examined in this report. Follow-up of Anomalous Area 490kEast requires, first of all, prospecting of the one anomalous sample site identified in Table 10. Results of this could dictate whether or not a sample grid is justified. Any grid sampling must be done with a consistent sampling method and the field presence of quartz, pyrite, orpiment and realgar should be systematically recorded when found.

### TABLE 10. Anomalous Area 490kEast: only samples with sum-total greater than 40.

| StaNo | Nad83E | Nad83N | Sum | AgPpm | AsPct | HgPpm | SPct | SbPpm | TePpm | TlPpm | AuPpm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 686 | 490249 | 7247352 | 49.7 | 0.25 | 92.00 | 0.76 | 1.97 | 0.14 | 0.07 | 1.05 | 0.002 |

## CONCLUSIONS

Anomalous Area 390kEast is judged to be the most significant of areas examined in this report. Anomalous Area 1 is also important. Anomalous Area 4 might rank next, especially if additional assays in the area turn out to be significant. All other areas are of lessor significance based on sampling to date.

It is recommended that immediate work be focused on: Anomalous Area 390kEast, and Anomalous Area 1. Anomalous Area 4 should be followed up pending additional results. If additional results are not available additional recce soil sampling in Anomalous Area 4 is justified.

Work should begin by prospecting of those sample sites with the highest “Sum” numbers. Presence of the minerals quartz, orpiment and realgar should be looked for, noted and sampled because of their obvious significance to discovery of Carlin type mineralization. Discovery of such indicators should be used to guide and prioritize soil sampling on systematic grids.

## SIGNATURE SIGNOFF

Respectfully submitted,

“Colin I. Godwin”

_________________________________ 
Colin I. Godwin, PhD, PEng, PGeo

Professor Emeritus, Department of Earth & Ocean Sciences

The University of British Columbia

August 13th 2014

# APPENDIX A: RvDfS OVERVIEW

RvDfS—Relative Value & Discrimination Factor Systematics—enhances discovery opportunities in geochemical analysis. The system is based on the fundamental distinction between background and anomalous populations, which is a fundamental division ignored by most statistical interpretations. Specifically, the technique uses: (i) all available geochemical analyses, (ii) statistical identification of anomalous populations, (iii) simple renumbering of anomalous and background populations to obtain relative values [R-values] that are additive, (iv) the addition of R-values to form geologically meaningful discrimination factors [D-factors], (v) direct application of D-factors to the interpretation of extensive, multi-element data sets, and (vi) geologically related multi-element D-factors, built from R-values, to identify underlying rock types and anomalous signatures for different types of mineral deposits.

Relative factors (R) are determined visually from log-probability plots and stem-leaf plots. R-values are assigned as follows: 1 = Low Background Population equal to or below the third quartile, 3 = High Background Population that is above the third quartile and up to the Mixed Population, 5 = Mixed Population of mixed High Background and Clearly Anomalous Populations, 7 = Clearly Anomalous Population immediately above the Mixed Population, 9 = Highly Anomalous Population, and 11 = Outliers of Highly Anomalous Population with exceptional concentrations (often equal to or greater than the 99.8th percentile).. Because geochemical elements are changed to numbers with genetic and population significance, those that occur in disparate amounts can be combined without the overshadowing of one element by another. For example, lead with an anomalous concentration of 100 ppm (say, R = 9) can be combined with zinc with an anomalous concentration of 2,500 ppm (say, R = 7) giving an R-value average of 8. Given chemically related elements, the D-factor that is the average of the summed R-values of related elements is likely to be more robust than the R-value for a single element. For example, Ni and Co correlate closely and the D-factor that is the combination of the two using averaged R-values is likely to be stronger than either element by itself.

Discrimination factors (D-factors) are the sum of the R-factors divided by the number of R-factors summed. Examples are D-factors for mafic rocks (DMafic) and precious metal deposits (DPrec), where: DMafic = (RMg + RTi + RV + RCr + RFe + RCo + RNi)/7, and DPrec = (RAg + RAu + RHg + RTl + RAs + RSb + RBi + RTe)/8. In both these cases, the associated R-values are for elements that provide a geological-association-description of mafic rocks or precious metal deposits. Generally, a D-factor of 4 or above represents values that are almost certainly statistically anomalous.

This new methodology, a “geologists’ factor analysis”, allows geological interpretation of data in a way that has genetic significance. And more specifically, where anomalous values exist for two or more elements, D-values from addition of R-values enhance the reliability of the anomaly—and therein lies their great practical advantage.

This approach, which defines factors using probability statistics, and geological knowledge and experience, complements traditional approaches. RvDfS will aid in discovery of mineral deposits from geochemical data sets for regional surveys, detailed soil grids and drill holes. RvDfS can also provide zoning vectors for deposit discovery.

Figure A1 demonstrates the methodology for visually assigning R-values. Log-probability graphs and stem-leaf diagrams are used together to facilitate decisions on partition values. A number of other graphs, such as histograms, can be helpful, but together the log-probability plot and stem leaf diagram seem to do the trick.

### Figure A1. Visual definition of background and anomalous populations.

R <=1 is the 75th percentile. R = 5 generally surrounds the visually picked inflection point that is marked by a “V” in the above figure. R = 11 marks extreme outliers and commonly is >= the 99.8th percentile. Note that elements associated with rock types (Ca & Sr, K & Rb, Mg, Na, etc.) do not represent bimodal background vs. anomalous distributions. Consequently, these elements are partitioned into broad groups following changes in slope along the log-probability curve.

### Main D-factors used to evaluate silver gold potential from the Carlincore Resources Limited data are:

DAgAu-Gold & Silver
DAgAu = (RAg + RAu)/2 to focus on precious metal deposits.

DHgTl-Mercury & Thallium
DHgTl = (RHg + RTl)/2 to help identify mobile elements related to epithermal and Carlin-type deposits.

DCarb: Carbonate rock factor
DCarb = (RMg + RCa + RSr)/3

DFels: Felsic rock factor
DFels = (RNa + RK + RRb + RSi + RGe)/5

DShal: Shale basin rock factor
DShal = (RV + RNb + RAl + RGa + RS + RSe)/6

DCarl: Carlin deposit factor
DCarl = (RSr + RBa + RAg + RAu + RZn + RCd + RHg + RTl + RP + RAs + RSb + RBi+ RS + RSe + RTe)/17

DPrec: Precious metal (Au, Ag) deposit factor
DPrec = (RAg + RAu + RHg + RTl + RAs + RSb + RTe)/7