Geophysical Field Report:

High Resolution Resistivity and Induced Polarization Survey

BREW Project

Resistivity/IP Survey

Dawson Mining District

NTS: 115O/03

Work Performed On:  August 3rd – August 12th, 2017

FOR:

White Gold Corp.

100 University Avenue, 8th Floor
Toronto, Ontario, Canada M5J 2Y1

(800) 564 6253

BY:

Jen Hanlon, M.Sc., GIT

GroundTruth Exploration Inc.

BOX 70, Dawson City, YT.

Date: October 31th, 2017

Table of Contents

1.0	Introduction	3

2.0	Survey Theory	4

2.1 Field Survey Operating Procedure	5

2.2 Data Processing	5

3.0 Survey Personnel and Program Dates	6

3.1 Survey Personnel	6

3.2 Program Dates	6

4.0	Zone One (BRWIP17-01 - BRWIP17-04)	7

4.1 Survey Summary	7

4.2 Survey Results	8

4.3 Interpretation	11

5.0	Zone Two (BRWIP17-05 - BRWIP17-07)	11

5.1 Survey Summary	11

5.2 Survey Results	13

5.3 Interpretation	15

6.0	Zone Three (BRWIP17-08 - BRWIP17-12)	15

6.1 Survey Summary	15

6.2 Survey Results	16

6.3 Interpretation	19

Appendix A: Description of Files and File Structure	20

Appendix B: SuperSting R8/IP Technical Specification	22

Appendix C: Extended dipole-dipole array	24

Table of Figures

Figure 1: Overview map of Brew 2017 RES/IP grid zones.	4

Figure 2: 2017 completed RES/IP grid on the Brew Zone 1.	8

Figure 3: BRWIP17-01 sections.	9

Figure 4: BRWIP17-02 sections.	9

Figure 5: BRWIP17-03 sections.	10

Figure 6: BRWIP17-04 sections.	10

Figure 7: 2017 completed RES/IP grid on the Brew Zone 2.	12

Figure 8: BRWIP17-05 sections.	13

Figure 9: BRWIP17-06 sections.	14

Figure 10: BRWIP17-07 sections.	14

Figure 11: 2017 completed RES/IP grid on the Brew Zone 3.	16

Figure 12: BRWIP17-08 sections.	17

Figure 13: BRWIP17-09 sections.	17

Figure 14: BRWIP17-10 sections.	18

Figure 15: BRWIP17-11 sections.	18

Figure 16: BRWIP17-12 sections.	19

# 1.0	Introduction

White Gold Corporation (WGO) headquartered in Toronto, ON commissioned GroundTruth Exploration Inc. (GroundTruth) headquartered in Dawson City, YT to complete high resolution resistivity and induced polarization (RES/IP) surveys on three different areas on the Brew (BRW) property during the 2017 field season.

The purpose of the RES/IP survey is to identify geological structure and delineate extent of mineralized zones that are indicated by soil anomalies. This report details the results of the RES/IP surveys. Additional surveying and interpretation is left to WGO’s discretion.

The three grid zones that are investigated on the Brew property are located within the Northwest corner of the property on adjacent ridgelines (see Figure 1). Note that Zone 1 is composed of 4 survey lines (BRWIP17-01 - BRWIP17-04), Zone 2 is composed of 3 survey lines (BRWIP17-05 - BRWIP17-07), and Zone 3 is composed of 5 survey lines (BRWIP17-08 - BRWIP17-12). Maps of the grids for each zone are shown in Figures 2, 7, and 11, respectively.

Figure 1: Overview map of Brew 2017 RES/IP grid zones. Zone 1 = lines 1-4; Zone 2 = lines 5-7; Zone 3 = lines 8-12.

# 2.0	Survey Theory

Resistivity and Induced Polarization surveys are an appropriate approach to lode-source gold exploration in Yukon Territories because of the resistivity contrasts inherent to the mineralization and geological structures that are associated with gold deposits. The non-invasive nature of RES/IP combined with its cost efficiency make it a valuable contribution to exploration efforts.

RES/IP surveys involve current injection from the ground surface to induce an electric field that is a function of the conductivity distribution in the subsurface. A current injection typically uses one sink electrode and one source electrode. A measurement of potential field is then acquired across two electrodes that are different from the current electrodes. Hundreds of potential field measurements are made at intervals along the RES/IP traverse for successive current injections to generate the final raw profile of apparent subsurface resistivity.

There are a wide number of array types used to perform RES/IP surveys, each involving a different configuration of current and potential electrodes. Different arrays have strengths and weaknesses in regards to the time necessary to complete the survey and the measurement sensitivity to vertical or horizontal subsurface features. GroundTruth utilizes an extended dipole-dipole array for the BRW project to adequately image the target zones. Details on the extended dipole-dipole array can be found in Appendix C.

## 2.1    Field Survey Operating Procedure

A crew of 5 GroundTruth personnel sets up and operates each survey. Brief operating procedures are as follows:

- The midpoint of a traverse is located and the length of the line is sighted using a compass and GPS.

- Minimal brush is cut along the line to place pickets and set up equipment.

- 84 electrodes are diligently inserted into the ground, equivalently spaced along the line at 5m and hammered to a depth of 50cm (10% of electrode spacing).

- Calcium Chloride (CaCl, 25% solution) is added to the base of all electrodes.

- Cables are laid and connected to the electrodes.

- Contact resistance test is conducted.

- Extra electrodes and CaCl solution is added to each electrode with CR >2,000 Ohms. CR test is repeated.

- Continue to add electrodes and CaCl until satisfactory CR values are achieved.

- Operator initializes survey.

- Operator uses DGPS and data collection software to document survey line parameters incl. electrode locations, topography, and notable geological/cultural features if present. Pickets are placed along the line every 50m.

- Crew cuts and prepares the next survey line.

## 2.2    Data Processing

Immediately after each survey is completed in the field, the data measurements are downloaded and reviewed for integrity. Any field errors are thus addressed before moving the equipment. RES/IP datasets are processed daily by the lead operator using EarthImager2D software provided by Advanced Geosciences Inc. Outlier/noisy data are removed and the cleaned dataset is inverted. Terrain correction to the inversion mesh is applied from topographic measurements collected in the field using a differential GPS. All raw data from the DGPS and SuperSting are archived for future consultation.

# 3.0	Survey Personnel and Program Dates

## 3.1    Survey Personnel

The following table summarizes the GroundTruth personnel involved in completing the survey lines on each of the three Brew grid zones. Note that on Zone 3 the GroundTruth team was composed of four personnel, instead of the usual five.

| Personnel | Position | Zone 1 (lines 1-4) | Zone 2 (lines 5-7) | Zone 3 (lines 8-12) |
| --- | --- | --- | --- | --- |
| Jen Hanlon | Lead Geophysical Operator and Crew Chief | ✔ | ✔ | ✔ |
| Pawel Kapa | Geo Technician | ✔ | ✔ | ✔ |
| Andrew Truax | Geo Technician | ✔ | ✔ | ✔ |
| Andrew Savage | Geo Technician | ✔ | ✔ |  |
| Patrick Dunbar | Geo Technician | ✔ | ✔ | ✔ |

Table 1: Summary of GroundTruth personnel involved in the RES/IP data acquisition on the three Brew grid zones.

## 3.2    Program Dates

Zone 1 (BRWIP17-01 - BRWIP17-04):

Mobilize	August 2nd

Field Surveys	August 3rd - August 5th

Zone 2 (BRWIP17-05 - BRWIP17-07):

Field Surveys	August 6th – August 7th

Zone 3 (BRWIP17-08 - BRWIP17-12):

Mobilize	August 8th

Field Surveys	August 9th – August 12th

# 4.0    Zone 1 (BRWIP17-01 - BRWIP17-04)

## 4.1    Survey Summary

An overview of the RES/IP grid in the Brew Zone 1 is shown in Figure 2. Brief specifications about the survey lines are outlined below.

Lines:	BRWIP17-01 – BRWIP17-04

Number of Electrodes	84

Electrode Spacing	5m

Line Length	415m

Array 	Extended dipole-dipole

Zone 1 on the Brew property is located in an old burn and the ground surface is either bare or partially covered by grasses, shrubs, and low lying plant material. After saturating the electrodes with calcium chloride solution, the rich but sometimes rocky soil typically lead to sufficient contact resistance (CR) values ranging between 500–3,300 Ohms. In situations where one side of the traverse had better contacts than the other, the array measurement direction was chosen to read from low to high CR.

Figure 2: 2017 completed RES/IP grid on the Brew Zone 1.

## 4.2    Survey Results

The following figures display the inverted resistivity and induced polarization results along each traverse in grid Zone 1. Note that the depth of penetration of the IP results is generally less than the resistivity results.

| (A) |  |
| --- | --- |
| (B) |  |

Figure 3: BRWIP17-01 sections. (A) Inverted resistivity (scale 50-10,000 Ohm-m). (B) Inverted IP (scale -10-75 ms).

| (A) |  |
| --- | --- |
| (B) |  |

Figure 4: BRWIP17-02 sections. (A) Inverted resistivity (scale 50-10,000 Ohm-m). (B) Inverted IP (scale -10-75 ms).

| (A) |  |
| --- | --- |
| (B) |  |

Figure 5: BRWIP17-03 sections. (A) Inverted resistivity (scale 50-10,000 Ohm-m). (B) Inverted IP (scale 0-75 ms).

| (A) |  |
| --- | --- |
| (B) |  |

Figure 6: BRWIP17-04 sections. (A) Inverted resistivity (scale 50-10,000 Ohm-m). (B) Inverted IP (scale 0-75 ms).

## 4.3     Interpretation

Interpretation of 2-D resistivity and induced polarization surveys first requires identifying anomalous zones that are caused by real subsurface electrical boundaries versus those that are artefacts formed during the inversion process. Real anomalous zones will trend between adjacent RES/IP lines and show correlation with crossline data. This section provides a brief qualitative description of the electrical conductivity and chargeability anomalies that trend between the RES/IP sections presented in section 4.2.

The resistivity sections show an extensive lateral resistive anomaly that undulates approximately mid-depth in the images. The IP sections show two zones of higher chargeability at depth on the NE and SW side of the grid. These two zones straddle a zone of low chargeability. Note that there are slight differences in the scale of the IP sections, making the background appear slightly more chargeable in BRWIP17-01 and BRWIP17-02 when compared to BRWIP17-03 and BRWIP17-04.

Similarities between the conductive and resistive units throughout Zone 1 of the Brew RES/IP survey lines inflicts confidence that these anomalies define a real subsurface electrical boundary. To further constrain this interpretation, it is recommended that known geological and geochemical information is incorporated. This will aid the interpreter to gain a better understanding of these anomalies and potentially aid them to find the geological structures and mineralized zones inherent to gold deposits.

# 5.0    Zone 2 (BRWIP17-05 - BRWIP17-07)

## 5.1    Survey Summary

An overview of the RES/IP grid in the Brew Zone 2 is shown in Figure 7. Brief specifications about the survey lines are outlined below.

Lines	BRWIP17-05 – BRWIP17-07

Number of Electrodes	84

Electrode Spacing	5m

Line Length	415m

Array 	Extended dipole-dipole

Due to the small number of survey lines on Zone 2 of the Brew, daily helicopter access is provided to the grid from the field crew’s initial camped position on Zone 1. This eliminates the need to use a mobilization day to tear down and set up a new camp. Ground cover on Zone 2 is an old burn with variable ground cover, including bare soil, rock, and mid-sized shrubs. Electrode contact resistance values in this zone are quite low, generally in the range of 400–1,500 Ohms. In situations where one side of the traverse had better contacts than the other, the array measurement direction was chosen to read from low to high CR.

Figure 7: 2017 completed RES/IP grid on the Brew Zone 2.

## 5.2    Survey Results

The following figures display the inverted resistivity and induced polarization sections along each traverse in grid Zone 2 on the Brew. Note that the depth of penetration of the IP results is generally less than the resistivity results.

| (A) |  |
| --- | --- |
| (B) |  |

Figure 8: BRWIP17-05 sections. (A) Inverted resistivity (scale 100-10,000 Ohm-m). (B) Inverted IP (scale 0-50 ms).

| (A) |  |
| --- | --- |
| (B) |  |

Figure 9: BRWIP17-06 sections. (A) Inverted resistivity (scale 100-10,000 Ohm-m). (B) Inverted IP (scale 0-50 ms).

| (A) |  |
| --- | --- |
| (B) |  |

Figure 10: BRWIP17-07 sections. (A) Inverted resistivity (scale 250-10,000 Ohm-m). (B) Inverted IP (scale 0-50 ms).

## 5.3     Interpretation

The 2-D RES/IP surveys acquired in grid Zone 2 of the Brew property show similar zones of resistivity and chargeability. Most notably is that the southwest side of the grid is more resistive than the northeast side. The defining electrical boundary trends approximately north-south through the grid. Additionally, there are two correlating chargeability anomalies sandwiched between a zone of low chargeability that are located at depth. To further constrain this interpretation, it is recommended that known geological and geochemical information is incorporated about the site. This will aid the interpreter to gain a better understanding of these anomalies and potentially aid them to identify geological structures and mineralized zones inherent to gold deposits.

# 6.0    Zone 3 (BRWIP17-08 - BRWIP17-12)

## 6.1    Survey Summary

An overview of the 2017 RES/IP grid in the Brew Zone 3 is shown in Figure 11. Brief specifications about the survey lines are outlined below.

Lines	BRWIP17-08 – BRWIP17-12

Number of Electrodes	84

Electrode Spacing	5m

Line Length	415m

Array 	Extended dipole-dipole

Placement of the survey lines on the Brew Zone 3 grid maximize north-south and east-west coverage of gold-in-soil anomalies and geological structures. On top of the ridge the terrain is similar to Zones 1 and 2; electrode contact resistances range between 1,000–3,000 Ohms and extends to 5,000 Ohms in particularly rocky zones. Terrain closer to the valley (i.e. BRWIP17-08), however, is covered with substantially more talus. This lead CR values on this line to be higher, generally in the range of 3,000–10,000 Ohms.

Figure 11: 2017 completed RES/IP grid on the Brew Zone 3.

## 6.2    Survey Results

The following figures display the inverted resistivity and induced polarization sections along each traverse in Zone 3 of the Brew. Note that the depth of penetration of the IP results can be notably less than the resistivity results.

| (A) |  |
| --- | --- |
| (B) |  |

Figure 12: BRWIP17-08 sections. (A) Inverted resistivity (scale 50-10,000 Ohm-m). (B) Inverted IP (scale 0-75 ms).

| (A) |  |
| --- | --- |
| (B) |  |

Figure 13: BRW17-09 sections. (A) Inverted resistivity (scale 50-10,000 Ohm-m). (B) Inverted IP (scale 0-75 ms).

| (A) |  |
| --- | --- |
| (B) |  |

Figure 14: BRWIP17-10 sections. (A) Inverted resistivity (scale 100-10,000 Ohm-m). (B) Inverted IP (scale 0-75 ms).

| (A) |  |
| --- | --- |
| (B) |  |

Figure 15: BRWIP17-11 sections. (A) Inverted resistivity (scale 100-10,000 Ohm-m). (B) Inverted IP (scale 0-75 ms)

| (A) |  |
| --- | --- |
| (B) |  |

Figure 16: BRWIP17-12 sections. (A) Inverted resistivity (scale 100-10,000 Ohm-m). (B) Inverted IP (scale 0-75 ms).

## 6.3     Interpretation

The 2D RES/IP surveys in Zone 3 of the Brew show good agreement between the data inversions of parallel and adjacent lines. Parallel lines BRWIP17-09 and BRWIP17-10 indicate two near-surface resistive zones that are underlain by a conductive layer. The other set of parallel lines (BRWIP17-11 and BRWIP17-12) indicate a resistive zone on the west and a more conductive zone on the east. Adjacent lines show that the grid is more conductive to the east. Similarly the IP inversions show zones of chargeability between parallel and adjacent surveys. To further constrain this interpretation, it is recommended that known geological and geochemical information is incorporated about the site. This will aid the interpreter to gain a better understanding of these anomalies and potentially aid them to identify geological structures and mineralized zones inherent to gold deposits.

# Appendix A: Description of Files and File Structure

This section explains the file naming structure and data content for each project.

Each RES/IP traverse has a unique Line ID created by combining: (1) the three letter project code for the property or zone, (2) an IP or RES data designation, (3) the last two digits of the year the survey was read, and (4) an identifying number for the traverse within each property or zone.

Example: ALBIP17-01, where ALB is the project code, IP is the type of data collected, 17 represents the year 2017, and 01 means that this is the first RES/IP dataset acquired on this property.

Each array dataset has a unique Data File ID. This ID is comprised by the date (yy-mm-dd), the first letter of the array type used (e.g. D for dipole-dipole or W for Wenner), and the number of times this array has been used that day.

Example: 170813D1

File Structure and Content:

- DATA

  - Line ID

    - Figures

      - figures of merged data pseudosections and inversions

    - GPS

      - Contains the DGPS raw data

    - Pictures

      - Pictures along the line

    - RAW

      - IP (data with IP data-misfits removed)

      - RES (data with RES data-misfits removed)

      - unprocessed data from SuperSting unit

    - XYZ

      - Inverted data for RES and IP saved in XYZ format

    - TRN

# Appendix B: E SuperSting R1/IP technical specification

Measurement modes	Apparent resistivity, resistance, self potential (SP), induced polarization (IP), battery voltage

Measurement range 	+/- 10V

Measuring resolution 	Max 30 nV, depends on voltage level

Screen resolution 	4 digits in engineering notation

Output current 	1mA – 2 A continuous, measured to high accuracy

Output voltage 	800 Vp-p, actual electrode voltage depends on transmitted current and ground resistivity

Output power 	200 W

Input gain ranging 	Automatic, always uses full dynamic range of receiver

Input impedance 	>20 MΩ

SP compensation 	Automatic cancellation of SP voltages during resistivity measurement. Constant and linearly varying SP cancels completely.

Type of IP measurement 	Time domain chargability (M), six time slots measured and stored in memory

IP current transmission 	ON+, OFF, ON-, OFF

IP time cycles 	0.5, 1 , 2 , 4 and 8 seconds (combined resistivity/IP mode)

Measure cycles	Running average of measurement displayed after each cycle. Automatic cycle stop when reading errors fall below user set limit or user set max cycles are done.

Resistivity time cycles 	Basic measure time is 0.4, 0.8, 1.2, 3.6, 7.2 or 14.4 seconds as selected by user via keyboard, autoranging and commutation adds about 1.4 s.

Signal processing 	Continuous averaging after each complete cycle. Noise errors calculated and displayed as percentage of reading. Reading displayed as resistance (ΔV/I) and apparent resistivity (Ωm). Resistivity is calculated using user entered electrode array coordinates.

Noise suppression 	Better than 100 dB at f>20 Hz

Better than 120 dB at power line frequencies (16 2/3, 20, 50 and 60 Hz) for measure cycles of 1.2 s and above

Total accuracy	Better than 1% of reading in most cases (lab measurements). Field measurement accuracy depends on ground noise and resistivity. Instrument will calculate and display running estimate of measuring accuracy.

System calibration	Calibration is done digitally by the microprocessor based on correction values stored in memory.

Supported manual 	Resistance, Schlumberger, Wenner, dipole-dipole, pole-dipole, pole-pole, SP-absolute, SP-gradient

Operating system 	Stored in re-programmable flash memory. New version can be downloaded from our web site and stored in the flash memory.

Data storage	Full resolution reading average and error are stored along with user entered coordinates and time of day for each measurement. Storage is effected automatically in a job oriented file system

Data display 	Apparent resistivity (Ohmmeter), injected current (mAmp) and measured voltage (mVolt) are displayed and stored in memory for each measurement

Memory capacity 	The memory can store 24,468 measurements in Resistivity Mode and 14,966 measurements in combined Resistivity/IP Mode

Data transmission 	RS-232C channel available to dump data from the instrument to a Windows type computer on user command.

Automatic multi-electrodes 	The SuperSting is designed to run dipole-dipole, pole-dipole, pole-pole, Wenner and Schlumberger surveys including roll-along surveys completely automatic with the Swift Dual Mode Automatic Multi-electrode system (patent 6,404,203) or with switch box and passive cables. The SuperSting can run any other array by using user programmed command files. These files are ASCII files and can be created using a regular text editor. The command files are downloaded to the SuperSting RAM memory and can at any time be recalled and run. Therefore there is no need for a fragile computer in the field.

Manual measurements 	The instrument has four banana pole screws for connecting current and potential electrodes during manual measurments

User controls 	20 key tactile, weather proof keyboard with alpha numeric entry keys and function keys.

On/off switch.

Measure button.

LCD night light switch (push to light).

Display 	Graphics LCD display (16 lines x 30 characters) with night light.

Power supply, field 	12V or 2x12 V DC external power (one or two 12 V batteries), connector on front panel.

Power supply, office 	DC power supply

Operating time 	Depends on survey conditions and size of battery used. Internal circuitry in auto mode adjusts current to save energy

Operating temperature 	-5 to +50ºC

Weight 	10.9 kg (24 lb.)

Dimensions 	Width 184 mm (7.25"), length 406 mm (16") and height 273 mm (10.75”)

# Appendix C: Extended Dipole-Dipole Array

The extended dipole-dipole array provides extended data coverage of the standard dipole-dipole array. The electrode configuration for dipole-dipole is shown below, where the current electrodes (A and B) and potential electrodes (M and N) are equivalently spaced by “a”, and separated by a factor “n” times the spacing “a”. A measurement of apparent resistivity can be calculated using the equation below the figure, where V = potential difference (V), I = current (Amp), and  𝜌A = apparent resistivity (Ohm-m).

Penetration depth of the extended dipole-dipole array (measurement locations shown below) is approximately 14 times the electrode spacing, which is equivalent to 70m using 5m electrode spacing, but is also dependent on: (1) the actual distribution of subsurface resistivity, and (2) the best achievable contact resistance values between the electrodes and the ground. The figure below shows the measurement locations (in pseudo depth) for an extended dipole-dipole array using 84 surface electrodes.