## JPR RAB Drilling

## Methods and Procedures

RAB drilling on the JPR property was conducted using Ground Truth Exploration’s, heli-portable, track mounted RAB drill. Standard operating procedures and description of the RAB are provided in Appendix D. All drill hole locations were located by GroundTruth Exploration geologists using a hand-held Garmin GPS. Once located, front and back sights were aligned with the hole using a compass and wooden pickets. The central picket was marked with the site ID, dip and azimuth. Where necessary timber platforms were constructed by Back Country Resources.

Drill sampling standard operating procedures are attached in Appendix B, Chapter 1- RAB and RC sample Technician Standard Operating Procedures. Before removing casing, an Optical Tele-viewer was used to survey the hole. Optical Tele-viewer procedures are summarized in Appendix _, Chapter 2- 2018 OTV-SOP. Check references - may be new ones

Phase I RAB holes, 12 in total, were not surveyed during or after drilling. Phase II RAB holes, 15 in total, were surveyed during or after drilling using a Geode GNSS but without base station correction.

Quality control

The quality control program for 2019 JPR RAB drilling consisted of the insertion of standards and blanks alternating one every 20 samples. Field blanks consisted of commercial limestone gravel.

| QC Type | Name | Value | -3 SD | +3 SD | # Inserted | # High Failures | # Low Failures | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blank | Limestone |  |  |  | 29 | 0 | - |  |
| Standard | CDN-GS1W | 1.063 | 0.949 | 1.177 | 9 | 0 | 0 |  |
| Standard | CDN-GS-5G | 4.77 | 4.17 | 5.37 | 9 | 0 | 0 |  |
| Standard | CDN-GS-P4G | 0.468 | 0.39 | 0.546 | 10 | 1 | 0 |  |

Analysis

JPR RAB samples were prepared and analyzed by ALS Global Laboratories of North Vancouver, BC.  The entire sample was first crushed to 70% passing -2 mm and then splitting off and pulverizing a 250 gram split to 85% passing -75 microns. A 0.5 gram cut of the pulp was then analyzed by ME-ICP41, which is an aqua regia digestion followed by ICP-AES analysis for 35 elements.  An additional 0.5 gram cut was analyzed by ME-MS42 for Te using an aqua regia digestion and ICP-ME analysis. Gold was analyzed for by AA-AU23 using a 30 gram charge for a standard fire assay with an AA finish.  Where necessary, samples with over limit ICP results (>100g/t Ag and >10,000ppm As and Pb) were re-run using ME-OG46, using a 0.40 gram cut, an aqua regia digestion and ICP-AES analysis, similar to ME-ICP41 but with different analytical calibration levels.

## JPR RC Drilling

### Methods and Procedures

RC drilling on the JPR property was conducted using Ground Truth Exploration’s, heli-portable, track mounted RC drill. Standard operating procedures and description of the RC are provided in Appendix D. All drill hole locations were located by GroundTruth Exploration geologists using a hand-held Garmin GPS. Once located, front and back sights were aligned with the hole using a compass and wooden pickets. The central picket was marked with the site ID, dip and azimuth. Where necessary timber platforms were constructed by Back Country Resources.

Drill sampling standard operating procedures are attached in Appendix B, Chapter 1- RAB and RC sample Technician Standard Operating Procedures. Before removing casing, an Optical Tele-viewer was used to survey the hole. Optical Tele-viewer procedures are summarized in Appendix _, Chapter 2- 2018 OTV-SOP. Check references - may be new ones

Drill hole collars were surveyed at the end of the season by Underhill Geomatics Ltd of Whitehorse, YT, using a Trimble GNSS system with base station correction.

Quality Control

The quality control program for 2019 JPR RC drilling consisted of the insertion of standards, blanks and duplicates every 10 samples. Standards were inserted in positions 10, 30, 60 and 80 within every 100 samples. Blanks, consisting of commercial limestone gravel, were inserted in positions 20, 40, 70 and 90. Field duplicates were inserted in positions 50 and 100.

Information on 2019 standards and blanks, including results are tabulated below. No systematic issues were noted.

| QC Type | Name | Value | -3 SD | +3 SD | # Inserted | # High Failures | # Low Failures | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blank | Limestone |  |  |  | 28 | 1 | - | 1 fail at 0.02 g/t |
| Standard | CDN-GS1V | 1.02 | 0.873 | 1.118 | 10 | 0 | 0 |  |
| Standard | CDN-GS-7G | 7.19 | 6.635 | 7.745 | 11 | 0 | 0 |  |
| Standard | CDN-GS-P4G | 0.468 | 0.39 | 0.546 | 8 | 0 | 0 |  |

### Analysis

JPR RC samples were prepared and analyzed by ALS Global Laboratories of North Vancouver, BC.  The entire sample was first crushed to 70% passing -2 mm and then splitting off and pulverizing a 250 gram split to 85% passing -75 microns. A 0.5 gram cut of the pulp was then analyzed by ME-ICP41, which is an aqua regia digestion followed by ICP-AES analysis for 35 elements.  An additional 0.5 gram cut was analyzed by ME-MS42 for Te using an aqua regia digestion and ICP-ME analysis. Gold was analyzed for by AA-AU25 using a 30 gram charge for a standard fire assay with an AA finish.  Where necessary, samples with over limit ICP results (>100g/t Ag and >10,000ppm As and Pb) were re-run using ME-OG46, using a 0.40 gram cut, an aqua regia digestion and ICP-AES analysis, similar to ME-ICP41 but with different analytical calibration levels.

## JPR Diamond Drilling

### Methods and Procedures

Collar marking protocols were the same for diamond drill holes as for the JPR RC holes. Timber platforms and rod racks were constructed by Back Country Resources. Once the drill was placed on the platform, a geologist would site the drill into place using a compass and the front and back sites marked prior to arrival. New Age Drilling carried out all coring work and placed them in core boxes which were transported via helicopter to Thunderdome Camp for logging and sampling.

All core logging and technical tasks were completed by geologists and geological technicians who were supervised by White Gold geologists.  The core depth markers were first checked for correctness and then one metre intervals were marked directly on the core with China markers. The start and end meterage of each core box was marked on the upper left and lower right respectively. A metal tag, noting hole identification, box number, and meterages was stapled to the top end of the core box for easy identification while stored.

Geotechnical data was collected by a supervised geotechnician or by the logging geologist for all holes, and included recovery, rock quality data (RQCD), a fracture count and magnetic susceptibility measurements. The logging geologist recorded lithology, alteration, mineralization, structural data and oxidation data. Once logging was completed, the core was photographed wet, with the hole ID, box number, and start/end meterages clearly visible on a white placard.

Drill hole collars were surveyed at the end of the season by Underhill Geomatics Ltd of Whitehorse, YT, using a Trimble GNSS system with base station correction.

### Sample Preparation

JPR diamond drill samples were prepared by a company geologist. Sample intervals are chosen based on the lithological, structural and mineralogical data acquired during the logging process and the geologist’s personal discretion. In general, after core is oriented, 2 m sample intervals are chosen in barren zones which are shortened to 1 m in altered and mineralized zones. A minimum sample length of 0.50 metres was used rarely.  Sample intervals are truncated at lithological, alteration and structural contacts. Core is then split in half using a gas-powered core saw by a core cutting technician who retains 50% as a mirror image of the sample in the core box. Samples are bagged in a 12”x20” ore bag for shipping. Drill core was then transported to the core storage area near the Thunderdome camp site.

Quality control

The quality control program for 2019 JPR diamond drilling consisted of the insertion of standards, blanks and duplicates every 10 samples. Standards were inserted in positions 10, 30, 60 and 80 within every 100 samples. Field blanks, consisting of syenite taken from the aptly named Deadrock Mountain, were inserted in positions 20, 40, 70 and 90. Field duplicates consisting of full 1/2 core, were inserted in positions 50 and 100.

Information on 2019 standards and blanks, including results are tabulated below. No systematic issues were noted.

| QC Type | Name | Value | -3 SD | +3 SD | # Inserted | # High Failures | # Low Failures | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blank | Syenite |  |  |  | 309 | 9 | - |  |
| Blank | Limestone |  |  |  | 5 | 0 | - - |  |
| Standard | Oreas 218 | 0.531 | 0.48 | 0.582 | 100 | 0 | 0 |  |
| Standard | CDN-ME-1308 1305 | 1.40 | 1.25 | 1.55 | 68 | 0 | 0 |  |
| Standard | CDN-GS-7G | 7.19 | 6.635 | 7.745 | 87 | 0 | 0 |  |
| Standard | CDN-GS-25 | 25.60 | 24.19 | 27.01 | 20 | 0 | 1 |  |

A number of syenite blanks (9) did exceed the 3 times lower detection limit of 0.015 g/t Au. Most of these were results from additional blanks inserted after suspected high grade samples. The four worst failures ranging from 0.037 to 0.088 g/t were all associated with original samples with grades in excess of 32 g/t Au and as high as 141 g/t Au.

### Analysis

JPR diamond drill core samples were prepared and analyzed by ALS Global Laboratories of North Vancouver, BC.  The entire sample was first crushed to 70% passing -2 mm and then splitting off and pulverizing a 500 gram split to 85% passing -75 microns. A 0.5 gram cut of the pulp was then analyzed by ME-ICP41, which is an aqua regia digestion followed by ICP-AES analysis for 35 elements.  An additional 0.5 gram cut was analyzed by ME-MS42 for Te using an aqua regia digestion and ICP-ME analysis. Gold was analyzed for by AA-AU23 using a 30 gram charge for a standard fire assay with an AA finish.  If Au results were >10 g/t a second 30 gram charge was used for a standard fire assay with a gravimetric finish. Where necessary samples with over limit ICP results (>100g/t Ag and >10,000ppm As and Pb) were re-run by ME-OG46, using a 0.40 gram cut, an aqua regia digestion and ICP-AES analysis, similar to ME-ICP41 but with different analytical calibration levels.

## WHT RAB Drilling

## Methods and Procedures

RAB drilling on the WHT property was conducted using Ground Truth Exploration’s, heli-portable, track mounted RAB drill. Standard operating procedures and description of the RAB are provided in Appendix _. All drill hole locations were located by GroundTruth Exploration geologists using a hand-held Garmin GPS. Once located, front and back sights were aligned with the hole using a compass and wooden pickets. The central picket was marked with the site ID, dip and azimuth. Where necessary timber platforms were constructed by Back Country Resources.

Drill sampling standard operating procedures are attached in Appendix B, Chapter 1- RAB and RC sample Technician Standard Operating Procedures. Before removing casing, an Optical Tele-viewer was used to survey the hole. Optical Tele-viewer procedures are summarized in Appendix _, Chapter 2- 2018 OTV-SOP. Check references - may be new ones

Drill hole collars not surveyed during or after drilling.

Quality control

The quality control program for 2019 WHT RAB drilling consisted of the insertion of standards and blanks alternating one every 20 samples. Field blanks consisted of commercial limestone gravel.

| QC Type | Name | Value | -3 SD | +3 SD | # Inserted | # High Failures | # Low Failures | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blank | Limestone GravelSyenite |  |  |  | 13 | 0 | - |  |
| Standard | Oreas 218 | 0.531 | 0.48 | 0.582 | 4 | 0 | 0 |  |
| Standard | CDN-GS-1V 1308 1305 | 1.02 | 0.873 | 1.118 | 5 | 0 | 0 |  |
| Standard | CDN-GS-1W | 1.063 | 0.949 | 1.177 | 1 | 0 | 0 |  |
| Standard | CDN-GS-1R | 1.21 | 1.045 | 1.375 | 1 | 0 | 0 |  |
| Standard | CDN-GS-7G | 7.19 | 6.635 | 7.745 | 4 | 0 | 0 |  |

Analysis

WHT RAB samples were prepared and analyzed by ALS Global Laboratories of North Vancouver, BC.  The entire sample was first crushed to 70% passing -2 mm and then splitting off and pulverizing a 250 gram split to 85% passing -75 microns. A 0.5 gram cut of the pulp was then analyzed by ME-ICP41, which is an aqua regia digestion followed by ICP-AES analysis for 35 elements.  An additional 0.5 gram cut was analyzed by ME-MS42 for Te using an aqua regia digestion and ICP-ME analysis. Gold was analyzed for by AA-AU25 using a 30 gram charge for a standard fire assay with an AA finish.  Where necessary, samples with over limit ICP results (>100g/t Ag and >10,000ppm As and Pb) were re-run using ME-OG46, using a 0.40 gram cut, an aqua regia digestion and ICP-AES analysis, similar to ME-ICP41 but with different analytical calibration levels.

## WHT Diamond Drilling

### Methods and Procedures

Collar marking protocols were the same for WHT diamond drill holes as for the WHT RAB holes. Timber platforms and rod racks were constructed by Back Country Resources. Once the drill was placed on the platform, a geologist would site the drill into place using a compass and the front and back sites marked prior to arrival. New Age Drilling carried out all coring work and placed them in core boxes which were transported via helicopter to Thistle Camp for logging and sampling.

All core logging and technical tasks were completed by geologists and geological technicians who were supervised by White Gold geologists.  The core depth markers were first  checked for correctness and then one metre intervals were marked directly on the core with China markers. The start and end meterage of each core box was marked on the upper left and lower right respectively. A metal tag, noting hole identification, box number, and meterages was stapled to the top end of the core box for easy identification while stored.

Geotechnical data was collected by a supervised geotechnician or by the logging geologist for all holes, and included recovery, rock quality data (RQCD), a fracture count and magnetic susceptibility measurements. The logging geologist recorded lithology, alteration, mineralization, structural data and oxidation data. Once logging was completed, the core was photographed wet, with the hole ID, box number, and start/end meterages clearly visible on a white placard.

Drill hole collars were surveyed at the end of the season by Underhill Geomatics Ltd of Whitehorse, YT, using a Trimble GNSS system with base station correction.

### Sample Preparation

Diamond drill samples were prepared by a company geologist. Sample intervals are chosen based on the lithological, structural and mineralogical data acquired during the logging process and the geologist’s personal discretion. In general, after core is oriented, 2 m sample intervals are chosen in barren zones which are shortened to 1 m in altered and mineralized zones. A minimum sample length of 0.50 metres was used rarely.  Sample intervals are truncated at lithological, alteration and structural contacts. Core is then split in half using a gas-powered core saw by a core cutting technician who retains 50% as a mirror image of the sample in the core box. Samples are bagged in a 12”x20” ore bag for shipping. Drill core was then transported to the core storage area near the Thunderdome camp site.

Quality control

The quality control program for 2019 WHT diamond drilling consisted of the insertion of standards, blanks and duplicates every 10 samples. Standards were inserted in positions 10, 30, 60 and 80 within every 100 samples. Field blanks, consisting of syenite taken from the aptly named Deadrock Mountain, were inserted in positions 20, 40, 70 and 90. Field duplicates consisting of full 1/2 core, were inserted in positions 50 and 100.

Information on 2019 standards and blanks, including results are tabulated below. No systematic issues were noted.

| QC Type | Name | Value | -3 SD | +3 SD | # Inserted | # High Failures | # Low Failures | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blank | Syenite |  |  |  | 188 | 0 | - |  |
| Standard | Oreas 218 | 0.531 | 0.48 | 0.582 | 62 | 0 | 1 |  |
| Standard | CDN-GS-1V 1308 1305 |  |  |  | 63 | 1 | 1 |  |
| Standard | CDN-GS-7G | 1.02 | 0.873 | 1.118 | 63 | 0 | 0 |  |

### Analysis

Diamond drill core samples were prepared and analyzed by ALS Global Laboratories of North Vancouver, BC.  The entire sample was first crushed to 70% passing -2 mm and then splitting off and pulverizing a 500 gram split to 85% passing -75 microns. A 0.5 gram cut of the pulp was then analyzed by ME-ICP41, which is an aqua regia digestion followed by ICP-AES analysis for 35 elements.  An additional 0.5 gram cut was analyzed by ME-MS42 for Te using an aqua regia digestion and ICP-ME analysis. Gold was analyzed for by AA-AU23 using a 30 gram charge for a standard fire assay with an AA finish.  If Au results were >10 g/t a second 30 gram charge was used for a standard fire assay with a gravimetric finish. Where necessary samples with over limit ICP results (>100g/t Ag and >10,000ppm As and Pb) were re-run by ME-OG46, using a 0.40 gram cut, an aqua regia digestion and ICP-AES analysis, similar to ME-ICP41 but with different analytical calibration levels.

## QV RC Drilling

### Methods and Procedures

RC drilling on the QV property was conducted using Ground Truth Exploration’s, heli-portable, track mounted RC drill. Standard operating procedures and description of the RC are provided in Appendix D. All drill hole locations were located by GroundTruth Exploration geologists using a hand-held Garmin GPS. Once located, front and back sights were aligned with the hole using a compass and wooden pickets. The central picket was marked with the site ID, dip and azimuth. Where necessary timber platforms were constructed by Back Country Resources.

Drill sampling standard operating procedures are attached in Appendix B, Chapter 1- RAB and RC sample Technician Standard Operating Procedures. Before removing casing, an Optical Tele-viewer was used to survey the hole. Optical Tele-viewer procedures are summarized in Appendix _, Chapter 2- 2018 OTV-SOP. Check references - may be new ones

Drill hole collars were not surveyed. I don’t think that a Geode was used.

Quality Control

The quality control program for 2019 JPR RC drilling consisted of the insertion of standards, blanks and duplicates every 10 samples. Standards were inserted in positions 10, 30, 60 and 80 within every 100 samples. Blanks, consisting of commercial limestone gravel, were inserted in positions 20, 40, 70 and 90. Field duplicates were inserted in positions 50 and 100.

Information on 2019 standards and blanks, including results are tabulated below. No systematic issues were noted.

| QC Type | Name | Value | -3 SD | +3 SD | # Inserted | # High Failures | # Low Failures | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blank | Limestone |  |  |  | 25 | 3 | - |  |
| Standard | CDN-GS1V | 1.02 | 0.873 | 1.118 | 6 | 0 | 0 |  |
| Standard | CDN-GS-7G | 7.19 | 6.635 | 7.745 | 10 | 0 | 0 |  |
| Standard | CDN-GS-P4G | 0.468 | 0.39 | 0.546 | 9 | 2 | 0 |  |

### Analysis

QV RC samples were prepared and analyzed by ALS Global Laboratories of North Vancouver, BC.  The entire sample was first crushed to 70% passing -2 mm and then splitting off and pulverizing a 250 gram split to 85% passing -75 microns. A 0.5 gram cut of the pulp was then analyzed by ME-ICP41, which is an aqua regia digestion followed by ICP-AES analysis for 35 elements.  An additional 0.5 gram cut was analyzed by ME-MS42 for Te using an aqua regia digestion and ICP-ME analysis. Gold was analyzed for by AA-AU23 using a 30 gram charge for a standard fire assay with an AA finish.  Where necessary, samples with over limit ICP results (>100g/t Ag and >10,000ppm As and Pb) were re-run using ME-OG46, using a 0.40 gram cut, an aqua regia digestion and ICP-AES analysis, similar to ME-ICP41 but with different analytical calibration levels.